3GPP TSG-RAN WG3 Meeting #107-e 	R3-200603

E-meeting, 24 February– 6 March, 2020

Agenda item:	13.2.1.2.

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR-IAB BL CR for TS 38.473) BH RLC channel mapping configuration in Donor-DU and intermediate IAB nodes

WID/SID:	NR_IAB-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the configuration in Donor-DU and intermediate IAB node for BH RLC channel mapping.

# 2	Configuration for BH RLC channel mapping

The following table summarizes the current status for BH RLC channel mapping configuration in the Donor-DU, intermediate IAB node, and access IAB node.

| Node | Configuration item | Purpose |
| --- | --- | --- |
| Donor DU | IP header (IP address and/or Flow Label and/or DSCP)  BH RLC channel ID | Egress BH RLC channel ID determination for DL IP packet |
| Intermediate IAB node | Downlink: Prior Hop ID + Ingress BH RLC channel ID + Next Hop ID  Egress BH RLC channel ID | Egress BH RLC channel ID determination for DL traffic received from an Ingress BH RLC CH |
|  | Uplink: Prior Hop ID + Ingress BH RLC channel ID + Next Hop ID  Egress BH RLC channel ID | Egress BH RLC channel ID determination for UL traffic received from an Ingress BH RLC CH |
| Access IAB Node | For U-plane, IP address + GTP-U TEID or DRB ID  BH RLC channel ID For C-plane, CP traffic BH RLC channel ID | Egress BH RLC channel ID determination for UL IP packet |

## 2.1 Donor-DU

The Donor-DU can only be configured by the F1AP procedure. Figure 1 shows an IAB topology adaptation example to be used in following analysis. In this example, the F1AP procedure is initiated towards the Donor-DU1 to configure the BH RLC channel mapping before topology adaptation, and towards the Donor-DU2 to configure the BH RLC channel mapping after the topology adaptation.

Figure 1: Intra-DU Topology Adaptation Example

There are two options for the F1AP procedure to configure the BH RLC channel mapping in Donor-DU.

- Option 1: Donor-CU initiates a UE-associated F1AP procedure to configure the BH RLC channel mapping in Donor-DU.

In this option, the BH RLC channel mapping information is considered as part of the context for the Child IAB node, i.e. IAB1-MT in above example. The F1AP UE associated procedure is related to IAB1-MT.

- Option 2: Donor-CU initiates a non-UE-associated F1AP procedure to configure the BH RLC channel mapping in Donor-DU.

In this option, the BH RLC channel mapping information is considered as the part of the configuration information of the Donor-DU, i.e. Donor-DU1 in above example.

The BH RLC channel mapping in the Donor-DU is either to map the related traffic to an existing BH RLC channel, or a new BH RLC channel that needs to be setup for the new traffic. Currently, the F1AP UE Context Setup/Modification procedure are used to setup/modify the BH RLC channel. The BH RLC channel mapping information can be considered as the context of a BH RLC channel, i.e. the list of the traffic to be mapped to a related BH RLC channel. When the new traffic requires a new BH RLC channel, it is natural to provide the context of the BH RLC channel mapping to Donor-DU as part of the F1AP UE Context Setup/Modification procedure. Of course, option 2 also works but at the cost of an additional F1AP procedure after the F1AP procedure to setup the new BH RLC channel. In case the mapping is performed to an existing BH RLC Channel, both options use one F1AP procedure to configure the BH RLC channel mapping.

After the topology adaptation, the BH RLC channel mapping information related to the BH RLC CH between Donor-DU1 and IAB1 shall be removed in Donor-DU1. With option 1, this is performed when the Donor-DU1 release the UE context for IAB1-MT. Option 1 does not require additional F1AP procedure to remove the BH RLC channel mapping information. But for Option 2, Donor-CU need to initiate a F1AP procedure to remove the BH RLC channel mapping information.

Even both options work, Option 1 is better than option 2 with less signaling.

Proposal 1: use F1AP UE Context Setup/Modification procedure to configure the BH RLC channel mapping in the Donor-DU.

## 2.2 Intermediate IAB node

In above example, IAB1 is the intermediate IAB node. Let’s assume 3 BH RLC channels (i.e. A1, A2 and A3) are already established between Donor-DU and IAB1.The Donor-CU is going to establish the BH RLC channel (i.e. B2) between IAB1 and IAB2, and to configure the BH RLC channel mapping in IAB1.

Considering RAN3 agreed “Configuration of downlink bearer mapping and routing should be performed by F1-AP”, and the information to be configured for UL BH RLC channel mapping and DL BH RLC channel mapping are the same, it is straightforward to use F1AP to configure both UL and DL BH RLC channel mapping in intermediate IAB node (e.g. IAB1 in this example).

Similarly, there are two options for the F1AP procedure to configure the BH RLC channel mapping in intermediate IAB, e.g. IAB1.

- Option 1: Donor-CU initiates a UE-associated F1AP procedure to configure the BH RLC channel mapping in IAB1.

In this option, the BH RLC channel mapping information is considered as part of the context for the Child IAB node, i.e. IAB2-MT in above example. The F1AP UE associated procedure is related to IAB2-MT.

When Donor-CU initiates the F1AP UE Context Setup/Modification procedure to setup the BH RLC channel (B2) between IAB2 and IAB1, the F1AP message to IAB1 can include the BH RLC channel mappingto be configured in IAB1 for both DL and UL. For example, the configuration includes:

- DL mapping information:

- Prior-hop BAP address: Donor-DU1’s BAP address, DL ingress BH RLC CH: A1

- Prior-hop BAP address: Donor-DU1’s BAP address, DL ingress BH RLC CH: A2

This means Donor-DU1 - IAB1 BH RLC channels A1 and A2 are mapped to IAB1-IAB2 BH RLC channel B2.

- UL mapping information:

- next-hop BAP address: Donor-DU1’s BAP address, UL Egress BH RLC CH: A1

This means IAB1-IAB2 BH RLC channel B2 are mapped to Donor-DU-IAB1 BH RLC channel A1.

A possible call flow example is shown as below:

Figure 2: use F1AP to configure intermediate IAB node for BH RLC channel mapping

- Option 2: Donor-CU initiates a non-UE-associated F1AP procedure to configure the BH RLC channel mapping in IAB1.

In this option, the BH RLC channel mapping information is considered as the part of the configuration information of the DU, i.e. IAB1-DU in above example.

Similarly to the reasons given in Section 2.1, using Option 2 requires more signaling in some scenarios, e.g. when establish a new BH RLC Channel, or when the child IAB node (i.e. IAB2 in this example) performs a topology adaptation. Based on this analysis, it is preferred to use UE-associated F1AP procedure to configure the BH RLC channel mapping in the intermediate IAB node.

Proposal 2: use UE-associated F1AP to configure both UL BH RLC channel mapping and DL BH RLC channel mapping in intermediate IAB node.

The Stage-3 implementation can be designed to use the same F1 UE CONTEXT SETUP/MODIFIACTION REQUEST message to configure both Donor-DU for DL BH RLC channel mapping, and the intermediate IAB node for UL and DL BH RLC channel mapping.  For example, the BH RLC channel mapping IE can be defined as a CHOICE for Donor-DU and intermediate IAB.

For Donor-DU, the BH RLC channel mapping information includes a list of {IP address, IPv6 Flow Label, DS} to be mapped to related BH RLC Channel.

For Intermediate IAB node, it can be further defined to contain the BH RLC channel mapping for DL and UL.

- For DL: includes a list of prior-hop BAP ID + DL Ingress BH RLC CH ID, which will be mapped to the related BH RLC CH.

- For UL: includes a list of next-hop BAP ID + UL Egress BH RLC CH ID, which the related BH RLC CH will be mapped to.

Proposal 3: Enhance the F1AP UE CONTEXT SETUP REQUEST/MODIFICATION REQUEST message to include the information to configure DL BH RLC channel mapping in Donor-DU, and UL/DL BH RLC channel mapping in intermediate IAB node.

# 3	Conclusion

In this contribution, we discussed how to configure the BH RLC channel mapping in Donor-DU, intermediate IAB node, and access IAB node. Our proposals area:

Proposal 1: use F1AP UE Context Setup/Modification procedure to configure the BH RLC channel mapping in the Donor-DU.

Proposal 2: use UE-associated F1AP to configure both UL BH RLC channel mapping and DL BH RLC channel mapping in intermediate IAB node.

Proposal 3: Enhance the F1AP UE CONTEXT SETUP REQUEST/MODIFICATION REQUEST message to include the information to configure DL BH RLC channel mapping in Donor-DU, and UL/DL BH RLC channel mapping in intermediate IAB node.

The proposed TP to TS38.473 BL CR can be found at the end of this contribution.

# References

[1]	R3-196760, Discussion on UL mapping in IAB

Proposed TP for TS38.473 BL CR:

-------------------------------------------Next Change -------------------------------------------

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB and BH RLC Channel configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

>>>	Unaffected parts skipped    <<<

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the BH RLC channel mapping Information IE is included in the BH RLC Channel to be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, use the received BH RLC channel mapping Information to perform the mapping the related traffic to the associated BH RLC channel.

-------------------------------------------Next Change -------------------------------------------

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

>>>	Unaffected parts skipped    <<<

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the BH RLC channel mapping Information IE is included in the BH RLC Channel to be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, use the received BH RLC channel mapping Information to perform the mapping the related traffic to the associated BH RLC channel.

If the BH RLC Channel To Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the BH RLC channel mapping Information IE is included in the BH RLC Channel to be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, use the received BH RLC channel mapping Information to overwrite the previously stored mapping information for the associated BH RLC channel.

If the BH RLC Channel To Be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall release the BH RLC Channels in the list.

If two UL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

-------------------------------------------Next Change -------------------------------------------

# 9	Elements for F1AP Communication

-------------------------------------------Next Change -------------------------------------------

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>UL BH Information | O |  | 9.3.1.y |  | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >> UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.x |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.z |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BH RLC channel mapping Information | O |  | 9.3.1.b |  | - |  |
| Child BAP Address | O |  | 9.3.1.v |  | YES | reject |

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>UL BH Information | O |  | 9.3.1.y |  | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >> UL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>UL BH Information | O |  | 9.3.1.y |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >> RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | YES | reject |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.x |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.z |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BH RLC channel mapping Information | O |  | 9.3.1.b |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.x |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.z |  |  |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BH RLC channel mapping Information | O |  | 9.3.1.b |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.x |  | - |  |
| Child BAP Address | O |  | 9.3.1.v |  | YES | reject |

-------------------------------------------Next Change -------------------------------------------

#### 9.3.1.b	BH RLC channel mapping Information

This IE includes information used by the gNB-DU to perform the BH RLC channel mapping.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE BH RLC channel mapping Information |  |  |  |  |
| >Donor-DU BH RLC channel mapping information List |  | 1 |  |  |
| >>Donor-DU BH RLC channel mapping information Item |  | 1.. <maxnoofAggregatedTraffic> |  |  |
| >>>Destination IP Address | M |  | 9.3.2.3 | This IE indicates the IP address of the destination node of the DL traffic. |
| >>>IPv6 Flow Label | O |  | BIT STRING (SIZE(20)) | This IE indicates the IPv6 Flow label of the DL traffic. |
| >>>DSCP | O |  | BIT STRING (SIZE(6)) | This IE indicates the DS information of the DL traffic. |
| >Intermediate IAB BH RLC channel mapping information |  |  |  |  |
| >>DL Ingress BH RLC CH List |  | 0..1 |  | For DL, this IE indicates the list of ingress BH RLC channels to be mapped to the related BH RLC channel. |
| >>>DL Ingress BH RLC CH Item |  | 1.. <maxnoofAggregatedDLBHRLCCH> |  |  |
| >>>>Prior-Hop BAP Address | M | 9.3.1.v |  |  |
| >>>>DL Ingress BH RLC CH ID | M | 9.3.1.x |  |  |
| >>UL Egress BH RLC CH List |  | 0..1 |  | For UL, this IE indicates the list of Egress BH RLC Channels that the related BH RLC channel can be mapped to. |
| >>>UL Egress BH RLC CH Item |  | 1.. <maxnoofAggregatedDLBHRLCCH > |  |  |
| >>>>Next-Hop BAP Address | M | 9.3.1.v |  |  |
| >>>>UL Egress BH RLC CH ID | M | 9.3.1.x |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAggregatedTraffic | Maximum no. of traffic aggregated into one BH RLC CH, the maximum value is FFS. |
| maxnoofAggregatedDLBHRLCCH | Maximum no. of DL Ingress BH RLC channels related to a BH RLC channel, the maximum value is FFS. |
| maxnoofAggregatedULBHRLCCH | Maximum no. of UL Egress BH RLC channels related to a BH RLC channel, the maximum value is FFS. |

-------------------------------------------Start of Change -------------------------------------------

### 9.4.5	Information Element Definitions

Unaffected parts skipped

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofAggregatedTraffic,

maxnoofAggregatedDLBHRLCCH,

maxnoofAggregatedULBHRLCCH

FROM F1AP-Constants

Unaffected parts skipped

#### -- B

BAPAddress ::= BIT STRING (SIZE(10))

BAPPathID ::= BIT STRING (SIZE(10))

BAPRoutingID ::= BIT STRING (SIZE(20))

BitRate ::= INTEGER (0..4000000000000,...)

BearerTypeChange ::= ENUMERATED {true, ...}

BHRLCchannelMappingInfo	::= CHOICE {

donorDUBHRLCchannelMappingInfo					DonorDUBHRLCchannelMappingInfo,

intermediateIABBHRLCchannelMappingInfo			IntermediateIABBHRLCchannelMappingInfo,

choice-extension							ProtocolIE-SingleContainer { { BHRLCchannelMappingInfo-ExtIEs} }

}

BHRLCchannelMappingInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DonorDUBHRLCchannelMappingInfo ::= SEQUENCE (SIZE(1..maxnoofAggregatedTraffic)) OF DonorDUBHRLCchannelMappingInfo-Item

DonorDUBHRLCchannelMappingInfo-Item ::= SEQUENCE {

destIPAddress				TransportLayerAddress,

iPv6FlowLabel 				BIT STRING (SIZE (20))	OPTIONAL,

dSCP		 				BIT STRING (SIZE (6))	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { DonorDUBHRLCchannelMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

DonorDUBHRLCchannelMappingInfo-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IntermediateIABBHRLCchannelMappingInfo	::= SEQUENCE {

dLIngressBHRLCCH				DLIngressBHRLCCH	OPTIONAL,

uLEgressBHRLCCH					ULEgressBHRLCCH		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IntermediateIABBHRLCchannelMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

IntermediateIABBHRLCchannelMappingInfo-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLIngressBHRLCCH	::= SEQUENCE (SIZE(1.. maxnoofAggregatedDLBHRLCCH)) OF DLIngressBHRLCCH-Item

DLIngressBHRLCCH-Item ::= SEQUENCE {

priorHopBAPAddress			BAPAddress,

dLIngressBHRLCCHID			BHChannelID,

iE-Extensions				ProtocolExtensionContainer { { DLIngressBHRLCCH-ItemExtIEs} } OPTIONAL,

...

}

DLIngressBHRLCCH-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULEgressBHRLCCH	::= SEQUENCE (SIZE(1.. maxnoofAggregatedULBHRLCCH)) OF ULEgressBHRLCCH-Item

ULEgressBHRLCCH-Item ::= SEQUENCE {

nextHopBAPAddress			BAPAddress,

uLEgressBHRLCCHID			BHChannelID,

iE-Extensions				ProtocolExtensionContainer { { ULEgressBHRLCCH-ItemExtIEs} } OPTIONAL,

...

}

ULEgressBHRLCCH-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannelID ::= INTEGER (1..32,...)-- the value is FFS

Unaffected parts skipped

BHChannels-ToBeModified-Item ::= SEQUENCE {

bHChannelID						BHChannelID,

bHQoSInformation				BHQoSInformation,

rLCmode				RLCMode	OPTIONAL,

bHRLCchannelMappingInfo	BHRLCchannelMappingInfo,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeModified-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeReleased-Item ::= SEQUENCE {

bHChannelID	BHChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeSetup-Item ::= SEQUENCE	{

bHChannelID						BHChannelID,

bHQoSInformation				BHQoSInformation,

rLCmode				RLCMode,

bHRLCchannelMappingInfo	BHRLCchannelMappingInfo,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeSetup-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------Next Change -------------------------------------------

### 9.4.7	Constant Definitions

Unaffected parts skipped

maxnoofslots							INTEGER ::= 320

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels						INTEGER ::= xxx

maxnoofRoutingEntries					INTEGER ::= xxx

maxnoofAggregatedTraffic				INTEGER ::= xx1

maxnoofAggregatedDLBHRLCCH				INTEGER ::= xx2

maxnoofAggregatedULBHRLCCH				INTEGER ::= xx3

-------------------------------------------End of Change -------------------------------------------