3GPP TSG-RAN3 Meeting #107-e	R3-201254

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for WWC BL CR for TS 38.413) Support for interfacing trusted non-3GPP access networks to the 5GC

Source: 	Huawei, Telecom Italia, BT, Broadcom, CMCC

Agenda item:	21.2.2

Document for:	Other

1. Introduction

CB: # 38_Email038-WWC_TPs

-  Check TPs 0390,0391,0392,0393 (from both AIs 21.2.2 and 21.2.3)

- revise if needed; check details

- go for agreement

(HW)

Summary of offline disc R3-201197

0390 rev in R3-201253

0391 rev in R3-201254

0392 rev in R3-201255

0393 rev in R3-201256

2. Annex – TP

1st Change

2	References

[y]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[z]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

#### 9.2.5.3	UPLINK NAS TRANSPORT

This message is sent by the NG-RAN node and is used for carrying NAS information over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | Ignore |
| W-AGF Identity List | O |  | 9.3.1.bb |  | YES | reject |
| TNGF Identity Information | O |  | Identity Information 9.3.1.bb |  | YES | reject |
| TWIF Identity Information | O |  | Identity Information 9.3.1.bb |  | YES | reject |

2nd Change

#### 9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >TNGF user location information |  |  |  |  |  |  |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [y]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >TWIF user location information |  |  |  |  |  |  |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [y]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23. 316 [x]. |  |  |
| >>Global Line Identifier | O |  | FFS | Encoded as defined in TS 23.003 [23]. | - |  |
| >>Global Cable Identifier | O |  | FFS | Encoded as defined in TS 23.003 [23]. | - |  |

Editor’s Note:	It is FFS on the detailed format and composition of the Global Line Identifier and the Global Cable Identifier.

3rd Change

#### 9.3.1.yy	Global TNGF ID

This IE is used to globally identify a TNGF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| CHOICE TNGF ID | M |  |  |  |
| >TNGF ID |  |  |  |  |
| >>TNGF ID | M |  | BIT STRING (SIZE(32, …)) |  |

4th Change

#### 9.3.1.aa	Global TWIF ID

This IE is used to globally identify a TWIF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| CHOICE TWIF ID | M |  |  |  |
| >TWIF ID |  |  |  |  |
| >>TWIF ID | M |  | BIT STRING (SIZE(32, …)) |  |

#### 9.3.1.bb	Identity Information

This IE indicates a list of identifiers of N3 terminations at W-AGF, or TNGF, or TWIF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Address | O |  | Transport Layer Address 9.3.2.4 |  |
| FQDN | O |  | OCTET STRING | The details of the IE definition is specified in TS 29.510 [z]. |

5th Change

9.4.4	PDU Definitions

<Unchanged Text Omitted>

WarningMessageContents,

WarningSecurityInfo,

WarningType,

RIMInformationTransfer,

RGLevelWirelineAccessCharacteristics,

IdentityInformation,

AuthenticatedIndication

<Unchanged Text Omitted>

id-WarningSecurityInfo,

id-WarningType,

id-RIMInformationTransfer,

id-RGLevelWirelineAccessCharacteristics,

id-W-AGFIdentityList,

id-AuthenticatedIndication,

id-TNGFIdentityInformation,

id-TWIFIdentityInformation

<Unchanged Text Omitted>

UplinkNASTransport-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE mandatory	}|

{ ID id-W-AGFIdentityList			CRITICALITY reject	TYPE W-AGFIdentityList				PRESENCE optional	}|

{ ID id-TNGFIdentityInformation			CRITICALITY reject	TYPE IdentityInformation				PRESENCE optional	}|

{ ID id-TWIFIdentityInformation			CRITICALITY reject	TYPE IdentityInformation				PRESENCE optional	},

...

}

<Unchanged Text Omitted>

9.4.5	Information Element Definitions

<Unchanged Text Omitted>

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

maxnoofAllowedAreas,

<Unchanged Text Omitted>

TimeUEStayedInCellEnhancedGranularity ::= INTEGER (0..40950)

TNAP-ID ::= OCTET STRING

TWAP-ID ::= OCTET STRING

TNGF-ID ::= CHOICE {

tNGF-ID				BIT STRING (SIZE(32, ...)),

choice-Extensions		ProtocolIE-SingleContainer { {TNGF-ID-ExtIEs} }

}

TNGF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

<Unchanged Text Omitted>

TWIF-ID ::= CHOICE {

tWIF-ID				BIT STRING (SIZE(32, ...)),

choice-Extensions		ProtocolIE-SingleContainer { {TWIF-ID-ExtIEs} }

}

TWIF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

<Unchanged Text Omitted>

UserLocationInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UserLocationInformationTNGF		CRITICALITY ignore	TYPE UserLocationInformationTNGF				PRESENCE mandatory}|

{ ID id-UserLocationInformationTWIF		CRITICALITY ignore	TYPE UserLocationInformationTWIF				PRESENCE mandatory}|

{ ID id-UserLocationInformationW-AGF	CRITICALITY ignore	TYPE UserLocationInformationW-AGF				PRESENCE mandatory},

...

}

<Unchanged Text Omitted>

UserLocationInformationTNGF ::= SEQUENCE {

tNAP-ID				TNAP-ID,

iPAddress			TransportLayerAddress,

portNumber			PortNumber															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationTNGF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationTNGF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformationTWIF ::= SEQUENCE {

tWAP-ID				TWAP-ID,

iPAddress			TransportLayerAddress,

portNumber			PortNumber															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationTWIF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationTWIF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged Text Omitted>

#### -- W

IdentityInformation ::= SEQUENCE {

iPAddress					TransportLayerAddress										OPTIONAL,

fQDN						FQDN														OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { IdentityInformation-ExtIEs} } 	OPTIONAL,

...

}

IdentityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged Text Omitted>

9.4.7	Constant Definitions

<Unchanged Text Omitted>

id-TNGFIdentityInformation												ProtocolIE-ID ::= xxx

id-TWIFIdentityInformation												ProtocolIE-ID ::= yyy

id-UserLocationInformationTWIF								ProtocolIE-ID ::= zzz

<Unchanged Text Omitted>

Change Ends