3GPP TSG-RAN3 Meeting #107-e	R3-201255

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for WWC BL CR for TS 29.413) Support for interfacing wireline 5G access networks to the 5GC

Source: 	Huawei, Telecom Italia, BT, Broadcom

Agenda item:	21.2.3

Document for:	Other

1. Introduction

CB: # 38_Email038-WWC_TPs

-  Check TPs 0390,0391,0392,0393 (from both AIs 21.2.2 and 21.2.3)

- revise if needed; check details

- go for agreement

(HW)

Summary of offline disc R3-201197

0390 rev in R3-201253

0391 rev in R3-201254

0392 rev in R3-201255

0393 rev in R3-201256

5. Annex – TP

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

/**Skip the unrevelant**/

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	MobilityRestriction List IE

-	Index to RAT/Frequency Selection Priority IE

UPLINK NAS TRANSPORT message:

-	W-AGF Identity Information IE: the information given within this IE between the W-AGF and the AMF contains a list of identifiers of N3 terminations at W-AGF as specified in TS 23.316 [x].

/**Skip the unrevelant**/

The User Location Information IE in the applicable NGAP messages between the N3IWF and the AMF includes the IP address and port number as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the TNGF and the AMF includes the TNAP ID, IP address and port number as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the W-AGF and the AMF includes the W-AGF User Location Information IE as specified in TS 38.413 [2].

The Security Key IE in the applicable NGAP messages includes the N3IWF key as specified in TS 33.501 [5].

The RAN UE NGAP ID IE in the applicable NGAP messages identifies the UE association over the NG interface within the N3IWF node, or the TNGF node, or the TWIF node, or the W-AGF node, as specified in TS 38.413 [2].