3GPP TSG-RAN WG3 #107-e	R3-20xxxx

# Online, 24th February - 06th March 2020

Agenda Item:	20.2.2

Source:	Ericsson, ZTE?

Title: 	TP to F1 BL CR: Support of NR V2X over F1

Document for:	others

# 1	TP to F1AP BL CR: addition of NR V2X SIBs to

START OF CHANGES

#### 9.3.1.18	gNB-DU System Information

This IE contains the system information generated by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MIB message | M |  | OCTET STRING | MIB message, as defined in TS 38.331 [8]. | - |  |
| SIB1 message | M |  | OCTET STRING | SIB1 message, as defined in TS 38.331 [8]. | - |  |
| V2X SIBs message | O |  | OCTET STRING | NR V2X SIB messages, as defined in TS 38.331 [8]. (FFS) | - | Ignore |

Editor’s note: pending on RAN2’s progress, currently SIBs X,Y and Z are the V2X SIBs defined in TS 38.331

NEXT CHANGE

Editor’s note: decision between option 1 or 2 will be based upon RAN3 discussion and RAN2 feedback.

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NEXT CHANGE

NEXT CHANGE

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

SymbolAllocInSlot,

Slot-Configuration-Item,

NumDLULSymbols,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Addresses-Info,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

LTEUESidelinkAggregateMaximumBitrate,

SidelinkUEInformation, (FFS)

CG-ConfigInfo (FFS)

FROM F1AP-IEs

NEXT CHANGE

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Addresses-Info,

id-SidelinkUEInformation, (FFS)

id-CG-ConfigInfo, (FFS)

id-v2xsIB-message,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofslots

FROM F1AP-Constants;

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-RequestedP-MaxFR2,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-SidelinkUEInformation (FFS),

id-CG-ConfigInfo, (FFS)

id-v2xsIB-message,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNRminus1,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN

UNCHANGED PART OMITTED

GNB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

GNB-DU-System-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-v2xsIB-message	CRITICALITY reject	EXTENSION V2XSIB-message	PRESENCE optional},

...

}

UNCHANGED PART OMITTED

#### -- S

SidelinkUEInformation (FFS)::= OCTET STRING

UNCHANGED PART OMITTED

#### -- V

V2XSIB-message ::= OCTET STRING

NEXT CHANGE

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

UNCHANGED PART OMITTED

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= xxx

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= xxy

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= xyy

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= yyy

(FFS)id-LTEV2XSidelinkInfoList						ProtocolIE-ID ::= xxz

(FFS)id-NRSidelinkInfoList							ProtocolIE-ID ::= xzz

id-SidelinkUEInformation	(FFS)					ProtocolIE-ID ::= xx1

id-v2xsIB-message									ProtocolIE-ID ::= xx2

id-CG-ConfigInfo	(FFS)							ProtocolIE-ID ::= xx3

END

-- ASN1STOP

END OF CHANGE