# 3GPP TSG-RAN WG3 #107-bis-e						R3-201810

# 20 April – 30 April 2020

# Title:	[DRAFT] Further reply LS on assistance indication for WUS

# Response to:	Reply LS on assistance indication for WUS from SA2 (S2-2001732)

# Release:	Release 15

# Work Item:

Source:	Qualcomm Incorporated [to be RAN3]

To:	SA2

Cc:	CT1, RAN2

Contact Person:

Name:	Luis Lopes

Tel. Number:

E-mail Address:	llopes@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

After further consideration of the issues raised by SA2 in the LS “Reply LS on assistance indication for WUS” and on the solution approved in TS 23.401 CR 3582, RAN3 has discussed a modified solution, which is briefly described below. RAN3 would like SA2 to review this, and provide feedback as to whether it could be acceptable from SA2’s perspective.

It is first assumed that the UE’s use of WUS is modified by RAN2 (with or without an associated capability), such that the UE only uses WUS for paging monitoring if no reselection takes place after release. RAN3 understands that this behaviour change was assumed in the SA2 solution.

Then, in order to minimize the RAN’s use of WUS:

- At release, a WUS supporting eNB provides the list of recommended cells with at least the last cell;

- During the paging procedure, an eNB that receives the S1 PAGING message decides whether to page using WUS depending on the last cell in the received list of recommended cells, i.e. use WUS if paging cell is the same as last cell (and UE is WUS-capable), or if the list of recommended cells is not received; otherwise WUS is not used.

This solution should guarantee that WUS is always used in the last cell in case of no UE idle mobility, provided the eNB supports WUS. On the other hand, it minimizes the use of WUS in other cells, particularly if WUS is deployed on a tracking area basis (as this minimizes WUS use caused by the last eNB not generating a list of recommended cells).

RAN3 would welcome feedback on this modification of the SA2 solution. From RAN3 perspective, this modification is attractive since it only impacts the behaviour of WUS supporting nodes and does not raise the concerns explained in the previous reply LS.

2. Actions:

To SA WG2 group.

ACTION: 	RAN3 kindly asks SA WG2 to review the modified solution described in this LS and provide feedback as to whether it could be acceptable from SA2’s perspective.

.

3. Date of Next RAN3 Meetings:

RAN3#108-bis	25-29 May 2020	e-meeting

RAN3#109	24-28 Aug 2020	Toulouse, FR