3GPP TSG-RAN WG3 Meeting #107bis-e	R3-201841

E-meeting, 20 – 30 April, 2020

Agenda item:	10.2.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.473) Introduction of RACH Assistance Information

Document for:	Text Proposal

# 1	Introduction

This TP includes updates to the SON BL CR for TS 38.473, by introducing support for transfer of RACH assistance information on the F1 interface according to discussion in [1]. RACH assistance information comprises RACH Failure Rate as well as non-UE associated procedures for transfering the NR UE RACH Reports from gNB-CU to gNB-DU. It also introduces a UE-associated procedure for transfer of NR UE RACH Reports from gNB-CU to gNB-DU.

The TP is based on [2].

# References

[1]		R3-201840, Discussion on PRACH Configuration Conflict Resolution based on CB#30 at RAN3#107-e, Nokia, Nokia Shanghai Bell

[2]	R3-201618, SON BL CR to TS 38.473

# Annex	- TP for TR 38.473

<<< start of changes >>>

Start of the first change

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| UE RACH Report Transfer | UE RACH REPORT TRANSFER |

Start of the next change

### 8.9.x	UE RACH Report Transfer

#### 8.9.x.1	General

The UE RACH Report Transfer procedure is initiated by the gNB-CU in order to to transfer UE RACH Reports for which there exists an active UE context. The procedure uses UE-associated signalling.

#### 8.9.x.2	Successful Operation

Figure 8.9.x.2-1: UE RACH Report Transfer procedure. Successful operation.

The gNB-CU initiates the procedure by sending a UE RACH REPORT TRANSFER message.

#### 8.9.x.3	Abnormal Conditions

Not applicable.

Start of the next change

#### 9.2.8.1	DU-CU RADIO INFORMATION TRANSFER

This message is sent by a gNB-DU to a gNB-CU, to convey radio-related information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE DU-CU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>DU-CU RIM Information | M |  | 9.3.1.91 |  | - | - |
| >RACH Assistance Information Transfer |  |  |  |  |  |  |
| >>DU-CU RACH Failure Rate Information | O |  | 9.2.X.Z1 |  | YES | ignore |

<< next change >>>

#### 9.2.8.2	CU-DU RADIO INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to convey radio-related information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE CU-DU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>CU-DU RIM Information | M |  | 9.3.1.92 |  | - | - |
| > RACH Assistance Information Transfer |  |  |  |  |  |  |
| >> NR UE RACH Report Information | O |  | 9.2.X.Z3 |  | YES | ignore |
| >> CU-DU RACH Failure Rate Information | O |  | 9.2.X.Z2 |  | YES | ignore |

<< next change >>>

#### 9.2.8.x	UE RACH REPORT TRANSFER

This message is sent by the gNB-CU to transfer UE RACH Reports for which there exist an active UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell To Report List |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 | The cell in which the RACH attempts were performed |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of Failed RACH Attempts on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts on the NUL carrier, for the SSB, extracted from multiple UE RACH Reports |  |  |
| >>>Number of Failed RACH Attempts on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts on the SUL carrier, for the SSB, extracted from multiple UE RACH Reports |  |  |

<< next change >>>

#### 9.2.X.Z1	DU-CU RACH Failure Rate Information

The DU-CU RACH Failure Rate Information IE contains RACH failure rate calculated at the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | 9.3.1.12 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>RACH Failure Rate on NUL Carrier | M |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>RACH Failure Rate on SUL Carrier | O |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

<< next change >>>

#### 9.2.X.Z2	CU-DU RACH Failure Rate Information

The CU-DU RACH Failure Rate Information IE provides RACH failure rate calculated at cells that are neighbor to cells served by the the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Reporting cell list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Reporting Cell ID | M |  | NR CGI 9.3.1.12 | The cell for which the RACH failure rate is calculated. |  |  |
| >>Neighbour Cell List |  | 1 |  | List of cells in the gNB-DU that are neighbours to the reporting cell |  |  |
| >>>Neighbour Cell Item |  | 1.. <maxCellingNBDU> |  |  |  |  |
| >>>Neighbour Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>SSB List |  |  |  | List of SSBs in the cell for which the RACH failure rate is calculated |  |  |
| >>>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>>RACH Failure Rate on NUL Carrier | M |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>>RACH Failure Rate on SUL Carrier | O |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

<< next change >>>

#### 9.2.X.Z3	NR UE RACH Report Information

The NR UE RACH Report Information IE contains information about failed RACH attempts at cells served by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 | The cell in which the RACH attempts were performed |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of Failed RACH Attempts on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts on the NUL carrier, for the SSB, extracted from multiple UE RACH Reports |  |  |
| >>>Number of Failed RACH Attempts on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts on the SUL carrier, for the SSB, extracted from multiple UE RACH Reports |  |  |

<<< end of changes >>>