3GPP TSG-RAN WG3 #107bis-e 	 																R3-201873

20- 30 April 2020 E-Meeting

Title: 	(TP for NR_Mob_enh BL CR for TS 38.413): Left issues for DAPS handover

Source: 	Huawei

Agenda item:	15.2.1

Document for:	Other

1. Introduction

In RAN3#107-e meeting, DAPS handover was discussed and achieved the following agreements [1]:

For DAPS indicator:

The DAPS HO indicator is configured per E-RAB/DRB, and check RAN2’s discussion, if the indication is not included in the RRC container, it should be added into the handover request message over X2/Xn interface.

Add the DAPS indicator in the E-RABs To Be Setup Item IE for X2 interface.

Add the DAPS indicator per E-RAB/DRB into the Handover Required message and Handover Request message over S1/NG interface.

The DAPS HO indicator per E-RAB / DRB be separately included in Source eNB to Target eNB Transparent Container / Source NG-RAN Node to Target NG-RAN Node Transparent Container IE over S1/NG interface

Add the DAPS indicator per E-RAB in the E-RABs Information List over S1/NG interface

For DAPS fallback mechanism:

During X2/Xn based DAPS HO, if target cell does not support DAPS operations, the fallback mechanism should be allowed over X2/Xn interface.

Through the explicit IE over the X2/Xn interface, source cell obtains the fallback result from target cell.

Left to target node implementation to decide whether or not to fallback to legacy HO or reject the HO.

During S1/NG based DAPS HO, if target cell does not support DAPS operations, the fallback mechanism should be allowed over S1/NG interface

During S1/NG based DAPS HO, the fallback mechanism is similar to that of the X2/Xn interface

For HO Success:

Introduce two new class 2 procedures to support the HO Success procedure similar to the X2/Xn interface.

And, the left issues are as below:

Further discuss which option should be applied to standardize the DAPS indicator per DRB for NR

Option 1:  Top-level indication + lists of DAPS DRB

Option 2:  Be included in DRB to QoS Flow Mapping List

Option 3:  UE Context level indication + lists of DAPS DRB

Further discuss which option should be applied in DAPS HO response message

Option 1：To carry more information in the DAPS HO response message, e.g, DAPS HO accepted, fallback to legacy HO

Option 2:  Only an indicator “DAPS HO accepted” is carried in the handover response message

Option 3：For DAPS HO, the response info per E-RAB / DRB should be considered

Further discuss the impact on E1 interface for DAPS HO.

Further discuss S1/NG DAPS HO Count.

To be continued on this basis...

In this paper, we would further discuss the left issues for DAPS HO.

2. Discussion

Issue1: DAPS indicator in the HANDOVER REQUIRED and HANDOVER REQUEST message

For S1/NG based DAPS HO, it has agreed to add the DAPS indicator per E-RAB/DRB into the Handover Required message and Handover Request message over S1/NG interface. Therefore, the DAPS HO indicator per E-RAB / DRB is separately included in Source eNB to Target eNB Transparent Container / Source NG-RAN Node to Target NG-RAN Node Transparent Container IE over S1/NG interface.

The potential three option3 are as below [2]:

Option 1:  Top-level indication + lists of DAPS DRB;

Option 2:  Be included in DRB to QoS Flow Mapping List;

Option 3:  UE Context level indication + lists of DAPS DRB;

Compared these options, Option 2 is simply and straight to indicate the per E-RAB/DRB indicator. Based on BLCR to TS38413 [3], Option 2 is used, i.e. the DAPS indicator per DRB is included in the DRBs to QoS Flows Mapping List IE, thus no change is needed.

Proposal 1: The DAPS indicator per DRB could be included in the DRBs to QoS Flows Mapping List IE.

Issue 2: DAPS indicator in the BEARER CONTEXT SETUP REQUEST message

Considering CU-CP/CU-UP separation, the target gNB-CU-UP can take charge of admission control, e.g. determine whether DAPS HO request for any concerned DRB is accepted after receiving the BEARER CONTEXT SETUP REQUEST message which includes the per DRB DAPS indicator.

Proposal 2: The per DRB DAPS HO indicator should be included in the BEARER CONTEXT SETUP REQUEST message.

Issue 3: DAPS response information in the HANDOVER REQUEST ACK and HANDOVER COMMAND message

Based on the request message, the target node would perform admission control and generate the HANDOVER REQUEST ACK message. In last meeting, three options are shown as below [2]:

Option 1：To carry more information in the DAPS HO response message, e.g, DAPS HO accepted, fallback to legacy HO

Option 2:  Only an indicator “DAPS HO accepted” is carried in the handover response message

Option 3：For DAPS HO, the response info per E-RAB / DRB should be considered

Firstly, we think admission control should be performed per E-RAB/ DRB, thus the response information should be per E-RAB/ DRB. Secondly, since fallback mechanism should be allowed when the target node does not support DAPS HO, the detailed response information, e.g, DAPS HO accepted or fallback to legacy HO considering NG based DAPS HO, or, DAPS HO accepted or fallback to legacy HO or falllback to rel14 MBB considering S1 based DAPS HO, should be included in the HANDOVER REQUEST ACK message. In this way, the source node can understand whether to keep data transmission with the UE after sending the DAPS HO command to the UE. For example, the source node would stop transmission for the concerned DRB after the DAPS HO command if the target node’s decision is fallback to legacy HO, if the target node’s decision is fallback to rel14 MBB or DAPS HO accepted, the source node would keep transmission for the concerned DRB.

In general, it is better to combine Option1 with Option3. Based on BLCR to TS38413 [3], the DAPS Reponse Information IE can be used for this purpose.

Proposal 3: The per E-RAB/ DRB response information, e.g, DAPS HO accepted or fallback to legacy HO or fallback to rel14 MBB, should be included in the HANDOVER REQUEST ACK and HANDOVER COMMAND message.

Issue 4: DAPS response information in the BEARER CONTEXT SETUP RESPONSE message

Considering CU-CP/CU-UP separation, the target gNB-CU-UP can determine whether it accept the DAPS HO request or not, e.g. it may allow DAPS HO request or it may just allow legacy HO or it may reject DAPS HO request. And then the gNB-CU-UP would inform its decision to the gNB-CU-CP via the BEARER CONTEXT SETUP RESPONSE message, i.e. the per DRB response information, e.g. DAPS HO accepted or fallback to legacy HO, should be included in the BEARER CONTEXT SETUP RESPONSE message.

Proposal 4: The DAPS HO response information, e.g, DAPS HO accepted or fallback to legacy HO, should be included in the BEARER CONTEXT SETUP RESPONSE message.

Issue 5: HO success procedure

In X2/Xn based DAPS HO procedure, HO SUCCESS message is introduced to inform the source node that the UE has successfully accessed to the target node. Similarly, for S1/NG-based DAPS HO, we need to consider how to implement feature like the HO success procedure over X2/Xn interface. There are two potential options:

Option 1: Introduce new class2 procedure;

Option 2: Re-use HO notify procedure

To make little spec impact, re-using HO notify procedure to inform successful DAPS HO seems better since all the IEs in current HO NOTIFY message can also be applicable for successful DAPS HO procedure.

Proposal 5: To re-use HO notify procedure for DAPS HO success.

3. Conclusion

This paper mainly discusses about S1/NG based DAPS handover and proposes:

Proposal 1: The DAPS indicator per DRB could be included in the DRBs to QoS Flows Mapping List IE.

Proposal 2: The per DRB DAPS HO indicator should be included in the BEARER CONTEXT SETUP REQUEST message.

Proposal 3: The per E-RAB/ DRB response information, e.g, DAPS HO accepted or fallback to legacy HO or falllback to rel14 MBB, should be included in the HANDOVER REQUEST ACK and HANDOVER COMMAND message.

Proposal 4: The DAPS HO response information, e.g, DAPS HO accepted or fallback to legacy HO, should be included in the BEARER CONTEXT SETUP RESPONSE message.

Proposal 5: To re-use HO notify procedure for DAPS HO success.

The TPs for the BLCRs are provided in annex and reference.

4. Reference

[1] Chairman notes, “Draft Report of 3GPP TSG RAN WG3 meeting #107-e”.

[2] R3-201156, Summary of offline discussion for CB: # 83_ Email043-MobEnh_DAPS_common, CATT

[3] R3-201422, Baseline CR for introducing Rel-16 NR mobility enhancement

[4] R3-201874	(TP for LTE_feMob-Core BL CR for TS 36.413): Left issues for DAPS handover

[5] R3-201875	Left issues for DAPS handover (E1AP CR)

[6] R3-202424	(TP for NR_Mob_enh BL CR for TS 38.423): Left issues for DAPS handover

[7] R3-202425	(TP for LTE_feMob-Core BL CR for TS 36.423): Left issues for DAPS handover

Annex - TP for NR_Mob_enh BL CR for TS 38.413

Start of changes

8.4.3	Handover Notification

8.4.3.1	General

The purpose of the Handover Notification procedure is to indicate to the AMF that the UE has arrived to the target cell and the NG-based handover has been successfully completed.

8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Handover notification

The target NG-RAN node shall send the HANDOVER NOTIFY message to the AMF when the UE has been identified in the target cell and the NG-based handover has been successfully completed.

8.4.3.3	Abnormal Conditions

Void.

Next change

### 8.4.x	Handover Success

#### 8.4.x.1	General

The Handover Success procedure is used during a DAPS Handover, to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: Handover Success, successful operation

The AMF initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

Next change

9.3.1.34	DRBs to QoS Flows Mapping List

This IE contains a list of DRBs containing information about the mapped QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs to QoS Flows Mapping Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >DRB ID | M |  | 9.3.1.53 |  |  |  |
| >Associated QoS Flow List | M |  | 9.3.1.99 | Contains information of the QoS flows mapped to the DRB |  |  |
| >DAPS Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

Next change

9.3.1.y	DAPS Response Information

The DAPS Response Indicator IE indicates that the per DRB response to a requested DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Information List |  |  |  |  |
| >DAPS Response Information Item |  | 1..<maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.3.1.53 |  |
| >>DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, fallback to legacy HO, …) | Indicates if the DAPS Handover is accepted |

Next change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-Cause,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-DataForwardingNotPossible,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-LastEUTRAN-PLMNIdentity,

id-LocationReportingAdditionalInfo,

id-MaximumIntegrityProtectedDataRate-DL,

id-NetworkInstance,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-PDUSessionAggregateMaximumBitRate,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PSCellInformation,

id-QosFlowAddOrModifyRequestList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-S-NSSAI,

id-TNLAssociationTransportLayerAddressNGRAN,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-DAPSInfo,

id-DAPSResponseInformation-List,

maxnoofAllowedAreas,

maxnoofAllowedS-NSSAIs,

maxnoofBPLMNs,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofForbTACs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNGConnectionsToReset,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofQosFlows,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofTACs,

maxnoofTAIforInactive,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs

FROM NGAP-Constants

///////////////////////////////////////////////////////////////////////unchange skiped /////////////////////////////////////////////////////////////////////

#### -- D

DataCodingScheme ::= BIT STRING (SIZE(8))

DataForwardingAccepted ::= ENUMERATED {

data-forwarding-accepted,

...

}

DataForwardingNotPossible ::= ENUMERATED {

data-forwarding-not-possible,

...

}

DataForwardingResponseDRBList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

dRB-ID								DRB-ID,

dLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

uLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{DataForwardingResponseDRBItem-ExtIEs}}		OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSInfo ::= SEQUENCE {

DAPSIndicator				ENUMERATED {DAPS-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSInfo-ExtIEs} } OPTIONAL,

...

}

DAPSInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSResponseInformation-List	::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DAPSResponseInformation–Item

DAPSResponseInformation-Item	::= SEQUENCE {

drbID						DRB-ID,

dapsresponseindicator	ENUMERATED {dapshoaccepted, fallback-to-legacy-HO,...},

iE-Extension				ProtocolExtensionContainer { { DAPSResponseInformation-Item-ExtIEs } }	OPTIONAL,

...

}

DAPSResponseInformation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

///////////////////////////////////////////////////////////////////////unchange skiped /////////////////////////////////////////////////////////////////////

#### -- T

TAC ::= OCTET STRING (SIZE(3))

TAI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

TAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAIBroadcastEUTRA-Item

TAIBroadcastEUTRA-Item ::= SEQUENCE {

tAI								TAI,

completedCellsInTAI-EUTRA		CompletedCellsInTAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {TAIBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

TAIBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAIBroadcastNR-Item

TAIBroadcastNR-Item ::= SEQUENCE {

tAI							TAI,

completedCellsInTAI-NR		CompletedCellsInTAI-NR,

iE-Extensions		ProtocolExtensionContainer { {TAIBroadcastNR-Item-ExtIEs} } OPTIONAL,

...

}

TAIBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAICancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAICancelledEUTRA-Item

TAICancelledEUTRA-Item ::= SEQUENCE {

tAI								TAI,

cancelledCellsInTAI-EUTRA		CancelledCellsInTAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {TAICancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

TAICancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAICancelledNR ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAICancelledNR-Item

TAICancelledNR-Item ::= SEQUENCE {

tAI							TAI,

cancelledCellsInTAI-NR		CancelledCellsInTAI-NR,

iE-Extensions		ProtocolExtensionContainer { {TAICancelledNR-Item-ExtIEs} } OPTIONAL,

...

}

TAICancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForInactive ::= SEQUENCE (SIZE(1..maxnoofTAIforInactive)) OF TAIListForInactiveItem

TAIListForInactiveItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {TAIListForInactiveItem-ExtIEs} } OPTIONAL,

...

}

TAIListForInactiveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForPaging ::= SEQUENCE (SIZE(1..maxnoofTAIforPaging)) OF TAIListForPagingItem

TAIListForPagingItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {TAIListForPagingItem-ExtIEs} } OPTIONAL,

...

}

TAIListForPagingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForRestart ::= SEQUENCE (SIZE(1..maxnoofTAIforRestart)) OF TAI

TAIListForWarning ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI

TargeteNB-ID ::= SEQUENCE {

globalENB-ID		GlobalNgENB-ID,

selected-EPS-TAI	EPS-TAI,

iE-Extensions		ProtocolExtensionContainer { {TargeteNB-ID-ExtIEs} } OPTIONAL,

...

}

TargeteNB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetID ::= CHOICE {

targetRANNodeID		TargetRANNodeID,

targeteNB-ID		TargeteNB-ID,

choice-Extensions		ProtocolIE-SingleContainer { {TargetID-ExtIEs} }

}

TargetID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer		RRCContainer,

iE-Extensions		ProtocolExtensionContainer { {TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSResponseInformation-List						CRITICALITY reject	EXTENSION DAPSResponseInformation-List	PRESENCE optional},

...

}

///////////////////////////////////////////////////////////////////////unchange skiped /////////////////////////////////////////////////////////////////////

#### -- U

/////////////////////////////////////////////////////////////// unchange skiped ////////////////////////////////////////////////////////////////////

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-DAPSInfo												ProtocolIE-ID ::= XXX

id-DAPSResponseInformation-List								ProtocolIE-ID ::= YYY

END

End of fourth change