3GPP TSG-RAN WG3 #107bis-e 	 																R3-201880

20- 30 April 2020 E-Meeting

Title: 	(TP for NR_Mob_enh BL CR for TS 38.423): Target Node Initiated CHO Modification

Source: 	Huawei

Agenda item:	15.3.1.3

Document for:	Other

1. Introduction

In last e-meeting, the target node initiated CHO modification was discussed. The following progress were made.

Reuse existing CHO Cancel and HO Request to enable target node to inform source node the change of reserved CHO resources for the UE.

It is FFS whether an explicit indicator is needed in conditional HO cancel message or a new cause value is sufficient.

Since the need of target node initiated CHO modification was confirmed and the agreements is to reuse existing CHO cancel and HO request message to implement such function.

The only FFS is whether an explicit indicator is needed in conditional HO cancel message or a new cause value is sufficient. This is not a complicated issue, and the technical arguments for an explicit indicator were already disclosed during the discussion at last e-meeting:

- There is no way to enforce the source node to trigger a new CHO request just based on a cause value in the Handover Cancel message from the target node.

Therefore, it is proposed RAN3 to agree on the new indicator to support the target node initiated CHO modification in truth.

Proposal: To introduce a new IE in the Conditional Handover Cancel message to enable the target node to indicate the source the change request for reserved CHO resources.

The TPs are provided in the annex and in [1].

2. Reference

[1] R3-201881	(TP for LTE_feMob BL CR for TS 36.423): Target Node Initiated CHO Modification.

Annex - TP for NR_Mob_enh BL CR for TS 38.423

Start of first change

### 8.2.B	Conditional Handover Cancel

#### 8.2.B.1	General

The Conditional Handover Cancel procedure is used to enable a target NG-RAN node to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

#### 8.2.B.2	Successful Operation

Figure 8.2.B.2-1: Conditional Handover Cancel, successful operation

The target NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source NG-RAN node. The target NG-RAN node shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

At the reception of the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved for candidate cells associated to the UE-associated signalling identified by the Source NG-RAN node UE XnAP ID IE and the Target NG-RAN node UE XnAP ID IE.If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

If the CHO Modification Information IE is included in the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that the reserved CHO resources at the target node are changed and initiate a conditional handover preparation procedure towards the same target cells indicated in the Candidate Cells To Be Cancelled List IE in the CONDITIONAL HANDOVER CANCEL message.

#### 8.2.B.3	Unsuccessful Operation

Not applicable.

#### 8.2.B.4	Abnormal Conditions

If the CONDITIONAL HANDOVER CANCEL message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the CONDITIONAL HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the source NG-RAN node shall ignore those non-associated candidate cells.

Next change

9.1.1.BB	CONDITIONAL HANDOVER CANCEL

This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared conditional handover.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | - | - |
| CHO Modification Information | O |  | ENUMERATED (CHO-Modification Required, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is 16. |

Next change

id-FastMCGRecoveryRRCTransfer-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-CHOinformation-Req,

id-CHOinformation-Ack,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-DAPSResponseInfo,

id-CHO-ModificationInfo,

-- TEXT OMITTED –

ConditionalHandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetCellsToCancel							CRITICALITY reject		TYPE TargetCellList								PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore		TYPE CHO-ModificationInfo						PRESENCE optional},

...

}

-- TEXT OMITTED –

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																			ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= YY0

id-CHOinformation-Ack																				ProtocolIE-ID ::= YY1

id-targetCellsToCancel																				ProtocolIE-ID ::= YY2

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= YY3

id-procedureStage																					ProtocolIE-ID ::= YY4

id-DAPSInfo																							ProtocolIE-ID ::= YY5

id-DAPSResponseInfo																					ProtocolIE-ID ::= YY6

id-CHO-ModificationInfo																				ProtocolIE-ID ::= xxx

END

-- ASN1STOP

End of change