3GPP TSG-RAN3 Meeting #107-bis-e	R3-201968

E-meeting, 20 -30 April 2020

Title: 	(TP for WWC BL CR for TS 29.413) Support for interfacing trusted non-3GPP access networks to the 5GC

Source: 	Huawei, BT, Broadcom

Agenda item:	21.2

Document for:	Other

1. Introduction

The support of trusted non-3GPP access network to the 5GC was discussed at RAN3#107-e meeting, with the following remaining issue:

- It is FFS whether to include a list of identifiers (i.e. FQDNs or IP addresses) in the Identity Information.

This contribution intends to address the above issue and provide TPs for TS 38.413 and TS 29.413.

2. Discussion

- As specified in TS 29.510, the TNGF Identity Information or the TWIF Identity Information contains one of the addressing parameters (IPv4 address, IPv6 address or endpoint FQDN) at least for the coexist case.

##### 6.1.6.2.52 Type: TngfInfo Table 6.1.6.2.52-1: Definition of type TngfInfo Attribute name; Data type; P; Cardinality; Description. ipv4EndpointAddresses; array(Ipv4Addr); C; 1..N; Available endpoint IPv4 address(es) of the N3 terminations (NOTE 1).. ipv6EndpointAddresses; array(Ipv6Addr); C; 1..N; Available endpoint IPv6 address(es) of the N3 terminations (NOTE 1).. endpointFqdn; Fqdn; C; 0..1; Available endpoint FQDN of the N3 terminations (NOTE 1).. NOTE 1: At least one of the addressing parameters (ipv4address, ipv6adress or endpointFqdn) shall be included in the TngfInfo.. ##### …… ##### 6.1.6.2.61 Type: TwifInfo Table 6.1.6.2.61-1: Definition of type TwifInfo Attribute name; Data type; P; Cardinality; Description. ipv4EndpointAddresses; array(Ipv4Addr); C; 1..N; Available endpoint IPv4 address(es) of the N3 terminations (NOTE 1). ipv6EndpointAddresses; array(Ipv6Addr); C; 1..N; Available endpoint IPv6 address(es) of the N3 terminations (NOTE 1). endpointFqdn; Fqdn; C; 0..1; Available endpoint FQDN of the N3 terminations (NOTE 1). NOTE 1: At least one of the addressing parameters (ipv4address, ipv6adress or endpointFqdn) shall be included in the TwifInfo..

For RAN3, the easy way to address the above Editor’s Note is to refer the TNGF Identity Information or the TWIF Identity Information definition in TS 29.510. This can avoid duplicated definition in different specifications, and avoid potential synchronous update in case of future change.

Hence the following proposal can be made.

Add the reference to TS 29.510 for the TNGF Identity Information and the TWIF Identity Information.

3. Conclusion

This contribution discusses the further RAN impact to support the trusted non-3GPP access networks connecting to 5GC. Based on the discussion in this paper, the following proposal is proposed.

- Add the reference to TS 29.510 for the TNGF Identity Information and the TWIF Identity Information.

4. Reference

- TS 29.510 Network Function Repository Services; Stage 3

- C4-201066, N3 terminations of TWIF for UPF selection, Nokia, Nokia Shanghai Bell, Charter Communications, Huawei.

5. Annex – TP

Change Begins

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

/**Skip the unrevelant**/

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	MobilityRestriction List IE

-	Index to RAT/Frequency Selection Priority IE

UPLINK NAS TRANSPORT message:

-	W-AGF Identity Information IE: the information given within this IE between the W-AGF and the AMF contains a list of identifiers of N3 terminations at W-AGF as specified in TS 23.316 [x].

-	TNGF Identity Information IE: the information given within this IE between the TNGF and the AMF contains a list of identifiers of NG-U terminations at TNGF as specified in TS 23.502 [4].

-	TWIF Identity Information IE: the information given within this IE between the TWIF and the AMF contains a list of identifiers of NG-U terminations at TWIF as specified in TS 23.502 [4].

NG SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

/**Skip the unrevelant**/

End Change