3GPP TSG-RAN WG3 #107bis-e	R3-201997

E-meeting, 20th – 30th April 2020

Source:	CATT

Title:	(TP on SON BLCR for 38.423) TP on Load Metrics

Agenda Item:	10.2.2.1

Document for:	Discussion and decision

# Introduction

This is a TP based on the discussion paper R3-201996.

# Text proposal for TS 38.423

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.1.3.D	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to neighbouring NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Hardware Load Indicator [FFS] | O |  | 9.2.2.ww |  |  |  |
| Cell Measurement Result |  | 0..1 |  |  |  |  |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.2.yy |  |  |  |
| >>NG TNL Capacity Indicator [It is FFS if reported per cell] | O |  | 9.2.2.xx |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.zz |  |  |  |
| >>Slice Available Capacity | O |  | 9.2.2.jj |  |  |  |
| >>Number of Active UEs | O |  | INTEGER (0..65535,...) | Value 1 means 0.1 active UEs, value 2 means 0.2 active UEs, value n means n/10 Active UEs. |  |  |
| >> RRC Connections | O |  | 9.2.2.kk | Counts only the UEs configured with the current cell as the PCell. |  |  |

| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| --- | --- |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.2.yy	Radio Resource Status

[The information is FFS]

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Radio Resource Status Type | M |  |  |  |
| >ng-eNB |  |  |  |  |
| >>DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage |
| >>UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage |
| >>DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage |
| >>UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage |
| >>DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage |
| >>UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >gNB |  |  |  |  |
| >>SSB Area Radio Resource Status List |  | 1 |  |  |
| >>>SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage [FFS] |
| >>>>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage [FFS] |
| >>>>SSB Area SUL GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL GBR PRB usage [FFS] |
| >>>>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage [FFS] |
| >>>>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage [FFS] |
| >>>>SSB Area SUL non-GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL non-GBR PRB usage [FFS] |
| >>>>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage [FFS] |
| >>>>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage [FFS] |
| >>>>SSB Area SUL Total PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL Total PRB usage [FFS] |
| >>>>SSB Area DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>>>SSB Area UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>>>SSB Area SUL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.2.m Number of RRC Connections

The Number of RRC Connections IE indicates the absolute number of UEs in RRC_CONNECTED mode.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number of RRC Connections | M |  | INTEGER (0..65535,...) |  |  |  |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.3.4	PDU Definitions

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusUpdate-IEs}},

...

}

ResourceStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-HardwareLoadIndicator		CRITICALITY reject	TYPE HardwareLoadIndicator			PRESENCE optional}|--FFS

{ ID id-CellMeasurementResult		CRITICALITY ignore	TYPE CellMeasurementResult		PRESENCE optional},

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

9.3.5	Information Element definitions

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- C

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

CellToReport ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReport-Item

CellToReport-Item	::= SEQUENCE {

cell-ID									Target-CGI,

sSBToReport-List                        SSBToReport-List								OPTIONAL,

sliceToReport-List                      SliceToReport-List								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellToReport-Item-ExtIEs} }	OPTIONAL,

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- D

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

DL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- N

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

NumberofActiveUEs::= INTEGER (0..65535,...)

NoofRRCConnections ::= INTEGER (0..65535,...)

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

NG-eNB-RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage						DL-Total-PRB-usage,

uL-Total-PRB-usage						UL-Total-PRB-usage,

dL-scheduling-PDCCH-CCE-usage			DL-scheduling-PDCCH-CCE-usage					OPIONAL,

uL-scheduling-PDCCH-CCE-usage			UL-scheduling-PDCCH-CCE-usage					OPIONAL,

iE-Extensions							ProtocolExtensionContainer { { NG-eNB-RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

NG-eNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- S

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item	::= SEQUENCE {

sSBIndex								INTEGER(0..63),

ssb-Area-DL-GBR-PRB-usage	DL-GBR-PRB-usage,

ssb-Area-UL-GBR-PRB-usage	UL-GBR-PRB-usage,

ssb-Area-SUL-GBR-PRB-usage	UL-GBR-PRB-usage	OPIONAL,

ssb-Area-dL-non-GBR-PRB-usage		DL-non-GBR-PRB-usage,

ssb-Area-uL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage,

ssb-Area-SUL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage							OPIONAL,

ssb-Area-dL-Total-PRB-usage			DL-Total-PRB-usage,

ssb-Area-uL-Total-PRB-usage			UL-Total-PRB-usage,

ssb-Area-SUL-Total-PRB-usage		UL-Total-PRB-usage								OPIONAL,

ssb-DL-scheduling-PDCCH-CCE-usage	DL-scheduling-PDCCH-CCE-usage					OPIONAL,

ssb-UL-scheduling-PDCCH-CCE-usage	UL-scheduling-PDCCH-CCE-usage					OPIONAL,

ssb-SUL-scheduling-PDCCH-CCE-usage	UL-scheduling-PDCCH-CCE-usage					OPIONAL,

iE-Extensions						ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- U

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

UL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

//////////////////////////////////////////////////////////TP end//////////////////////////////////////////////////////////