3GPP TSG-RAN WG3 meeting #107bis-e	R3-202093

E-meeting, 20th – 30th April, 2020

Agenda Item:	13.3.3

Source: 	Huawei

Title:	(TP for NR_IAB BL CR for TS 38.401): IAB-node migration in EN-DC

Document for:	Agreement

- Introduction

In previous RAN3 meetings, the procedure about Intra-CU topology adaptation for IAB node in SA mode was discussed and has been captured in [1].

In addition, for the IAB node in EN-DC scenario, the intra-donor CU migration should also be considered, but haven’t been discussed yet. Therefore, in this paper, a procedure about IAB-node migration in EN-DC mode is provided, as shown in the appendix part.

Proposal 1: RAN3 agree the TP in appendix to include the IAB migration procedure for EN-DC scenario.

# References

- R3-201597. BL CR to 38401 Support for IAB. Huawei.

# Appendix: Text Proposal for NR-IAB BL CR for TS 38.463

------------------------------------------------Start of Text Proposal-----------------------------------------------------

8	Overall procedures in gNB-CU/gNB-DU Architecture

>>>> Unchanged parts are skipped<<<<

### 8.2.x	Intra-CU topology adaptation procedure

#### 8.2.x.1	Intra-CU topology adaptation procedure in SA

During the intra-CU topology adaptation in SA, both the source and the target parent node are served by the same IAB-donor-CU. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may further have common nodes with the target path. Figure 8.2.x-1 shows an example of the topology adaptation procedure, where the target parent node uses a different IAB-donor-DU than the source parent node.

>>>> Unchanged parts are skipped<<<<

#### 8.2.x.2	Intra-CU topology adaptation procedure in NSA using MCG SRB

This procedure is used for the case the migrating IAB-MT moves from source parent node to target parent node within the same IAB-donor-CU when only MCG SRB is available for IAB-node during EN-DC operation. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may further have common nodes with the target path. Figure 8.2.x.2-1 shows the topology adaptation procedure using MCG SRB of IAB-MT in EN-DC, where the target parent node uses a different IAB-donor-DU than the source parent node.

UE

Migrating IAB-node

Source parent IAB-node

Intermediate hop IAB-node on the source path

Source IAB-donor-DU

Target parent IAB-node

Intermediate hop IAB-node on the target path

Target IAB-donor-DU

IAB-donor-CU

Source Path

Target Path

MeNB

12. RRC Reconfiguration procedure is performed between the MeNB and the migrating IAB-MT

15. Configuration of BAP route and mapping rules along target path between migrating IAB-node and target IAB-donor-DU via target parent IAB-node. Configuration of UL mapping on migrating IAB-node.

16. Redirection of migrating IAB-node-DU’s F1 association to new TNL address(es).

19. Release of BAP route along source-path between migrating IAB-node and source IAB-donor-DU via source parent IAB-node.

Downlink User data

Uplink User data

1. ULInformationTransferMRDC (containing NR RRC MeasurementReport)

2. RRC TRANSFER

3. UE CONTEXT MODIFICATION REQUEST

4. UE CONTEXT MODIFICATION RESPONSE

5. UE CONTEXT SETUP REQUEST

6. UE CONTEXT SETUP RESPONSE

7. UE CONTEXT MODIFICATION REQUEST

8. UE CONTEXT MODIFICATION RESPONSE

10. SGNB MODIFICATION REQUEST

9. SGNB MODIFICATION REQUIRED

13. SGNB MODIFICATION CONFIRM

11. SGNB MODIFICATION REQUEST ACKNOWLEDGE

14. Random Access Procedure

Downlink User data

Uplink User data

17. UE CONTEXT RELEASE COMMAND

18. UE CONTEXT RELEASE COMPLETE

Figure 8.2.x.2-1: IAB intra-CU topology adaptation procedure using MCG SRB in EN-DC

1.	The migrating IAB-MT sends an ULInformationTransferMRDC message to the MeNB

2.	The MeNB sends RRC TRANSFER message to the IAB-donor-CU.

3.	The IAB-donor-CU may send UE CONTEXT MODIFICATION REQUEST message to the source parent node gNB-DU to query the latest SCG configuration.

4.	The source parent node gNB-DU responds with an UE CONTEXT MODIFICATION RESPONSE message that includes full configuration information.

5.	The IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node gNB-DU to create a UE context for migrating IAB-MT and setup one or more bearers. These bearers are used by the migrating IAB-MT for its own data and/or signalling traffic. The UE CONTEXT SETUP REQUEST message includes a CG ConfigInfo.

6.	The target parent node gNB-DU responds the IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

7.	The IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node gNB-DU, this message includes the Transmission Action Indicator which indicating the source parent node gNB-DU to stop the data transmission to the migrating IAB-node.

8.	The source parent node gNB-DU responds the IAB-donor-CU with a UE CONTEXT MODIFICATION RESPONSE message.

9. 	The IAB-donor-CU sends an SGNB MODIFICATION REQUIRED message to the MeNB.

10/11.	 The MeNB initiated SgNB Modification procedure may be triggered by the SgNB initiated SgNB Modification procedure (e.g. to provide information such as data forwarding addresses, new SN security key, measurement gap, etc...).

12.	The MeNB and the migrating IAB-MT perform RRC Connection Reconfiguration procedure.

13.	The MeNB sends an SGNB MODIFICATION CONFIRM message to the IAB-donor-CU.

14.	The migrating IAB-MT performs random access procedure at the target parent node gNB-DU.

15-19. The remaining steps of the procedure follows steps 11-15 of the intra-CU topology adaptation procedure in SA scenario as defined in clause 8.2.x.1.

#### 8.2.x.3	Intra-CU topology adaptation procedure in NSA using SCG SRB (SRB3)

This procedure is used for the case the migrating IAB-MT moves from source parent node to target parent node within the same IAB-donor-CU when SCG SRB (SRB3) is available for IAB-node during EN-DC operation. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may further have common nodes with the target path. The procedure is the same as intra-CU topology adaptation procedure in SA scenario as defined in clause 8.2.x.1 but the UE CONTEXT SETUP REQUEST message includes a CG-ConfigInfo in step 3.

------------------------------------------------End of Text Proposal-----------------------------------------------------