3GPP TSG-RAN WG3 Meeting #107bis-e	R3-202258

# Online Meeting, 20th – 24th April 2020

Agenda Item:	10.3.1

Source:	Ericsson

Title:	TP for introducing Area scope for neighbour cell configuration on NGAP

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we propose to introduce an Area scope configuration IE for neighbour cells, i.e. carrier frequencies limitations on NGAP, synching the 38.413 specification to RAN2 agreements.

# 2	Discussion

The area Configuration For Neighbour cells IE is required to log specific neighbour cells’ related measurements instead of all neighbour cells’ related measurements. . The areaConfigForNeighbour is applicable only for logged MDT.

The areaConfigForNeighbour information element in the latest appoved running 38.331 CR [1] is attached below

areaConfigForNeighbour information element

AreaConfigForNeighbour-r16 ::=   SEQUENCE {

dl-CarrierFreq	                   ARFCN-ValueNR,

frequencyBandList                MultiFrequencyBandListNR,

cellList                         SEQUENCE (SIZE (1..32)) OF  PhysCellId  OPTIONAL

}

If AreaConfigForNeighbour is configured, it indicates the frequency for which UE is requested to perform measurement logging for neighbour cells. UE should perform measurement logging for the frequency in SIB4 of the current serving cell whose DL-carrierfrequency and at least one FrequencyBandIndicator are included in the AreaConfigForNeighbour. If not configured, the UE should perform measurement logging for all the neighbour cells.

The information elements specified above are presented below:

#### MultiFrequencyBandListNR

The IE MultiFrequencyBandListNR is used to configure a list of one or multiple NR frequency bands.

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-START

MultiFrequencyBandListNR ::=        SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR

-- TAG-MULTIFREQUENCYBANDLISTNR-STOP

-- ASN1STOP

#### FreqBandIndicatorNR

The IE FreqBandIndicatorNR is used to convey an NR frequency band number as defined in TS 38.101-1 [15] and TS 38.101-2 [39].

-- ASN1START

-- TAG-FREQBANDINDICATORNR-START

FreqBandIndicatorNR ::=             INTEGER (1..1024)

-- TAG-FREQBANDINDICATORNR-STOP

-- ASN1STOP

#### ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2.

-- ASN1START

-- TAG-ARFCN-VALUENR-START

ARFCN-ValueNR ::=               INTEGER (0..maxNARFCN)

-- TAG-ARFCN-VALUENR-STOP

-- ASN1STOP

Based on the above the Area configuration for neighbour cells should be enabled for the logged MDT configuration within the MDT Configuration-NR. Taking into accout the input from RAN2 the Area configuration for neighbour cells IE should be specified as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Cell List | O | 1 .. <maxnoofCellID> |  |  |
| >PCI | M |  | INTEGER (0..1007) | Physical Cell Identity of the neighbour cell |
| DL Carrier Frequency | M |  | INTEGER (0..maxNARFCN) | This IE is used to indicate the ARFCN applicable for a downlink (TDD) NR global frequency raster, as defined in TS 38.101-1 and TS 38.101-2, clause 5.4.2. The maxNARFCN is the Maximum value of the NR carrier frequency |
| MultiFrequency Band List NR | M | 1 .. <maxnoofFreqforMDT> |  | This IE is used to configure a list of one or multiple NR frequency bands |
| >Frequency Band Indicator NR |  |  | INTEGER (1..1024) | This IE is used to convey an NR frequency band number as defined in TS 38.101-1 and TS 38.101-2. |

The MDT Configuration-NR IE should be modified to inlcue the above Area configuration for neighbour cells IE as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…) |  | - | - |
| CHOICE Area Scope of MDT | M |  | FFS |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  |  |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [x]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration. Value “1” indicates “activate” and value “0” indicates “do not activate”. |  |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.x1 |  |  |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.x4 |  |  |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.x5 |  |  |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.x6 |  |  |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.x7 |  |  |  |
| >>M8 Configuration | O |  | 9.3.1.x8 |  |  |  |
| >>M9 Configuration | O |  | 9.3.1.x9 |  |  |  |
| >>MDT Location Information | O |  | 9.3.1.x10 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x11 (FFS: pending to RAN2) |  |  |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (1.28, 2.56, 5.12, 10.24, 20.48, 30.72, 40.96 and 61.44) | This IE is defined in TS 38.331 [18]. Unit: [second] |  |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120) | This IE is defined in TS 38.331 [18]. Unit: [minute]. |  |  |
| >>CHOICE Report Type | M |  |  |  |  |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>> Logged Event Trigger Config | M |  | 9.3.1.x14 |  |  |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.x11 |  |  |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.x12 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x13 |  |  |  |
| >> Area configuration for neighbour | O |  | 9.3.1.x14 |  |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.3.1.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

- Update the logged MDT Configuration-NR IE by introducing the Area configuration for neighbour cell IE for logged MDT configurations

Agree on updating the MDT BL CR for TS 38.413 as is proposed below.

# 3	Conclusion

Agree on the proposals below

- Update the logged MDT Configuration-NR IE by introducing the Area configuration for neighbour cell IE for logged MDT configurations

Agree on updating the MDT BL CR for TS 38.413 as is proposed below.

# 4	References

- [NR/MDT] running 38.331 CR to support SON/MDT

# TP for MDT BL CR for TS 38.413

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xx4	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…) |  | - | - |
| CHOICE Area Scope of MDT | M |  | FFS |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  |  |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [x]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration. Value “1” indicates “activate” and value “0” indicates “do not activate”. |  |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.x1 |  |  |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.x4 |  |  |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.x5 |  |  |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.x6 |  |  |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.x7 |  |  |  |
| >>M8 Configuration | O |  | 9.3.1.x8 |  |  |  |
| >>M9 Configuration | O |  | 9.3.1.x9 |  |  |  |
| >>MDT Location Information | O |  | 9.3.1.x10 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x11 (FFS: pending to RAN2) |  |  |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (1.28, 2.56, 5.12, 10.24, 20.48, 30.72, 40.96 and 61.44) | This IE is defined in TS 38.331 [18]. Unit: [second] |  |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120) | This IE is defined in TS 38.331 [18]. Unit: [minute]. |  |  |
| >>CHOICE Report Type | M |  |  |  |  |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>> Logged Event Trigger Config | M |  | 9.3.1.x14 |  |  |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.x11 |  |  |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.x12 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x13 |  |  |  |
| >> Area configuration for neighbour | O |  | 9.3.1.x14 |  |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.3.1.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x14		Area configuration for neighbour cells

This IE indicates the frequency for which UE is requested to perform measurement logging for neighbour cells. UE should perform measurement logging for the frequency in SIB4 of the current serving cell whose DL-carrierfrequency and at least one FrequencyBandIndicator are included in the AreaConfigForNeighbour. If not configured, the UE should perform measurement logging for all the neighbour cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Cell List | O | 1 .. <maxnoofCellID> |  |  |
| >PCI | M |  | INTEGER (0..1007) | Physical Cell Identity of the neighbour cell |
| DL Carrier Frequency | M |  | INTEGER (0..maxNARFCN) | This IE is used to indicate the ARFCN applicable for a downlink (TDD) NR global frequency raster, as defined in TS 38.101-1 and TS 38.101-2, clause 5.4.2. The maxNARFCN is the Maximum value of the NR carrier frequency |
| MultiFrequency Band List NR | M | 1 .. <maxnoofFreqforMDT> |  | This IE is used to configure a list of one or multiple NR frequency bands |
| >Frequency Band Indicator NR |  |  | INTEGER (1..1024) | This IE is used to convey an NR frequency band number as defined in TS 38.101-1 and TS 38.101-2. |

| Range bound | Explanation |
| --- | --- |
| maxnoofFreqforMDT | Maximum no. of neighbour frequencies subject for MDT scope. Value is 8. |
| maxnoofCellIDforMDT | Maximum no. of neighbour cell PCI. Value is 32. |

<<<<<<<<<<<<<<<<<<<< End of 2nd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 3rd Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-Cause,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-ExtendedRATRestrictionInformation,

id-GUAMIType,

id-LastEUTRAN-PLMNIdentity,

id-LocationReportingAdditionalInfo,

id-MaximumIntegrityProtectedDataRate-DL,

id-NetworkInstance,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-PDUSessionAggregateMaximumBitRate,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PSCellInformation,

id-QosFlowAddOrModifyRequestList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-RAT-Information,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TargetRNC-ID,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-MDTConfiguration,

maxnoofAllowedAreas,

maxnoofAllowedS-NSSAIs,

maxnoofBPLMNs,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofForbTACs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNGConnectionsToReset,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofQosFlows,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofTACs,

maxnoofTAIforInactive,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofBluetoothName,

maxnoofWLANName,

maxnoofMDTPLMNs,

maxnoofSensorName,

maxnoofFreqforMDT,

maxNRARFCN

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

#### -- A

-- TEXT OMITTED –

AreaOfInterestTAIItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestTAIItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestTAIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfNeighCellsConfig ::=   SEQUENCE {

physcellist						PhysCellList		OPTIONAL,

dl-CarrierFreq					NRARFCN,

multiFrequencyBandListNR		MultiFrequencyBandListNR,

iE-Extensions			ProtocolExtensionContainer { { AreaScopeOfNeighCellsConfig -ExtIEs} }	OPTIONAL,

...

}

AreaScopeOfNeighCellsConfig -ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- TEXT OMITTED –

LoggedMDTNr ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

loggedMDTTrigger					LoggedMDTTrigger,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration	OPTIONAL,

wLANMeasurementConfiguration		WLANMeasurementConfiguration		OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration		OPTIONAL,

areaScopeofNeighCellsConfig			AreaScopeOfNeighCellsConfig		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {LoggedMDTNr-ExtIEs} } OPTIONAL,

...

}

LoggedMDTNr-ExtIEs	NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- TEXT OMITTED –

MDT-Location-Info ::= BIT STRING (SIZE (8))

MultiFrequencyBandListNR ::= SEQUENCE (SIZE(1..maxnoofFreqforMDT)) OF MultiFrequencyBandIndicator

MultiFrequencyBandIndicator ::= INTEGER (1..1024)

NotificationControl ::= ENUMERATED {

notification-requested,

...

}

NRARFCN ::= INTEGER (0.. maxNRARFCN)

NRCellIdentity ::= BIT STRING (SIZE(36))

-- TEXT OMITTED –

PeriodicRegistrationUpdateTimer ::= BIT STRING (SIZE(8))

PhysCellID = INTEGER (0..1007)

PhysCellList = SEQUENCE (SIZE (1..32)) OF PhysCellID

PLMNIdentity ::= OCTET STRING (SIZE(3))

-- TEXT OMITTED –

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

id-DownlinkNASTransport						ProcedureCode ::= 4

id-DownlinkNonUEAssociatedNRPPaTransport	ProcedureCode ::= 5

id-DownlinkRANConfigurationTransfer			ProcedureCode ::= 6

id-DownlinkRANStatusTransfer				ProcedureCode ::= 7

id-DownlinkUEAssociatedNRPPaTransport		ProcedureCode ::= 8

id-ErrorIndication							ProcedureCode ::= 9

id-HandoverCancel							ProcedureCode ::= 10

id-HandoverNotification						ProcedureCode ::= 11

id-HandoverPreparation						ProcedureCode ::= 12

id-HandoverResourceAllocation				ProcedureCode ::= 13

id-InitialContextSetup						ProcedureCode ::= 14

id-InitialUEMessage							ProcedureCode ::= 15

id-LocationReportingControl					ProcedureCode ::= 16

id-LocationReportingFailureIndication		ProcedureCode ::= 17

id-LocationReport							ProcedureCode ::= 18

id-NASNonDeliveryIndication					ProcedureCode ::= 19

id-NGReset									ProcedureCode ::= 20

id-NGSetup									ProcedureCode ::= 21

id-OverloadStart							ProcedureCode ::= 22

id-OverloadStop								ProcedureCode ::= 23

id-Paging									ProcedureCode ::= 24

id-PathSwitchRequest						ProcedureCode ::= 25

id-PDUSessionResourceModify					ProcedureCode ::= 26

id-PDUSessionResourceModifyIndication		ProcedureCode ::= 27

id-PDUSessionResourceRelease				ProcedureCode ::= 28

id-PDUSessionResourceSetup					ProcedureCode ::= 29

id-PDUSessionResourceNotify					ProcedureCode ::= 30

id-PrivateMessage							ProcedureCode ::= 31

id-PWSCancel								ProcedureCode ::= 32

id-PWSFailureIndication						ProcedureCode ::= 33

id-PWSRestartIndication						ProcedureCode ::= 34

id-RANConfigurationUpdate					ProcedureCode ::= 35

id-RerouteNASRequest						ProcedureCode ::= 36

id-RRCInactiveTransitionReport				ProcedureCode ::= 37

id-TraceFailureIndication					ProcedureCode ::= 38

id-TraceStart								ProcedureCode ::= 39

id-UEContextModification					ProcedureCode ::= 40

id-UEContextRelease							ProcedureCode ::= 41

id-UEContextReleaseRequest					ProcedureCode ::= 42

id-UERadioCapabilityCheck					ProcedureCode ::= 43

id-UERadioCapabilityInfoIndication			ProcedureCode ::= 44

id-UETNLABindingRelease						ProcedureCode ::= 45

id-UplinkNASTransport						ProcedureCode ::= 46

id-UplinkNonUEAssociatedNRPPaTransport		ProcedureCode ::= 47

id-UplinkRANConfigurationTransfer			ProcedureCode ::= 48

id-UplinkRANStatusTransfer					ProcedureCode ::= 49

id-UplinkUEAssociatedNRPPaTransport			ProcedureCode ::= 50

id-WriteReplaceWarning						ProcedureCode ::= 51

id-SecondaryRATDataUsageReport				ProcedureCode ::= 52

id-UplinkRIMInformationTransfer				ProcedureCode ::= 53

id-DownlinkRIMInformationTransfer			ProcedureCode ::= 54

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedS-NSSAIs				INTEGER ::= 8

maxnoofBPLMNs						INTEGER ::= 12

maxnoofCellIDforWarning				INTEGER ::= 65535

maxnoofCellinAoI					INTEGER ::= 256

maxnoofCellinEAI					INTEGER ::= 65535

maxnoofCellinTAI					INTEGER ::= 65535

maxnoofCellsingNB					INTEGER ::= 16384

maxnoofCellsinngeNB					INTEGER ::= 256

maxnoofCellsinUEHistoryInfo			INTEGER ::= 16

maxnoofCellsUEMovingTrajectory		INTEGER ::= 16

maxnoofDRBs							INTEGER ::= 32

maxnoofEmergencyAreaID				INTEGER ::= 65535

maxnoofEAIforRestart				INTEGER ::= 256

maxnoofEPLMNs						INTEGER ::= 15

maxnoofEPLMNsPlusOne				INTEGER ::= 16

maxnoofE-RABs						INTEGER ::= 256

maxnoofErrors						INTEGER ::= 256

maxnoofForbTACs						INTEGER ::= 4096

maxnoofMultiConnectivity				INTEGER ::= 4

maxnoofMultiConnectivityMinusOne	INTEGER ::= 3

maxnoofNGConnectionsToReset			INTEGER ::= 65536

maxnoofPDUSessions					INTEGER ::= 256

maxnoofPLMNs						INTEGER ::= 12

maxnoofQosFlows						INTEGER ::= 64

maxnoofRANNodeinAoI					INTEGER ::= 64

maxnoofRecommendedCells				INTEGER ::= 16

maxnoofRecommendedRANNodes			INTEGER ::= 16

maxnoofAoI							INTEGER ::= 64

maxnoofServedGUAMIs					INTEGER ::= 256

maxnoofSliceItems					INTEGER ::= 1024

maxnoofTACs							INTEGER ::= 256

maxnoofTAIforInactive				INTEGER ::= 16

maxnoofTAIforPaging					INTEGER ::= 16

maxnoofTAIforRestart				INTEGER ::= 2048

maxnoofTAIforWarning				INTEGER ::= 65535

maxnoofTAIinAoI						INTEGER ::= 16

maxnoofTimePeriods					INTEGER ::= 2

maxnoofTNLAssociations				INTEGER ::= 32

maxnoofXnExtTLAs					INTEGER ::= 16

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofMDTPLMNs						INTEGER ::= 16

maxnoofCellIDforMDT					INTEGER ::= 32

maxnoofTAforMDT						INTEGER ::= 8

maxnoofBluetoothName				INTEGER ::= 4

maxnoofWLANName						INTEGER ::= 4

maxnoofSensorName					INTEGER ::= 3

maxnoofFreqforMDT					INTEGER ::= 8

maxNRARFCN							INTEGER ::= 3279165

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

id-AMF-TNLAssociationToAddList							ProtocolIE-ID ::= 6

id-AMF-TNLAssociationToRemoveList						ProtocolIE-ID ::= 7

id-AMF-TNLAssociationToUpdateList						ProtocolIE-ID ::= 8

id-AMFTrafficLoadReductionIndication					ProtocolIE-ID ::= 9

id-AMF-UE-NGAP-ID										ProtocolIE-ID ::= 10

id-AssistanceDataForPaging								ProtocolIE-ID ::= 11

id-BroadcastCancelledAreaList							ProtocolIE-ID ::= 12

id-BroadcastCompletedAreaList							ProtocolIE-ID ::= 13

id-CancelAllWarningMessages								ProtocolIE-ID ::= 14

id-Cause												ProtocolIE-ID ::= 15

id-CellIDListForRestart									ProtocolIE-ID ::= 16

id-ConcurrentWarningMessageInd							ProtocolIE-ID ::= 17

id-CoreNetworkAssistanceInformationForInactive			ProtocolIE-ID ::= 18

id-CriticalityDiagnostics								ProtocolIE-ID ::= 19

id-DataCodingScheme										ProtocolIE-ID ::= 20

id-DefaultPagingDRX										ProtocolIE-ID ::= 21

id-DirectForwardingPathAvailability						ProtocolIE-ID ::= 22

id-EmergencyAreaIDListForRestart						ProtocolIE-ID ::= 23

id-EmergencyFallbackIndicator							ProtocolIE-ID ::= 24

id-EUTRA-CGI											ProtocolIE-ID ::= 25

id-FiveG-S-TMSI											ProtocolIE-ID ::= 26

id-GlobalRANNodeID										ProtocolIE-ID ::= 27

id-GUAMI												ProtocolIE-ID ::= 28

id-HandoverType											ProtocolIE-ID ::= 29

id-IMSVoiceSupportIndicator								ProtocolIE-ID ::= 30

id-IndexToRFSP											ProtocolIE-ID ::= 31

id-InfoOnRecommendedCellsAndRANNodesForPaging			ProtocolIE-ID ::= 32

id-LocationReportingRequestType							ProtocolIE-ID ::= 33

id-MaskedIMEISV											ProtocolIE-ID ::= 34

id-MessageIdentifier									ProtocolIE-ID ::= 35

id-MobilityRestrictionList								ProtocolIE-ID ::= 36

id-NASC													ProtocolIE-ID ::= 37

id-NAS-PDU												ProtocolIE-ID ::= 38

id-NASSecurityParametersFromNGRAN						ProtocolIE-ID ::= 39

id-NewAMF-UE-NGAP-ID									ProtocolIE-ID ::= 40

id-NewSecurityContextInd								ProtocolIE-ID ::= 41

id-NGAP-Message											ProtocolIE-ID ::= 42

id-NGRAN-CGI											ProtocolIE-ID ::= 43

id-NGRANTraceID											ProtocolIE-ID ::= 44

id-NR-CGI												ProtocolIE-ID ::= 45

id-NRPPa-PDU											ProtocolIE-ID ::= 46

id-NumberOfBroadcastsRequested							ProtocolIE-ID ::= 47

id-OldAMF												ProtocolIE-ID ::= 48

id-OverloadStartNSSAIList								ProtocolIE-ID ::= 49

id-PagingDRX											ProtocolIE-ID ::= 50

id-PagingOrigin											ProtocolIE-ID ::= 51

id-PagingPriority										ProtocolIE-ID ::= 52

id-PDUSessionResourceAdmittedList						ProtocolIE-ID ::= 53

id-PDUSessionResourceFailedToModifyListModRes			ProtocolIE-ID ::= 54

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 55

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 56

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 57

id-PDUSessionResourceFailedToSetupListSURes				ProtocolIE-ID ::= 58

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListCxtRelCpl						ProtocolIE-ID ::= 60

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61

id-PDUSessionResourceModifyListModCfm					ProtocolIE-ID ::= 62

id-PDUSessionResourceModifyListModInd					ProtocolIE-ID ::= 63

id-PDUSessionResourceModifyListModReq					ProtocolIE-ID ::= 64

id-PDUSessionResourceModifyListModRes					ProtocolIE-ID ::= 65

id-PDUSessionResourceNotifyList							ProtocolIE-ID ::= 66

id-PDUSessionResourceReleasedListNot					ProtocolIE-ID ::= 67

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 68

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 69

id-PDUSessionResourceReleasedListRelRes					ProtocolIE-ID ::= 70

id-PDUSessionResourceSetupListCxtReq					ProtocolIE-ID ::= 71

id-PDUSessionResourceSetupListCxtRes					ProtocolIE-ID ::= 72

id-PDUSessionResourceSetupListHOReq						ProtocolIE-ID ::= 73

id-PDUSessionResourceSetupListSUReq						ProtocolIE-ID ::= 74

id-PDUSessionResourceSetupListSURes						ProtocolIE-ID ::= 75

id-PDUSessionResourceToBeSwitchedDLList					ProtocolIE-ID ::= 76

id-PDUSessionResourceSwitchedList						ProtocolIE-ID ::= 77

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 78

id-PDUSessionResourceToReleaseListRelCmd				ProtocolIE-ID ::= 79

id-PLMNSupportList										ProtocolIE-ID ::= 80

id-PWSFailedCellIDList									ProtocolIE-ID ::= 81

id-RANNodeName											ProtocolIE-ID ::= 82

id-RANPagingPriority									ProtocolIE-ID ::= 83

id-RANStatusTransfer-TransparentContainer				ProtocolIE-ID ::= 84

id-RAN-UE-NGAP-ID										ProtocolIE-ID ::= 85

id-RelativeAMFCapacity									ProtocolIE-ID ::= 86

id-RepetitionPeriod										ProtocolIE-ID ::= 87

id-ResetType											ProtocolIE-ID ::= 88

id-RoutingID											ProtocolIE-ID ::= 89

id-RRCEstablishmentCause								ProtocolIE-ID ::= 90

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 91

id-RRCState												ProtocolIE-ID ::= 92

id-SecurityContext										ProtocolIE-ID ::= 93

id-SecurityKey											ProtocolIE-ID ::= 94

id-SerialNumber											ProtocolIE-ID ::= 95

id-ServedGUAMIList										ProtocolIE-ID ::= 96

id-SliceSupportList										ProtocolIE-ID ::= 97

id-SONConfigurationTransferDL							ProtocolIE-ID ::= 98

id-SONConfigurationTransferUL							ProtocolIE-ID ::= 99

id-SourceAMF-UE-NGAP-ID									ProtocolIE-ID ::= 100

id-SourceToTarget-TransparentContainer					ProtocolIE-ID ::= 101

id-SupportedTAList										ProtocolIE-ID ::= 102

id-TAIListForPaging										ProtocolIE-ID ::= 103

id-TAIListForRestart									ProtocolIE-ID ::= 104

id-TargetID												ProtocolIE-ID ::= 105

id-TargetToSource-TransparentContainer					ProtocolIE-ID ::= 106

id-TimeToWait											ProtocolIE-ID ::= 107

id-TraceActivation										ProtocolIE-ID ::= 108

id-TraceCollectionEntityIPAddress						ProtocolIE-ID ::= 109

id-UEAggregateMaximumBitRate							ProtocolIE-ID ::= 110

id-UE-associatedLogicalNG-connectionList				ProtocolIE-ID ::= 111

id-UEContextRequest										ProtocolIE-ID ::= 112

id-UE-NGAP-IDs											ProtocolIE-ID ::= 114

id-UEPagingIdentity										ProtocolIE-ID ::= 115

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 116

id-UERadioCapability									ProtocolIE-ID ::= 117

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 118

id-UESecurityCapabilities								ProtocolIE-ID ::= 119

id-UnavailableGUAMIList									ProtocolIE-ID ::= 120

id-UserLocationInformation								ProtocolIE-ID ::= 121

id-WarningAreaList										ProtocolIE-ID ::= 122

id-WarningMessageContents								ProtocolIE-ID ::= 123

id-WarningSecurityInfo									ProtocolIE-ID ::= 124

id-WarningType											ProtocolIE-ID ::= 125

id-AdditionalUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 126

id-DataForwardingNotPossible							ProtocolIE-ID ::= 127

id-DL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 128

id-NetworkInstance										ProtocolIE-ID ::= 129

id-PDUSessionAggregateMaximumBitRate					ProtocolIE-ID ::= 130

id-PDUSessionResourceFailedToModifyListModCfm			ProtocolIE-ID ::= 131

id-PDUSessionResourceFailedToSetupListCxtFail			ProtocolIE-ID ::= 132

id-PDUSessionResourceListCxtRelReq						ProtocolIE-ID ::= 133

id-PDUSessionType										ProtocolIE-ID ::= 134

id-QosFlowAddOrModifyRequestList						ProtocolIE-ID ::= 135

id-QosFlowSetupRequestList								ProtocolIE-ID ::= 136

id-QosFlowToReleaseList									ProtocolIE-ID ::= 137

id-SecurityIndication									ProtocolIE-ID ::= 138

id-UL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 139

id-UL-NGU-UP-TNLModifyList								ProtocolIE-ID ::= 140

id-WarningAreaCoordinates								ProtocolIE-ID ::= 141

id-PDUSessionResourceSecondaryRATUsageList				ProtocolIE-ID ::= 142

id-HandoverFlag											ProtocolIE-ID ::= 143

id-SecondaryRATUsageInformation							ProtocolIE-ID ::= 144

id-PDUSessionResourceReleaseResponseTransfer			ProtocolIE-ID ::= 145

id-RedirectionVoiceFallback								ProtocolIE-ID ::= 146

id-UERetentionInformation								ProtocolIE-ID ::= 147

id-S-NSSAI												ProtocolIE-ID ::= 148

id-PSCellInformation									ProtocolIE-ID ::= 149

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType						ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation						ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-ManagementBasedMDTPLMNList							ProtocolIE-ID ::= 888	--will be assigned by MCC

id-MDTConfiguration										ProtocolIE-ID ::= 889

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

END

-- ASN1STOP