3GPP TSG-RAN3 Meeting #107bis-e	R3-202338

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.413): Mobility restriction list for NPN

Source: 	Huawei

Agenda item:	16.3.2

Document for:	Discussion

1. Introduction

Last RAN3#107e meeting discussed the mobility restriction list and capture the agreements to the BLCR. This paper discusses the remaining issues.

2. Discussion

2.1 The application of mobility restriction for NPN.

Besides the NPN mobility information, the following information is included in the mobility restriction list. It is needed to discuss whether this information can be applied to NPN.

| Information | Applicable to SNPN? | Applicable to PNI-NPN? |
| --- | --- | --- |
| Equivalent PLMNs | No | Yes |
| RAT Restrictions | e-UTRA is restricted Note: this may depend on RAN2 first study | e-UTRA is restricted Note: this may depend on RAN2 first study |
| Forbidden Area Information | Yes | Yes |
| Service Area Information | Yes | Yes |
| Last E-UTRAN PLMN Identity | No | No |
| Core Network Type Restriction for Serving PLMN | EPC is always forbidden. | EPC could be forbidden or not forbidden. |
| Core Network Type Restriction for Equivalent PLMNs | No | Yes |

Currently the BL CR for XnAP has captured a Note as follows. Then similarly, the NGAP BL CR should add such a similar note.

- NOTE: if the Serving NID IE is included in this IE, the Equivalent PLMNs IE and the Last E-UTRAN PLMN IE are not included and the Core Network Type Restriction for Serving PLMN IE is set to "EPCForbidden", following principles specified in TS 23.501 [9].

In addition, the node can be further updated to include: the RAT restrictions for SNPN.

Add a note on the application of mobility restriction for NPN in the NG BLCR. And update the note in the Xn BLCR.

2.2 Procedural texts for NPN

Currently some procedural texts are described in RAN3 specifications when the NG-RAN receives the mobility restriction list and performs the following actions.

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation;

-	assign proper RNA(s) for the UE when moving the UE to RRC_INACTIVE state.

Clearly the above actions can be applied to NPNs (either SNPN or PNI-NPN). Since this NPN mobility information is optional, it is beneficial to indicate in the procedural texts.

Add the procedural texts on NPN mobility information in NGAP BLCR and XnAP BLCR.

3. Conclusion

In this paper, we have discussed issues on the mobility restriction list for NPN, and have the following proposals,

- Add a note on the application of mobility restriction for NPN in the NG BLCR. And update the note in the Xn BLCR.

- Add the procedural texts on NPN mobility information in NGAP BLCR and XnAP BLCR.

The corresponding TP for TS 38.413 is provided in Annex, and the corresponding TP for TS 38.423 is provided in [1].

4. Reference

- R3-202339, (TP for NPN BL CR for TS 38.423): Mobility restriction list for NPN, Huawei

Annex – TP for TS 38.413 (on the top of BL R3-201592)

Change Begins

8.3.1	Initial Context Setup

<unchanged Text Omitted>

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

<unchanged Text Omitted>

The NG-RAN node shall use the information in the Mobility Restriction List IE if present in the INITIAL CONTEXT SETUP REQUEST message to

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation;

-	assign proper RNA(s) for the UE when moving the UE to RRC_INACTIVE state;

-	perform the above actions for NPN if the NPN Mobility Information IE is contained in the Mobility Restriction List IE.

<Unchanged Text Omitted>

Next Change

8.4.2	Handover Resource Allocation

<Unchanged Text Omitted>

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

<Unchanged Text Omitted>

The target NG-RAN node shall use the information in the Mobility Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the target NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation;

-	assign proper RNA(s) for the UE when moving the UE to RRC_INACTIVE state;

-	perform the above actions for NPN if the NPN Mobility Information IE is contained in the Mobility Restriction List IE.

<Unchanged Text Omitted>

Next Change

8.6.2	Downlink NAS Transport

<Unchanged Text Omitted>

8.6.2.2	Successful Operation

Figure 8.6.2.2-1: Downlink NAS transport

The AMF initiates the procedure by sending a DOWNLINK NAS TRANSPORT message to the NG-RAN node. If the UE-associated logical NG-connection is not established, the AMF shall allocate a unique AMF UE NGAP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the AMF UE NGAP ID IE in the DOWNLINK NAS TRANSPORT message, the NG-RAN node establishes the UE-associated logical NG-connection.

If the RAN Paging Priority IE is included in the DOWNLINK NAS TRANSPORT message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

The NAS-PDU IE contains an AMF – UE message that is transferred without interpretation in the NG-RAN node.

If the Mobility Restriction List IE is contained in the DOWNLINK NAS TRANSPORT message, the NG-RAN node shall overwrite any previously stored mobility restriction information in the UE context. The NG-RAN node shall use the information in the Mobility Restriction List IE if present in the DOWNLINK NAS TRANSPORT message to:

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation;

-	assign proper RNA(s) for the UE when moving the UE to RRC_INACTIVE state;

-	perform the above actions for NPN if the NPN Mobility Information IE is contained in the Mobility Restriction List IE.

<Unchanged Text Omitted>

Next Change

9.3.1	Radio Network Layer Related IEs

<Unchanged Text Omitted>

9.3.1.85	Mobility Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the NG-RAN provides information about the target of the mobility action towards the UE, e.g., handover, or for SCG selection during dual connectivity operation or for assigning proper RNAs or for above aspects for NPN. NG-RAN behaviour upon receiving this IE is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.3.3.5 |  | - |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of "equivalent PLMNs" as defined in TS 24.501 [26]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the Serving PLMN and Equivalent PLMNs. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >RAT Restriction Information | M |  | BIT STRING { e-UTRA (0), nR (1) ,nR-unlicensed (2)) } (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bits 2-7 reserved for future use. | - |  |
| >Extended RAT Restriction Information | O |  | 9.3.1.126 | If this IE is included, the RAT Restriction Information IE is ignored. | YES | ignore |
| Forbidden Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the forbidden TAI. | - |  |
| Service Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Service Area Restriction information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Allowed TACs |  | 0..<maxnoofAllowedAreas> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the allowed TAI. | - |  |
| >Not Allowed TACs |  | 0..<maxnoofAllowedAreas> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the not-allowed TAI. | - |  |
| Last E-UTRAN PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the E-UTRAN PLMN ID from where the UE formerly handed over to 5GS and which is preferred in case of subsequent mobility to EPS. | YES | ignore |
| Core Network Type Restriction for Serving PLMN | O |  | ENUMERATED( EPCForbidden,…) | Indicates whether the UE is restricted to connect to EPC for the Serving PLMN as specified in TS 23.501 [9]. | YES | ignore |
| Core Network Type Restriction for Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  |  | YES | ignore |
| >PLMN Identity | M |  | 9.3.3.5 | Includes any of the Equivalent PLMNs listed in the Mobility Restriction List IE for which CN Type restriction applies as specified in TS 23.501 [9]. | - |  |
| >Core Network Type Restriction | M |  | ENUMERATED( EPCForbidden, 5GCForbidden,…) | Indicates whether the UE is restricted to connect to EPC or to 5GC for this PLMN. |  |  |
| NPN Mobility Information | O |  | 9.3.1.X2 | NOTE: if the Serving NID IE is included in this IE, the Equivalent PLMNs IE and the Last E-UTRAN PLMN IE are not included, and the RAT Restriction Information is set as e-URTA restricted, and the Core Network Type Restriction for Serving PLMN IE is set to "EPCForbidden", following principles specified in TS 23.501 [9]. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMNs. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of allowed PLMNs. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofAllowedAreas | Maximum no. of allowed or not allowed Tracking Areas. Value is 16. |

Change Ends