3GPP TSG-RAN3 Meeting #107-BIS-e	R3-202710

Electronic meeting, 20th - 30th April 2020

3GPP RAN WG2 Meeting #109bis-e	R2-2003989

Online, 20th – 30th April, 2020

Title:	Response LS on SRS and SSB configuration for NR Positioning

Response to:	R1-1911634

Release:	Release 16

Work Item:	NR_pos-Core

Source:	RAN2

To:	RAN3

Cc:	RAN1

Contact Person:

#### Name:	Yi Guo

E-mail Address:	yi.guo@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 has discussed the issue of support of aperiodic SRS. From RAN2 perspective, the RRC signalling can currently configure UE with aperiodic SRS for positioning, but RAN2 would like to check with RAN3 if it is feasible to complete the work on aperiodic SRS from NRPPa perspective.

RAN2 also discussed the support of semi-persistent SRS and concluded the following:

Agreements:

The Activation/Deactivation of the SP SRS is recommended by LMF to gNB; and the activation/deactivation MAC CE is sent by gNB to UE (no stage 3 impact for RAN2).

Leave the design of NRPPa message for SRS triggering to RAN3 while capture the purpose and content of the message at stage 2 level in RAN2.

In addition, RAN2 discussed the spatial relation of SRS, and agreed the following:

Agreements:

Spatial relation of SRS is recommended by the LMF and decided by the gNB.  It is up to gNB implementationwhether to follow the LMF recommendation.  The gNB informs the LMF of its decision.

UE does not report RSRP of DL-PRS in RRC procedures for SRS configuration.

Keep the current SSB configuration for the DL-only positioning in the LPP message.

Keep the current configuration of SSB in RRC for UL-only positioning. This means that the RRC configuration can carry the full SSB configuration or SSB index and PCI.

For the assistance information in NRPPa for SSB configuration for UL-only positioning, it should include both TF configuration and SSB index in the NRPPa message.

RAN2 understand that the design of NRPPa message is needed to realize above agreements, and therefore respectfully ask RAN3 to take RAN2 agreements into account in their further work.

2. Actions:

To RAN3:

ACTION 1:

RAN2 respectfully ask RAN3 if it is feasible to complete the work on aperiodic SRS from RAN3 perspective, and feedback if any;

ACTION 2:

RAN2 respectfully ask RAN3 to take into account corresponding agreements made by RAN2 in their further work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #110e	01 – 12 June 2020, E-Meeting

TSG RAN WG2 Meeting #111	24 – 28 August 2020, Toulouse, France