3GPP TSG-RAN WG3 Meeting#107bis-e	R3-202597

E-Meeting, 20th – 30th April 2020

Title: 	(TP to BL CR#0153 “Common CP/UP aspects of CIoT UEs when connected to 5GC” for TS38.413) Support of Coverage Enhancement

Source: 	Huawei, Ericsson, Nokia, Nokia Shanghai Bell

Agenda item:	14.3.2.3

Document for:	other

1. Introduction

In this contribution, based on the latest status of BL CRs and SA2 TS, we further discuss the support of coverage enhancement in NGAP, and provide the corresponding TP to the BL CR#0153.

2. Discussion

When NB-IoT and MTC connecting to 5GC, the following descriptions can be found in TS23.502:

If the AMF has Paging Assistance Data for CE capable UE stored in the UE Context in AMF, and Enhanced Coverage is not restricted for the UE then the AMF shall include Paging Assistance Data for CE capable UE in the N2 paging message for all NG-RAN nodes selected by the AMF for paging. The NG-RAN includes Paging Assistance Data for CE capable UE, if available, in the N2 UE Context Release Complete message. The AMF stores the received Paging Assistance Data for CE capable UE in the UE context for subsequent Paging procedure. Table 5.2.2.2.2-1: UE Context in AMF Field; Description. Paging Assistance Data for CE capable UE; Paging Assistance Data for Enhanced Coverage level and cell ID provided by the last NG-RAN the UE was connected to..

It is obviously that in order to support coverage enhancement aspect, it is needed to include Paging Assistance Data for CE capable UE (Cell ID and Coverage Level) in NGAP PAGING message and UE CONTEXT RELEASE COMPLETE message.

Observation 1: it is needed to introduce Paging Assistance Data for CE capable UE (Cell ID and Coverage Level) in NGAP PAGING message and UE CONTEXT RELEASE COMPLETE message.

In NGAP BL CR#0153 “Common CP/UP aspects of CIoT UEs when connected to 5GC” [1], the Enhanced Coverage Restriction IE and Extended Connected Time IE are included to support Coverage Enhancement feature, but the paging Assistance Data for CE capable UE (Cell ID and Coverage Level) (applies to both CP and UP solutions) related aspects are missing.

In NGAP BL CR#188, “Introduction of Suspend-Resume” [2], although based on the R3-193161 CIoT Work Structure [3], the CR was intended to introduce CIoT UP solution in NGAP, it is noticed that the Cell Identifier and Coverage Enhancement Level IE is included in the UE CONTEXT RELEASE COMPLETE and UE CONTEXT SUSPEND REQUEST messages in the CR, PAGING message part is missing.

Considering of the current status, there are two options to proceed:

Option 1:

- BL CR#153, introduce Paging Assistance Data for CE capable UE (Cell ID and Coverage Level) in NGAP PAGING message and UE CONTEXT RELEASE COMPLETE message, and furthermore introduce the IE in the UE CONTEXT SUSPEND REQUEST message (the procedure and message should be copied from CR#188).

- BL CR#188, remove the Cell Identifier and Coverage Enhancement Level IE in UE CONTEXT RELEASE COMPLETE message and UE CONTEXT SUSPEND REQUEST message.

Option 2:

- BL CR#188, introduce Cell Identifier and Coverage Enhancement Level IE in PAGING message.

Defer to the R3-193161 CIoT Work Structure [3] endorsed in RAN3#104 meeting,  considering that the related information applies to both CP and UP solution rather than UP only, it is proposed to choose option1 to further proceed.

“Division of Tasks” (slide 6 of R3-193161, noted) are endorsed

Proposal 1: BL CR#153, introduce Paging Assistance Data for CE capable UE IE (Cell ID and Coverage Level) in PAGING and UE CONTEXT RELEASE COMPLETE, and UE CONTEXT SUSPEND REQUEST messages.

Proposal 2: BL CR#188, remove the Cell Identifier and Coverage Enhancement Level IE.

3. Conclusion

- In this contribution, we analyse the support of coverage enhancement when NB-IoT and eMTC connecting to 5GC, get the following observation and proposals:

Observation 1: it is needed to introduce Paging Assistance Data for CE capable UE (Cell ID and Coverage Level) in NGAP PAGING message and UE CONTEXT RELEASE COMPLETE message.

Proposal 1: BL CR#153, introduce Paging Assistance Data for CE capable UE IE (Cell ID and Coverage Level) in PAGING and UE CONTEXT RELEASE COMPLETE, and UE CONTEXT SUSPEND REQUEST messages.

Proposal 2: BL CR#188, remove the Cell Identifier and Coverage Enhancement Level IE.

It is proposed to agree the TP to NGAP BL CR#153 provided in section 5, highlighted in blue.

4. Reference

- R3-200033	Common CP/UP aspects of CIoT UEs when connected to 5GC	Ericsson	LTE_eMTC5-Core, NB_IOTenh3-Core, NGAP CR#0153

- R3-200034 Introduction of Suspend-Resume, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, LTE_eMTC5, NB_IOTenh3, NGAP CR#188

- R3-193161CIoT Work Structure, Huawei, Qualcomm Incorporated

5. Text Proposal to the NGAP BL CR#0153

| Reason for change: |  | Introduce existing LTE IEs to support Enhanced Coverage restriction when connecting eMTC/NB-IoT UEs to 5GC; Introduce Paging Assistance Data for CE Capable UE (Cell ID and Coverage Level) in Paging and UE Context Release Complete when connecting eMTC/NB-IoT UEs to 5GC. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Summary of change: |  | Introduction of: - the Enhanced Coverage Restriction IE in the following NGAP messages: PAGING, INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE, DOWNLINK NAS TRANSPORT. - the Extended Connected Time IE in the following NGAP messages: INITIAL CONTEXT SETUP REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE, UE CONTEXT RESUME RESPONSE and DOWNLINK NAS TRANSPORT. - Paging Assistance Data for CE Capable UE IE in NGAP PAGING message and UE CONTEXT RELEASE COMPLETE message. |  |  |  |
| Consequences if not approved: |  | Cannot support CIoT UEs when connecting to 5GC. |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.3.2, 8.4.2.2, 8.4.4.2, 8.5.1.2, 8.6.2.2, 9.2.2.1, 9.2.2.6, 9.2.3.4, 9.2.3.9, 9.2.4.1, 9.2.5.2, 9.3.1.xxx (new), 9.3.1.yyy (new), 9.3.3.X (new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |
|  |  | Y | N |  |  |
| Other specs |  |  | X | Other core specifications | TS/TR ... CR ... TS/TR ... CR ... |
| affected: |  |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |  |

------------------------------------------

Start of change

------------------------------------------

8.3.3	UE Context Release (AMF initiated)

8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the AMF to order the release of the UE-associated logical NG-connection due to various reasons, e.g., completion of a transaction between the UE and the 5GC, or release of the old UE-associated logical NG-connection when the UE has initiated the establishment of a new UE-associated logical NG-connection, etc. The procedure uses UE-associated signalling.

8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE context release: successful operation

The AMF initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the NG-RAN node.

The UE CONTEXT RELEASE COMMAND message shall contain both the AMF UE NGAP ID IE and the RAN UE NGAP ID IE if available, otherwise the message shall contain the AMF UE NGAP ID IE.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the NG-RAN node shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

If the PDU Session Resource List IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the User Location Information IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the Information on Recommended Cells and RAN Nodes for Paging IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall, if supported, store it and may use it for subsequent paging.

For each PDU session for which the Secondary RAT Usage Information IE is included in the PDU Session Resource Release Response Transfer IE, the SMF shall handle this information as specified in TS 23.502 [10].

If the Paging Assistance Data for CE Capable UE IE is included in the UE CONTEXT RELEASE COMPLETE message, the AMF shall, if supported, store it and use it for subsequent paging, as specified in TS 23.502 [10].

------------------------------------------

Skip to next change

------------------------------------------

8.5.1	Paging

8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

The AMF initiates the Paging procedure by sending the PAGING message to the NG-RAN node.

At the reception of the PAGING message, the NG-RAN node shall perform paging of the UE in cells which belong to tracking areas as indicated in the TAI List for Paging IE.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12] and TS 36.304 [29].

For each cell that belongs to any of the tracking areas indicated in the TAI List for Paging IE, the NG-RAN node shall generate one page on the radio interface.

If the Paging Priority IE is included in the PAGING message, the NG-RAN node may use it according to TS 23.501 [9].

If the UE Radio Capability for Paging IE is included in the PAGING message, the NG-RAN node may use it to apply specific paging schemes.

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present, according to TS 38.300 [8].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 38.300 [8].

If the Paging Origin IE is included in the PAGING message, the NG-RAN node shall transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the Enhanced Coverage Restriction IE is included in the PAGING message, the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the Paging Assistance Data for CE Capable UE IE is included in the PAGING message, it may be used for paging the indicated CE capable UE, according to TS 23.502 [10].

------------------------------------------

Skip to next change

------------------------------------------

9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Release Response Transfer | O |  | OCTET STRING | Containing the PDU Session Resource Release Response Transfer IE specified in subclause 9.3.4.21. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.yyy |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

------------------------------------------

Skip to next change

------------------------------------------

9.2.4.1	PAGING

This message is sent by the AMF and is used to page a UE in one or several tracking areas.

Direction: AMF  gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.xxx |  | YES | ignore |
| Paging Assistance Data for CE capable UE | O |  | 9.3.1.yyy |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

------------------------------------------

Skip to next change

------------------------------------------

9.3.1.yyy	Paging Assistance Data for CE Capable UE

This IE provides Assistance Data for paging CE capable UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.3.1.9 |  |
| Coverage Enhancement Level | M |  | OCTET STRING | Includes either the UEPagingCoverageInformation message as defined in 10.2.2 of TS 36.331 [21], or the UEPagingCoverageInformation-NB message as defined in 10.6.2 of TS 36.331 [21]. |

------------------------------------------

Skip to next change

------------------------------------------

START OF ASN.1 CHANGES

9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

SKIPPED ASN.1 UNCHANGED

NR-CGI,

NRPPa-PDU,

NumberOfBroadcastsRequested,

OverloadResponse,

OverloadStartNSSAIList,

PagingAssisDataforCEcapabUE,

PagingDRX,

PagingOrigin,

PagingPriority,

PDUSessionAggregateMaximumBitRate,

SKIPPED ASN.1 UNCHANGED

id-NRPPa-PDU,

id-NumberOfBroadcastsRequested,

id-OldAMF,

id-OverloadStartNSSAIList,

id-PagingAssisDataforCEcapabUE,

id-PagingDRX,

id-PagingOrigin,

id-PagingPriority,

id-PDUSessionResourceAdmittedList,

id-PDUSessionResourceFailedToModifyListModCfm,

NEXT CHANGE

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseComplete-IEs} },

...

}

UEContextReleaseComplete-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging	PRESENCE optional		}|

{ ID id-PDUSessionResourceListCxtRelCpl				CRITICALITY	reject	TYPE PDUSessionResourceListCxtRelCpl					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		}|

{ ID id-PagingAssisDataforCEcapabUE					CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE						PRESENCE optional},

...

}

NEXT CHANGE

-- **************************************************************

--

##### -- PAGING

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PagingIEs} },

...

}

PagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UEPagingIdentity				CRITICALITY ignore	TYPE UEPagingIdentity		PRESENCE mandatory}|

{ ID id-PagingDRX						CRITICALITY ignore	TYPE PagingDRX				PRESENCE optional}|

{ ID id-TAIListForPaging				CRITICALITY ignore	TYPE TAIListForPaging		PRESENCE mandatory}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority			PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging	PRESENCE optional}|

{ ID id-PagingOrigin					CRITICALITY ignore	TYPE PagingOrigin				PRESENCE optional}|

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging	PRESENCE optional}|

{ ID id-Enhanced-CoverageRestriction	CRITICALITY ignore	TYPE Enhanced-CoverageRestriction	PRESENCE optional}|

{ ID id-PagingAssisDataforCEcapabUE		CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE	PRESENCE optional},

...

}

NEXT CHANGE

9.4.5	Information Element Definitions

SKIPPED ASN.1 UNCHANGED

COUNTValueForPDCP-SN12 ::= SEQUENCE {

pDCP-SN12			INTEGER (0..4095),

hFN-PDCP-SN12		INTEGER (0..1048575),

iE-Extensions		ProtocolExtensionContainer { {COUNTValueForPDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNTValueForPDCP-SN12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueForPDCP-SN18 ::= SEQUENCE {

pDCP-SN18			INTEGER (0..262143),

hFN-PDCP-SN18		INTEGER (0..16383),

iE-Extensions		ProtocolExtensionContainer { {COUNTValueForPDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNTValueForPDCP-SN18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CoverageEnhancementLevel ::= OCTET STRING

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress		TransportLayerAddress,

choice-Extensions		ProtocolIE-SingleContainer { {CPTransportLayerInformation-ExtIEs} }

}

CPTransportLayerInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-EndpointIPAddressAndPort		CRITICALITY reject	TYPE EndpointIPAddressAndPort		PRESENCE mandatory},

...

}

SKIPPED ASN.1 UNCHANGED

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pERScalar		INTEGER (0..9, ...),

pERExponent		INTEGER (0..9, ...),

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PacketLossRate ::= INTEGER (0..1000, ...)

PagingAssisDataforCEcapabUE ::= SEQUENCE {

eUTRA-CGI							EUTRA-CGI,

coverageEnhancementLevel			CoverageEnhancementLevel,

...

}

PagingAssisDataforCEcapabUE-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptInformation ::= SEQUENCE {

pagingAttemptCount					PagingAttemptCount,

intendedNumberOfPagingAttempts		IntendedNumberOfPagingAttempts,

nextPagingAreaScope					NextPagingAreaScope									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PagingAttemptInformation-ExtIEs} }	OPTIONAL,

...

}

PagingAttemptInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptCount ::= INTEGER (1..16, ...)

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

SKIPPED ASN.1 UNCHANGED

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType											ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation					ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-Enhanced-CoverageRestriction							ProtocolIE-ID ::= xxx

id-Extended-ConnectedTime								ProtocolIE-ID ::= xxx

id-PagingAssisDataforCEcapabUE							ProtocolIE-ID ::= xxx

END OF CHANGES

------------------------------------------

End of change!

------------------------------------------