3GPP TSG-RAN3 Meeting #107bis-e	R3-202649

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.413): NG Configuration aspects for NPN

Source: 	Huawei

Agenda item:	16.2

Document for:	Discussion

Annex – TP for TS 38.413 (on the top of BL R3-201592)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

9.3.3.Y1	NID

This IE is used to identify (together with a PLMN identifier) a Stand-Alone Non-Public Network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | BIT STRING (SIZE(44)) | Defined in TS 23.003 [23]. |

>>>> NEXT CHANGE <<<<

9.3.3.Y6	Cell CAG List

This IE indicates the list of CAG IDs supported by a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell CAG List |  | 1..<maxnoofCAGsperCell> |  |  |
| >CAG ID | M |  | 9.3.3.Y2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGsperCell | Maximum no. of CAGs per cell. Value is 64. Max is 12 in this release. |

>>>> NEXT CHANGE <<<<

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

// skip unchanged text same section //

NGRANTraceID ::= OCTET STRING (SIZE(8))

NID ::= BIT STRING (SIZE(44))

9.4.7	Constant Definitions

// skip unchanged text same section //

maxnoofCAGSperCell					INTEGER ::= 64

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>