3GPP TSG-RAN WG3 #108-e							                          R3-203900

Online, 1-11 June 2020

Agenda item:	10.2.2.1

Source:	CMCC

Title:	Remaining issues of MLB

Document for:	Discussion and decision

Introduction

At last RAN3 meetings, the mobility load balancing has been discussed and several essential agreements have been achieved.

In this contribution, we discuss remaining issues on MLB.

Discussion

- 2.1 Number of active UEs

Last meeting has agreed to introduce the Number of Active UEs, but the details are still FFS,

Reporting of Number of Active UEs:

Main options are:

- Xn only (two companies)

- F1+Xn, based on TS 38.314 section 4.1.1.3 (per DRB level measurements)

- F1 + Xn, based on TS 38.314 section 4.1.1.3.5 Mean number of Active UEs per cell

Additional question on whether to also report on X2. To be continued...

Most of the companies have expressed the usefulness of this metric, which provides additional information that PRB usage and CAC cannot. As proposed in last email discussion, there’re basically two ways of defining this metric: 1. by referring legacy E1 signaling (inactivity notification); or 2. by referring to definitions specified in TS38.314. In our opinion, the second way, i.e. reusing definitions in TS38.314 is more straight-forward than implication from inactivity notification from CU-CP to CU-UP, and the first way is hard to achieve a unified definition within the scope of standardization when there’s no E1 interface.

Some companies argued that the definitions in TS38.314 don’t reflect the status of PDCP buffer since metrics for the Number of Active UEs are only measured at RLC and MAC. However, we do not see any obstacle of introducing a metric as such without accounting for PDCP buffer, since the Number of Active UEs is measured over a period of time, and there’s little chance for a situation where there’s buffered data at PDCP while no data at RLC&MAC for a specific UE over a period of time. After confirmation with our RAN2 colleague, RAN2 prefers to consider split bearer case in R17. In order to finalize this topic within R16, we can start with non-split bearer case; and may follow the RAN2 progress for split bearer case in R17.

On the other hand, some company pointed out that the assumption that RAN2-defined metric can reflect UP load status linearly in case of many UEs is controversial, we want to clarify that the assumption is based on the statistical manner in average. In addition, we also agree that merely a mean value is not enough to provide abundant information to support broad variety of use cases in NR, and reusing Number of active UEs per DRB specified in TS38.314 is potentially the most straight-forward way to differentiate various services supported in NR, which will be discussed in detail later. As a result, at least Xn and F1 should support reporting the Number of Active UEs with same definition on both interfaces.

Another open issue is which parameters in TS38.314 should be adopted as the metric for load reporting. According to email discussion of last meeting, most of companies agree to report the Number of Active UEs based on TS38.314, Section 4.1.1.3.5 Mean number of Active UEs per cell. As stated above, we also think number of Active UEs per DRB per cell specified in TS38.314 is beneficial to help indicate different services. More specifically, the number of Active UEs per DRB per cell actually counts the number of DRBs with buffered data per QoS level for all UEs under each cell, and various services can be classified with different QoS levels. With clear load reporting per QoS level, neighbor nodes will get more information for potentially better subsequent operations by MLB.

Proposal 1: Report number of Active UEs at least on F1 and Xn.

Proposal 2: Report Mean number of Active UEs per cell.

Proposal 3: Report number of Active UEs per DRB per cell specified in TS38.314 for load reporting.

- 2.2 Per slice & per SUL reporting

During last meeting, some company proposed to introduce slice related metrics for load reporting. We see benefits to support per slice reporting, and SA5 also defines slice-related parameters for RRM policy in TS28.541 which defines the policy to set the ratios for the split of radio resources between supported S-NSSAIs. However, because of the limited time we have in R16 and consensus may be hard to achieve, we would like to discuss it further in R17.

As per SUL reporting, we also think it is beneficial and identify a scenario where the source will NOT trigger a handover request to a potential target with high SUL load, in such case UL load and SUL load cannot be represented by a single UL metric. We also would like to discuss it further in R17.

Proposal 4: Whether to support per slice and per SUL reporting is further discussed in R17.

- 2.3 RRC Connections on X2

We notice that the RRC Connections reported on X2 is still FFS. Since in EN-DC the Master node will always know the exact number of UEs in RRC-Connected state, as well as the exact number of SRB3 established, there is no need to report RRC Connections on X2

Proposal 5: No need to report RRC Connections on X2, and remove FFS.

- 2.4 Cause Values

We check the Cause value in the latest version [4] as well as the BLCR [5] of 38423, and find that the Cause value for Parameter adapting is still missing in both [4] and [5]. In our opinion, the cause value due to parameter adapting should be clearly specified, and should be distinguishable from handover optimization. Thus, we propose to add related Cause value for Parameter adapting in 38423, reuse LTE as baseline. A TP to 38423 is provided in our companion contribution.

Proposal 6: Add Cause value for Parameter adapting in TS38.423, reuse LTE as baseline.

Proposal

The paper discussed the load information for MLB use case, and came to the following proposals:

Proposal 1: Report number of Active UEs at least on F1 and Xn.

Proposal 2: Report Mean number of Active UEs per cell.

Proposal 3: Report number of Active UEs per DRB per cell specified in TS38.314 for load reporting.

Proposal 4: Whether to support per slice and per SUL reporting is further discussed in R17.

Proposal 5: No need to report RRC Connections on X2, and remove FFS.

Proposal 6: Add Cause value for Parameter adapting in TS38.423, reuse LTE as baseline.

References

- R3-201475 addition of SON features, CMCC

- Chairman notes of SONMDT, RAN2 109-e

- TS38.314 v010

- TS38.423 v16.1.0

- R3-202918 addition of SON features, Samsung