3GPP TSG-RAN WG#109-e	R3-204716

# 17 - 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0415 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of values for maximum Integrity Protected Data Rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-07-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In C1-202079, CT1 defined a value of “null” for the maximum data rate for integrity protection. This value is to be used by the UE when it does not support N3 data transfer. Although it is possible that such value will not be passed to the RAN, or used in NGAP procedures, it seems safe practice to add this codepoint since in future further codepoints may be added or the usage of this codepoint could be extended, and the RAN IE should preferably be formally aligned with the NAS IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new codepoint of “null” is added to the Maximum Integrity Protected Data Rate IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No alignment with NAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.103, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.103	Maximum Integrity Protected Data Rate

This IE indicates the maximum aggregate data rate for integrity protected DRBs for a UE as defined in TS 38.300 [8].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Integrity Protected Data Rate | M |  | ENUMERATED (64kbps, max UE rate, …,null) | Defines the upper bound of the aggregate data rate of user plane integrity protected data for either UL or DL. Usage of the null value is specified in TS 24.501 [26]. |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

**  skip unchanged asn  **

#### -- M

MaskedIMEISV ::= BIT STRING (SIZE(64))

MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)

MessageIdentifier ::= BIT STRING (SIZE(16))

MaximumIntegrityProtectedDataRate ::= ENUMERATED {

bitrate64kbs,

maximum-UE-rate,

...,

null

}

MICOModeIndication ::= ENUMERATED {

true,

...

}