3GPP TSG-RAN WG3 Meeting #109e	R3-204923

# Online, 17-28 August 2020

Agenda Item:	22.3.3

Source:	Lenovo, Motorola Mobility

Title: 	5G MBS Transmission Mode and Area Control

Document for:	Discussion and Approval

1	Discussion

A Rel-17 Work Item on NR support of Multicast and Broadcast Services was agreed in RP-201038 , and one of objectives is:

-	Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

This contribution discusses how to support dynamic control of 5G MBS transmission area.

2	Discussion

For PTM, there are two modes SC-PTM and MC-PTM. In SC-PTM mode, the gNB schedules the multicast traffic in a single cell via a cell specific G-RNTI. In the MC-PTM mode, the gNB schedules the multicast traffic among multiple cells using a same G-RNTI and radio resources among these cells. Because the gNB-CU have full information (e.g. the UE distributions), for a single UE, the gNB-CU makes the decision on which modes is configured to the UE i.e. PTP mode only, SC-PTM mode only, MC-PTM mode only, or both PTP and SC-PTM/MC-PTM modes.

For SC-PTM and MC-PTM, the multicast area should also be selected. It could be better the gNB-CU makes the decision on the multicast area.

It is up to the gNB-CU makes the decision on which modes is configured to the UE i.e. PTP mode only, SC-PTM mode only, MC-PTM mode only, or both PTP and SC-PTM/MC-PTM modes.

It is up to the gNB-CU makes the decision on the 5G MBS data transmission area of a 5G MBS session.

A basic procedure of data transmission mode and area decision during 5G MBS Bearer Setup procedure is provided in Figure 1.

1.	When gNB-CU decides to establish a 5G MBS bearer, the gNB-CU sends an 5G MBS BEARER SETUP REQUEST message to the gNB-DU. The 5G MBS BEATER SETUP REQUEST message may include data transmission mode and data transmission area. The data transmission mode is SC-PTM or MC-PTM. The data transmission area is a cell or a cell list on which the 5G MBS data to be transmitted.

2a.	If SC-PTM is used for a cell, the gNB-DU allocates a G-RNTI per cell for the 5G MBS session. The gNB-DUs also provides the gNB-DU to gNB-CU RRC container (which includes layer 2 configuration) per cell to the gNB-CU.

3a.	For a UE in RRC_CONNECTED state, the gNB-CU provides the gNB-CU to gNB-DU RRC container which includes the 5G MBS Configuration RRC message in RRC DOWNLINK TRANSFER message to the gNB-DU.

4a. gNB-DU sends the 5G MBS Configuration via a RRC dedication signaling to UE in RRC_CONNECTED.

3b.	 For UEs in RRC_IDLE and RRC_INACTIVE, the gNB-CU provide gNB-CU to gNB-DU RRC containers (5G MBS Configuration per cell (SC-PTM) or 5G MBS Configuration of a list of cells) to the gNB-DU via NON UE RRC DOWNLINK TRANSFER message.

4b. gNB-DU sends the 5G MBS Configuration via MCCH to UEs in RRC_IDLE and RRC_INACTIVE.

Figure 1. 5G MBS Bearer Setup procedure with data transmission area management

Similarly, the gNB-CU can modify the data transmission mode and/or area by 5G MBS Bearer Modification procedure.

5G MBS data transmission mode and 5G MBS transmission area management is achieved by 5G MBS Bearer Setup or 5G MBS Bearer Modification procedure.

3	Conclusion

This contribution discusses how to support dynamic control of 5G MBS transmission area. And we propose:

- It is up to the gNB-CU makes the decision on which modes is configured to the UE i.e. PTP mode only, SC-PTM mode only, MC-PTM mode only, or both PTP and SC-PTM/MC-PTM modes.

- It is up to the gNB-CU makes the decision on the 5G MBS data transmission area of a 5G MBS session.

- 5G MBS data transmission mode and 5G MBS transmission area management is achieved by 5G MBS Bearer Setup or 5G MBS Bearer Modification procedure.

A text proposal is provided in Annex.

Annex (text proposal to TS 38.401)

x.x.x	5G MBS Bearer context setup

Figure x.x.x-1 shows 5G MBS Bearer Setup procedure with 5G MBS data transmission mode and area management.

Figure x.x.x-1. 5G MBS Bearer Setup procedure with data transmission area management

1.	When gNB-CU decides to establish a 5G MBS bearer, the gNB-CU sends an 5G MBS BEARER SETUP REQUEST message to the gNB-DU. The 5G MBS BEATER SETUP REQUEST message may include data transmission mode and data transmission area. The data transmission mode is SC-PTM or MC-PTM. The data transmission area is a cell or a cell list on which the 5G MBS data to be transmitted.

2a.	If SC-PTM is used for a cell, the gNB-DU allocates a G-RNTI per cell for the 5G MBS session. The gNB-DUs also provides the gNB-DU to gNB-CU RRC container (which includes layer 2 configuration) per cell to the gNB-CU.

3a.	For a UE in RRC_CONNECTED state, the gNB-CU provides the gNB-CU to gNB-DU RRC container which includes the 5G MBS Configuration RRC message in RRC DOWNLINK TRANSFER message to the gNB-DU.

4a. gNB-DU sends the 5G MBS Configuration via a RRC dedication signaling to UE in RRC_CONNECTED.

3b.	 For UEs in RRC_IDLE and RRC_INACTIVE, the gNB-CU provide gNB-CU to gNB-DU RRC containers (5G MBS Configuration per cell (SC-PTM) or 5G MBS Configuration of a list of cells) to the gNB-DU via NON UE RRC DOWNLINK TRANSFER message.

4b. gNB-DU sends the 5G MBS Configuration via MCCH to UEs in RRC_IDLE and RRC_INACTIVE.