3GPP TSG-RAN3 Meeting #109-e	R3-204992

E-meeting, 17 - 28 August 2020

Title: 	Introducing UE Radio Capability Mapping procedure for EN-DC

Source: 	Huawei

Agenda item:	31.3.6

Document for:	Discussion

1. Introduction

Last RAN3#108-e meeting discussed the UE radio capability mapping procedure for EN-DC with the summary in [1].

- X2 mapping procedure for RACS: To be continued as Rel-16 correction…

This contribution proposes to introduce the mapping procedure over X2.

2. Discussion

- In TS 37.340, the following is already captured.

- For MR-DC with 5GC, the SN may retrieve the UE radio capability information associated to a UE radio capability ID from the 5GC.

- Hence for EN-DC, to introduce the mapping procedure over X2 can allow that this is logically aligned with the mapping procedure over NG.

- Also if the mapping is not introduced, the MN has to perform as follows, but both have their drawnbacks.

- The MN needs to transfer the full size of UE radio capability information at each the SN addition procedure together with the UE radio capability ID since the sgNB may not retrieve the UE capability information either from the MN, or from the EPC. This may incur large signalling overhead, which is the objective of the WI.

- The MN sends the UE capability ID to the SN, but SN rejects the addition request. Then MN initiates the SN additional request carrying both the UE capability ID and capability information. This seems not a good approach.

- In addition, another concern is about the nested structure. For example, the X2 SN addition procedure is nested with the mapping procedure, which may have impact on the setting of TXnDCprep. Given that that nested procedure is already supported for SN initiated modification procedure, as described in TS 36.243, it seems this is not a big issue either.

- Hence the following proposal is made:

Introduce the UE radio capability mapping procedure for EN-DC.

3. Conclusion

This contribution continuously discuss remaining issue on RACS with the following proposals.

- Introduce the UE radio capability mapping procedure for EN-DC.

The CRs for TS 36.423, TS 37.340 and TS 36.300 can be found in [2-4].

4. Reference

- R3-204009  Summary of Offline Discussion on RACS stage3 issues, Huawei (Moderator)

- R3-204993	Introducing UE Radio Capability Mapping procedure for EN-DC,	Huawei

- R3-204994	Introducing UE Radio Capability Mapping procedure for EN-DC,	Huawei

- R3-205363	Introducing UE Radio Capability Mapping procedure for EN-DC	Huawei