3GPP TSG-RAN3 #109-e	R3-205008

17th August – 28th August 2020

Online

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0648 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Successful Handover Report CR 38.473 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  |  | Date: |  | 2020-08-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the New WID on enhancement of data collection for SON/MDT in NR that was approved in RAN #86, in the objectives of the WID, support of Successful Handover Reports is included. We propose that RAN3 will support the signalling of successful HO report. Since all the serving cell beam management related configuration takes place at gNB-DU, a successful HO report including RLM and Beam Failure Detection (BFD) related issue is required to be forwarded from gNB-CU of the target node to the source gNB and thereof to the source gNB-DU. Therefore, gNB-DU can optimize the beam configurations and provide such new configurations as part of the RRC CellGroupConfig IE, which is signalled to the UE by the gNB-CU. Therefore, we propose to capture a solution based on F1 signaling of the successful handover information from CU to the DU of the RAN node owning the source cell in the handover process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Successful HO report container IE in the ACCESS AND MOBILITY INDICATION message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Successful Handover Report will not be supported in F1 AP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.1 and 9.2.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

============ Start of 1st change ==============

## 8.11	Self Optimisation Support procedures

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful Handover report container IE the gNB-DU shall take it into account for optimisation of mobility parameters

#### 8.11.1.3	Abnormal Conditions

Not applicable.

============ Start of next change ==============

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO report container FFS | O |  | OCTET STRING | Successful-HO-Report-r16 contained in the UEInformationResponse message (TS 38.331 [18]) FFS | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |