3GPP TSG-RAN WG3 Meeting #109-e	R3-205147

# Electronic, August 17 – 28, 2020

Agenda item:

Source: 	Qualcomm Incorporated

Title: 	TP to BL CR TS 38.473: Addition of missing parameters

Document for:		Discussion and Decision

# 1. 	Discussion

This document is a F1AP mirror of [5]. The below discussion is also in [5], and the changes are equivalent. It should be noted however that the need for the F1AP Positioning Information Exchange procedure needs to be confirmed (i.e. parts of this TP may not be required if this procedure is not required).

Also for this reason, the TP in Annex B of [5] is not mirrored as the corresponding IE has a different structure. Further alignment is therefore required.

## 1.1	NR UL Measurement Report Configuration

At RAN1#101-e, the following agreement was made [1]:

Agreement:

Introduce the following new parameters to the higher layer parameter list:

| NR_pos-Core | NR UL Measurement Report Configuration | FFS in RAN3 WG | Expected Propagation Delay | Expected Propagation Delay | New | For providing an indication of when the SRS is expected to arrive in time at the gNB relative to the UL-RTOA reference time. | +/- 500us Granularity 4 Ts | NRPPa 38.455 | Similar to expected propagation delay in SLmAP. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos-Core | NR UL Measurement Report Configuration | FFS in RAN3 WG | Delay Uncertainty | Delay Uncertainty | New | For providing an indication of when the SRS is expected to arrive in time at the gNB with uncertainty (search window). | Maximum 32us Granularity 4 Ts | NRPPa 38.455 | Similar to delay uncertainty in SLmAP. |

In addition, TS 38.305 [2] specifies the following for the TRP measurement request information that may be signalled from the LMF to the gNBs (Tables 8.10.2.4-2 (Multi-RTT), 8.13.2.3-2 (UL-TDOA), and 8.14.2.3-2 (UL-AoA) in [2]):

Information

PCI, GCI, and TRP ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty of candidate TRPs (search window), for reception of SRS by candidate TRPs

Start time, duration and report characteristics for the measurements

Currently, the (UL-RTOA) reference time relative to which the search window parameter above are defined is supposed to be present in IE SRS Configuration (9.2.y). Adding the Expected Propagation Delay and Delay Uncertainty to the IE SRS Configuration (9.2.y) seems not appropriate since the SRS Configuration can also be included in the POSITIONING INFORMATION RESPONSE/UPDATE messages as well (i.e., gNBLMF direction).

Therefore, a new IE Measurement Time Information is proposed for the MEASUREMENT REQUEST message which contains the Reference Time together with the Expected Propagation Delay and Delay Uncertainty summarized above. With these parameters, a TRP should be able to know when the UE UL-SRS is expected to arrive in time at the TRP. Note, this is equivalent to SLmAP [3], but generally applicable to all UL measurements.

The SFN Initialization Time in IE SRS Configuration (9.2.y) is then proposed to be moved one level higher to the POSITIONING INFORMATION RESPONSE/UPDATE messages.

Proposal 1: Agree to the TP in Annex A for the F1AP BL CR.

# References

[1]	RAN1 Chairman’s Notes, RAN1 Meeting #101-e, May 25th – June 5th, 2020.

[2]		3GPP TS 38.305, "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[3]		3GPP TS 36.459, "SLm interface Application Protocol (SLmAP)".

[4]		3GPP TS 38.331, "NR; Radio Resource Control (RRC) protocol specification".

[5]		R3-205146, “TP to BL CR TS 38.473: Addition of missing parameters”, Qualcomm Incorporated, RAN3#109-3, August 2020.

# Annex A: Text Proposal

#### 9.2.x.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

Editor’s note: message contents are FFS; TRP-specific IEs can be added to the message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536,…) |  | YES | reject |
| Measurement Quantities |  | 0..1 |  |  | YES | reject |
| >Measurement Quantities Item |  | 1..<maxnoofMeas> |  |  | EACH |  |
| >>Measurement Type | M |  | ENUMERATED (UL RTOA, UL AoA, gNB RX-TX,…) |  |  | - |
| >>Measurement Time Information | O |  | 9.3.z |  |  |  |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | - |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) [FFS] |  | YES | - |
| SRS Configuration | O |  | 9.3.1.o |  | YES | ignore |
| TRP List |  | 1 |  |  |  |  |
| >TRP List Item |  | 1..<maxnoofTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.p |  |  |  |

### >>> NEXT CHANGE <<<

#### 9.2.x.13	POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.d |  | YES | ignore |

Editor’s note; may need to be complete pending to SRS transmission conclusion.

#### 9.2.x.14		POSITIONING INFORMATION RESPONSE

This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS Configuration | O |  | 9.3.1.o |  | YES | ignore |
| Reference Time | O |  | SFN Initialization Time 9.2.y5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### >>> NEXT CHANGE <<<

### 9.3.1.o		SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set List |  | 1 |  | (FFS … |
| >SRS Resource set item |  | 1..<maxnoSRS-ResourceSets> |  |  |
| >>Point A | M |  | INTEGER (0..3279165) NR ARFCN |  |
| >>Subcarrier Spacing | M |  | ENUMERATED(15kHz, 30kHz, 60kHz, 120kHz) |  |
| >>CP Type | M |  | ENUMERATED(Normal, Extended) |  |
| >>Offset To Carrier | M |  | INTEGER(0..2199) | First usable RB to Point A in the number of PRBs |
| >>BWP Start | M |  | INTEGER(0..274) | Start PRB of the UL BWP to the first usable RB |
| >>SRS Resource Set ID | M |  | INTEGER(0.. 255) |  |
| >>SRS Resource | M | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >>>CHOICE SRS Resource type |  |  |  |  |
| >>>> SRS Resource | M |  | 9.2.y1 |  |
| >>>> Positioning SRS Resource | M |  | 9.2.y2 |  |
| SRS Resource Set List |  | 1 |  | … FFS) |

### >>> NEXT CHANGE <<<

### 9.3.z	Measurement Time Information

This information element contains measurement time information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Reference Time | M |  | SFN Initialization Time 9.2.y5 |  |
| Search Window Parameters | O |  |  |  |
| >Expected Propagation Delay | M |  | INTEGER (-3841..3841,…) | Indicates when the SRS is expected to arrive in time at the TRP relative to the Reference Time. Granularity 4Ts, where Ts=1/(15103 2048) seconds. Centre of the search window. |
| >Delay Uncertainty | M |  | INTEGER (1..246,…) | Indicates the uncertainty of the expected SRS arrival time at the TRP Granularity 4Ts, where Ts=1/(151032048) seconds. Single-sided search window. |