3GPP TSG-RAN WG3 Meeting#109-e	R3-205363

E-Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR |  | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing UE Radio Capability Mapping procedure for EN-DC |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-RAN |  |  |  |  |  |  |  | Date: |  | 2020-08-05 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed that the RACS feature is supported in EN-DC. For EN-DC, since some SN nodes (e.g. the SN-only node) have no control plane with the EPC, those SN nodes are not able require the UE capabilities from the EPC. Therefore, there is a need for the en-gNB to request the eNB to send the UE radio capability information associated with a UE radio capability ID. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in EN-DC the UE Radio Capability ID mapping procedure. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because only a new UE Radio Capability Mapping procedure is introduced. The impact can be considered isolated. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed principle for RACS for EN-DC would not be supported. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

20.2.1	X2-CP Functions

The X2AP protocol supports the following functions:

-	Intra LTE-Access-System Mobility Support for UE in ECM-CONNECTED:

-	Context transfer from source eNB to target eNB;

-	Control of user plane tunnels between source eNB and target eNB;

-	Handover cancellation.

-	Support of DC for UE in ECM-CONNECTED:

-	Establishment, Modification and Release of a UE context at the SeNB;

-	Control of user plane tunnels between MeNB and SeNB for a specific UE for split bearer and data forwarding;

-	Provision of the TNL information of the S1 user plane tunnels for SCG bearers.

-	Support of EN-DC for UE in ECM-CONNECTED:

-	Establishment, Modification and Release of a UE context at the SgNB;

-	Control of user plane tunnels between MeNB and SgNB for a specific UE for split bearer, SCG split bearer and data forwarding;

-	Provision of the TNL information of the S1 user plane tunnels for SCG bearers and SCG split bearers.

-	Support of inter-eNB UE Context Resume:

-	Retrieval of UE context for a UE which attempts to resume its RRC connection in an eNB different from where the RRC connection was suspended.

-	Load Management;

-	General X2 management and error handling functions:

-	Error indication;

-	Setting up the X2;

-	Resetting the X2;

-	Updating the X2 configuration data;

-	X2 Release;

-	X2AP Message Transfer;

-	Registration;

-	X2 Removal.

-	Mobility failure event notification and information exchange in support of handover settings negotiation;

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation.

-	Support of cell resource coordination between eNB and en-gNB.

-	Support of UE Radio Capability ID Mapping between eNB and en-gNB.

<Unchanged Text Omitted>

20.2.2.32	Partial Reset procedure for EN-DC

The Partial Reset procedure for EN-DC is triggered by the en-gNB or the MeNB to initiate the release of the resources for a list of UEs with the EN-DC configuration.

Figure 20.2.2.32-1: Partial Reset procedure for EN-DC (initiated at the en-gNB)

Figure 20.2.2.32-2: Partial Reset procedure for EN-DC (initiated at the MeNB)

#### 20.2.2.xx	UE Radio Capability ID Mapping procedure

The purpose of the UE Radio Capability ID Mapping Request procedure is to enable the en-gNB to request the MeNB to provide the UE Radio Capability information that maps to a specific UE Radio Capability ID.

Figure 20.2.2.xx-1: UE Radio Capability ID Mapping procedure

END OF CHANGE