3GPP TSG-RAN3 Meeting #109-e	R3-205646

E-meeting, 17 - 28 August 2020

Title: 	Potential solutions to network slice service continuity

Source: 	Huawei

Agenda item:	17.2

Document Type:	Discussion and Decision

Annex 1– TP for TR 38.832

6	Study necessity and mechanisms to support service continuity

6.1	Scenario and issue description

Editor Note: capture the description of scenario and issue.

6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

6.2.x Handover procedures for use case: Slice resource shortage in case of Intra-RA mobility and Inter-RA mobility

6.2.x.1 Xn based handover

Fig. 1: Slice re-mapping/fallback determined by T-gNB

- The S-gNB sends the HANDOVER REQUEST message to the T-gNB, which may include the slice re-mapping/fallback list.

- If UE’s ongoing slice(s) is rejected due to, e.g., high overload conditions, based on the slice re-mapping/fallback list provided by the S-gNB or the AMF in advance, the T-gNB makes the slice re-mapping/fallback decision. And it may send the decision in the HANDOVER REQUEST ACKNOWLEDGE message sent to the S-gNB.

- The T-gNB may send the slice re-mapping/fallback decision to the AMF through the PATH SWITCH REQUEST message.

- The AMF responds the PATH SWITCH REQUEST ACKNOWLEDGE message.

6.2.x.2 NG based handover

Fig. 2: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

Editor’s note:	it is FFS whether the Slice re-mapping/fallback list is included in the HANDOVER REQUIRED message.

- The AMF sends the HANDOVER REQUEST message to the T-gNB, which may include the slice re-mapping/fallback list.

- If UE’s ongoing slice(s) is rejected due to, e.g., high overload conditions, based on the slice re-mapping/fallback list, the T-gNB may include the re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message sent to AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

6.2.y Handover procedures for use case: Non-supported slice in case of Inter-RA mobility

6.2.y.1 Xn based handover

Fig. 3: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUEST message to the T-gNB, which may include the slice re-mapping/fallback list.

- If UE’s ongoing slice(s) is not supported, based on the slice re-mapping/fallback list provided by the S-gNB or the AMF in advance, the T-gNB makes the slice re-mapping/fallback decision. And it may include the decision in the HANDOVER REQUEST ACKNOWLEDGE message sent to the S-gNB.

- The T-gNB may send the slice re-mapping/fallback decision to the AMF through the PATH SWITCH REQUEST message.

- The AMF responds the PATH SWITCH REQUEST ACKNOWLEDGE message.

6.2.y.2 NG based handover

Fig. 4: Slice re-mapping/fallback determined by the AMF

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

Editor’s note:	it is FFS whether the Slice re-mapping/fallback list is included in the HANDOVER REQUIRED message.

- If UE’s ongoing slice(s) is not supported by T-gNB, the AMF may make the slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message sent to the T-gNB.

Editor’s note:	the Slice re-mapping/fallback determined by the T-gNB remains further study.

- The T-gNB responds to the AMF through the HANDOVER REQUEST ACKNOWLEDGE message.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.