3GPP TSG-RAN WG3 Meeting #110-e	R3-207185

Electronic Meeting, Nov 2nd – 12th, 2020

Agenda item:		15.2

Source:	Qualcomm Incorporated, Samsung

Title:	RAN visible QoE information reporting by UE

Document for:		approval

# 1. Technical Proposal

# 6	Potential NR QoE solutions and procedures

Editor note: Description of potential NR QoE solutions and procedures, including but not limited to reuse Trace or MDT Functionality/Framework.

## 6.x RAN visible QoE information reporting by UE

Editor note: This section describes the potential procedure for UE to report RAN visible QoE information.

It is FFS whether RAN awareness of QoE information is useful, and whether UE reporting is needed.

QoE aware by gNB enables close loop QoE optimization by RAN. It is too complicated for gNB to understand the real QoE metrics. RAN visible QoE information is simplified QoE information abstracted from QoE metrics by UE. gNB uses the RAN visible QoE information for close loop QoE optimization. Figure 6.x-1 shows the message flow for RAN visible QoE information reporting.

Figure 6.x-1: RAN visible QoE informationreporting

- gNB sends the RAN visible QoE configuration to UE, may along with the QoE measurement configuration container transmited from CN or OAM.

- UE receives and applies the RAN-visible QoE configuration and/or QoE measurement configuration container. The RAN visible QoE Configuration may be so that the corresponding RAN visible QoE information can be a unique score or a combination of scores reflecting the QoE metrics useful for RAN (such as buffer level). The encoding of RAN visible QoE configuration and RAN visible reporting is FFS. The RAN-visible report is provided from the application layer of the UE to the UE’s RRC layer by means of an AT command. The UE’s RRC layer then includes the RAN-visible report, along with the QoE report container, but as a separate IE, in the MeasReportAppLayer IE, and sends it to the RAN.

- gNB reads the RAN visible QoE information and/or forwards the QoE report container to QoE server accordingly.