3GPP TSG-RAN WG3 Meeting #111-e	R3-211272

# Online, January 25th – February 4th 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0737s | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CP-based Congestion Indication for IAB Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh |  |  |  |  |  |  | Date: |  | 2021-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categsories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13)s Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enabling CP-based congestion detection in IAB Networks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a congestion indicator in GNB-DU STATUS INDICATION message. Definition of IAB Congestion Indication IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CP-based congestion detection in IAB Networks not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7.2, 9.2.1.15, 9.3.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Change 1-------------------------------------------

### 8.2.7	gNB-DU Status Indication

#### 8.2.7.1	General

The purpose of the gNB-DU Status Indication procedure is informing the gNB-CU that the gNB-DU is overloaded so that overload reduction actions can be applied. The procedure uses non-UE associated signalling.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: gNB-DU Status Indication procedure

If the gNB-DU Overload Information IE in the GNB-DU STATUS INDICATION message indicates that the gNB-DU is overloaded, the gNB-CU shall apply overload reduction actions until informed, with a new GNB-DU STATUS INDICATION message, that the overload situation has ceased.

The detailed overload reduction policy is up to gNB-CU implementation.

If the IAB Congestion Indication IE is contained in the GNB-DU STATUS INDICATION message, the gNB-CU shall apply measures for congestion mitigation.

#### 8.2.7.3	Abnormal Conditions

Void.

-------------------------------------------Change 2-------------------------------------------

#### 9.2.1.15	GNB-DU STATUS INDICATION

This message is sent by the gNB-DU to indicate to the gNB-CU its status of overload.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |
| IAB Congestion Indication | O |  | 9.3.1.x |  | YES | reject |

-------------------------------------------Change 3-------------------------------------------

#### 9.3.1.x	IAB Congestion Indication

The IAB Congestion Indication IE indicates a downlink congestion in an IAB-node.

Editor’s NOTE: the content of this IE is FFS

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Congestion Indication Type | M |  |  |  |
| >BAP Routing ID-based |  |  |  | The content of this IE is FFS. |
| >>Congested BAP Routing ID List |  | 0..1 |  |  |
| >>>Congested BAP Routing ID List Item |  | 1 .. <maxnoofCongBAPRoutingIDs> |  |  |
| >>>>BAP Routing ID | M |  | 9.3.1.110 |  |
| >Link of BH-RLC Channel-based |  |  |  |  |
| >>Congested Link or BH RLC Channel List |  | 0..1 |  |  |
| >>>Congested Link or BH RLC Channel List Item |  | 1 .. <maxnoofChildIABNodes> |  |  |
| >>>>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| >>>>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| >>>>BH RLC CH ID | O |  | 9.3.1.113 | The content of this IE is FFS. |

| Range bound | Explanation |
| --- | --- |
| maxnoofChildIABNodes | Maximum number of child nodes for which the congestion can be indicated. Value is FFS. |
| maxnoofCongBAPRoutingIDs | Maximum number of BAP Routing IDs for which the congestion can be indicated. Value is FFS. |

-------------------------------------------End of changes-------------------------------------------