3GPP TSG-RAN WG3 Meeting #111-e	R3-211342

# Online, January 25th – February 4th 2021

Agenda Item:	15.2

Source:	Ericsson, Samsung, Qualcomm, China Mobile

Title:	pCR for TR 38.890: QoE Visibility at the RAN

Document for:	Agreement

# Introduction

This pCR was produced in the CB: # NRQoE5-RAN_visible.

# pCR to TR 38.890

-------------------------------------------Change 1-------------------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

QoE measurement: An application layer measurement configured by OAM, see details in [3] [4] [5] [6] for different service type.

QoE report: The result of a QoE measurement.

Radio-related measurements: Measurements on the radio layer, whose purpose is to help network to further evaluate and improve the QoE.

Radio-related information: Information other than “radio-related measurements”, e.g. feature info, mobility history info or dual connectivity status. FFS on Radio related information only from UE or RAN node or both.

RAN-visible QoE: Includes RAN-visible QoE metrics and RAN-visible QoE values.

RAN-visible QoE metrics: A subset of QoE metrics data collected from UE, which are useful for RAN.

RAN-visible QoE values: A set of values derived from QoE metrics data through a model/function defined in collaboration with SA4.

Editor’s NOTE: The above three definitions may subject to further refinements once further consensus are reached.

-------------------------------------------Change 2-------------------------------------------

## 6.7 	RAN visible QoE information reporting by UE

Editor's NOTE: This section describes the procedure for UE to report RAN visible QoE information.

The RAN may not be able to understand or make use of the legacy QoE metrics, as they are assembled by the OAM, sent inside containers and intended to be processed by the Measurement Collection Entity in the network. If the RAN needs to make use of the QoE concept, there might be requirements that QoE information should be visible by the RAN. RAN-visible QoE  information is simplified QoE information abstracted from QoE metrics by UE, which the RAN may use for various types of d optimizations.

The RAN-visible QoE metrics can be derived from individual SA4-defined QoE metrics deemed useful for the RAN, such as buffer level. RAN-visible QoE metrics may be e.g. be simplified values derived from individual useful SA4-defined QoE metrics or combinations of these values. SA4 to be consulted on how the values are derived.

The following holds:

- The RAN-visible QoE can be used for all services.

- The RAN is responsible for assembling the RAN-visible QoE measurement configuration.

- The RAN is responsible for triggering i.e. activating the RAN-visible QoE measurement.

- FFS whether RAN can explicitly ask the UE to report certain RAN-visible QoE metrics or just an indication to report the fixed set of RAN-visible QoE metrics predefined per service type.

- The RAN should be able to configure RAN-visible QoE autonomously for a given service type only if the application layer QoE for the same service type is already configured.

- The RAN visible QoE value can be generated by UE and QoE server. NOTE: RAN generating RAN visible QoE values requires that RAN reads the QoE report in XML format, which is left to implementation.

- The RAN-visible QoE values are delivered to the RAN as a separate IE, visible to the RAN.

- The RAN is not allowed to change the existing configuration of legacy QoE metrics specified by SA4.

Figure 6.7-1 shows the message flow for RAN visible QoE information reporting.

QoE

Server

RAN node

UE

QoE

Server

RAN node

UE

Figure 6.7-1: RAN-visible QoE information reporting

1.	A  RAN node assembles and sends the RAN-visible QoE configuration to a UE, which may be sent along with the QoE measurement configuration container transmitted from the OAM, directly or via the CN.

2.	The UE receives and applies the RAN-visible QoE configuration and/or QoE measurement configuration container. The RAN visible QoE configuration may be so that the corresponding RAN visible QoE information that is reported can be a unique value or a combination of values reflecting the QoE metrics useful for RAN (such as buffer level). The RAN-visible report is provided from the application layer of the UE to the UE’s RRC layer by means of an AT command. The UE’s RRC layer then includes the RAN-visible report, along with the QoE report container, but as a separate IE, in the MeasReportAppLayer IE, and sends it to the RAN.

3.	The  RAN node reads the RAN-visible QoE information and/or forwards the (legacy) QoE report container to the QoE server accordingly. Alternatively, the OAM server may generate the RAN-visible QoE report and send it to the RAN.

### 6.7.1 	Initial analysis on RAN visible QoE metrics

The table below takes the QoE metrics for streaming service as an example and provides an initial analysis of potential benefits to RAN. In general, whether any metric is beneficial for RAN when it is visible to RAN, should be studied per metric in the normative phase.

Table 1: Initial analysis on QoE Report visibility at RAN for streaming service

| Metric | Description | Initial analysis |
| --- | --- | --- |
| Round-trip time |  | If Round-trip time is large, RAN could try to compensate based on RAN part delay, but the cost might be significant. RTT is also related with many factors like UE capability, radio quality, radio load, etc., to adjust radio transmission delay for one user may impact other users, and the effect for the whole system performance is unpredictable. |
| Jitter duration |  | There are other factors affecting jitter, e.g. buffer size available at UE side, processing delay etc., if RAN already fulfil QoS requirement, and Jitter duration is still bad, further analysis are needed to see whether and what RAN could do to improve. |
| Corruption duration | The time period from the NPT time of the last good frame (since the NPT time for the first corrupted frame cannot always be determined) before the corruption, to the NPT time of the first subsequent good frame | If the RAN can know the results of this metric, the RAN can adjust the resource allocation of the UE to satisfy the user experience |
| Average Throughput |  | RAN could measure RAN side throughput by itself and make adjustment accordingly, so this metric has some relation with RAN though, the benefit seems unclear. |
| Initial playout delay | from the fetch of the first media Segment (or sub-segment) and the time at which media is retrieved from the client buffer |  |
| Device information… | a list of device information objects | this metric may have potential privacy issues because it exposes the user information. However, the videoWidth videoHeight in Devise information may reflect the UE expectations of QoE, it is possible that RAN schedule the UE first with higher QoE expectation. |
| Rendered viewports | a list of viewports that have been rendered during the media presentation | This metric may have potential privacy issues because it exposes the user behaviour |
| Codec Information |  |  |
| Buffer level | list of buffer occupancy level measurements during playout at normal speed. | If the RAN can know the results of buffer level, the RAN can adjust the resource allocation of the UE to ensure there is enough buffer for the streaming, this might help to improve the scheduling efficiency. |
| Representation switch events | to record switch events during playing | This metric is also related with user behaviour during playing |
| Play List | A list of playback periods. A playback period is the time interval between a user action and whichever occurs soonest of the next user action, the end of playback or a failure that stops playback | This metric records the user action of playback during playing. The playlist may be used to derive many other metrics, and an example calculation of a few stalling-related metrics The initial stalling, the frequency of stalling, and the stalling duration may reflect UE experience, it is possible that RAN schedule the UE first with bad UE experience to prevent further QoE degradation. |
| MPD (Media presentation description) Information | This metric can be used to report Representation information from the MPD, so that reporting servers without direct access to the MPD can understand the used media characteristics | Not sure if there are any additional benefits if RAN understands the used media characteristics, anyway RAN should first to behave according to the received QoS parameters which actually reflects media characteristics. |
| Interactivity Summary: | Summarizes the measurements of interactivity usage according to different metrics such as user consumption of rendered interactivity content or engagement with user interface (UI) functionality, such as viewing, clicking on or selection of hyperlinks, radio buttons, check boxes and other forms of UI displays or controls. | This seems also to be related with user behaviour during playing period. |
| Interactivity Event List | A time-ordered list of interactivity events occurring during the playout of the main program, each containing detailed information on the incidences of interactivity usage during that event, as covered by an instance of the interactivity usage report. | Similar as above |

-------------------------------------------End of changes-------------------------------------------