3GPP TSG-RAN WG3 Meeting #112-e	R3-211532

E-meeting, 17-28 May 2021

Title: 	Consideration on EN-DC Resource Status Reporting

Source: 	Huawei, Qualcomm Incorporated, BT, Nokia, Nokia Shanghai Bell, ZTE

Agenda item:	9.3.8.1

Document Type:	Discussion

- Introduction

In this contribution, we discuss an error in the en-gNB initiated EN-DC Resource Status Reporting Initiation procedure, and provide the potential solutions to discuss.

2. Discussion

In X2AP: Resource Status Reporting Initiation and Resource Status Reporting procedures, eNB1 and eNB2 are used to illustrate the initiating node and the peer node, therefore eNB1 Measurement ID IE and eNB2 Measurement ID IE are used in the corresponding messages.

In XnAP: Resource Status Reporting Initiation and Resource Status Reporting procedures, NG-RAN node1 and NG-RAN node2 are used to illustrate the initiating node and the peer node, therefore NG-RAN node1 Measurement ID IE are used and NG-RAN node2 Measurement ID IE in the corresponding messages.

In X2AP: EN-DC Resource Status Reporting Initiation and EN-DC Resource Status Reporting procedures, at the beginning of Rel-16 the specification only allows the eNB to request the reporting of load measurements to the en-gNB, and therefore eNB Measurement ID and en-gNB Measurement ID were used in the corresponding messages [1].

But in RAN3#110 meeting, in order to avoid unnecessary SN triggered SN release due to UE power saving, it was agreed for the en-gNB to request the reporting of load measurements to the eNB, and the EN-DC Resource Status Reporting Initiation and EN-DC Resource Status Reporting procedures were updated to support this scenario [2].

As defined in TS 36.423, the EN-DC RESOURCE STATUS REQUEST message, supports direction en-gNB  eNB, but the measurement IDs are defined as below:

9.1.2.45	EN-DC RESOURCE STATUS REQUEST

This message is sent by the eNB to the en-gNB or by the en-gNB to the eNB to initiate the requested measurement according to the parameters given in the message.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| eNB Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the eNB | YES | reject |
| en-gNB Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095, ...) | Allocated by the en-gNB | YES | ignore |
| … |  |  |  |  |  |  |

Issue 1: wrong measurement ID in EN-DC RESOURCE STATUS REQUEST

The eNB Measurement ID is defined as a mandatory IE with assigned criticality reject, but when the en-gNB initiates the procedure, it cannot know the eNB measurement ID and is not able to provide such information.

In order to make the current procedure work, it is needed to update the eNB Measurement ID IE and en-gNB Measurement ID IE, by using IE name like “E-UTRAN Node1 Measurement ID” and “E-UTRAN Node2 Measurement ID” in the tabular and asn.1.

Note: the changes in the tabular and asn.1 are backward compatible.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| E-UTRAN Node1 Measurement ID | M |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN Node1 | YES | reject |
| E-UTRAN Node 2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095, ...) | Allocated by the E-UTRAN Node2 | YES | ignore |

ENDCResourceStatusRequest-IEs X2AP-PROTOCOL-IES ::= { { ID id-E-UTRAN-Node1-Measurement-ID-ENDC CRITICALITY reject TYPE Measurement-ID-ENDC PRESENCE mandatory }¦ { ID id-E-UTRAN-Node2-Measurement-ID-ENDC CRITICALITY ignore TYPE Measurement-ID-ENDC PRESENCE conditional}¦ -- The IE shall be present if the Registration Request EN-DC IE is set to “Stop” or to “Add” { ID id-Registration-Request CRITICALITY reject TYPE Registration-Request-ENDC PRESENCE mandatory }¦ { ID id-ReportingPeriodicity CRITICALITY ignore TYPE ReportingPeriodicity-ENDC PRESENCE optional }¦ { ID id-ReportCharacteristics CRITICALITY ignore TYPE ReportCharacteristics-ENDC PRESENCE conditional}¦ -- The IE shall be present if the Registration Request EN-DC IE is set to “Start” { ID id-CellToReport-NR-ENDC CRITICALITY ignore TYPE CellToReport-NR-ENDC-List PRESENCE optional } ¦ { ID id-InterfaceInstanceIndication CRITICALITY reject TYPE InterfaceInstanceIndication PRESENCE optional }¦ { ID id-CellToReport-E-UTRA-ENDC CRITICALITY ignore TYPE CellToReport-E-UTRA-ENDC-List PRESENCE optional}, ... }

Considering the alignment of IE names among the messages of EN-DC Resource Status Reporting Initiation procedure, it is also needed to do the same change in the X2AP: EN-DC RESOURCE STATUS RESPONSE and EN-DC RESOURCE STATUS FAILURE messages.

And considering the alignment of IE names between EN-DC Resource Status Reporting Initiation procedure and the EN-DC Resource Status Reporting procedure, it is also better to do the change in EN-DC RESOURCE STATUS UPDATE message as well.

Proposal 1: Update the eNB Measurement ID IE and en-gNB Measurement ID IE to E-UTRAN Node1 Measurement ID IE and E-UTRAN Node2 Measurement ID IE in the following X2AP messages:

  - EN-DC RESOURCE STATUS REQUEST

  - EN-DC RESOURCE STATUS RESPONSE

  - EN-DC RESOURCE STATUS FAILURE

  - EN-DC RESOURCE STATUS UPDATE

Issue 2: Inaccurate cause values

Currently there are two cause values defined to support Resouce Status Reporting Initiation and the Resource Status Reporting procedures:

| ExistingMeasurementID | The action failed because measurement-ID is already used. |
| --- | --- |
| Unknown eNB Measurement ID | The action failed because some eNB Measurement-ID is unknown. |

For EN-DC Resouce Status Reporting Initiation and the EN-DC Resource Status Reporting procedures, the existing “ExistingMeasurementID” can be reused, but the latter one seems not be able to be reused to indicate that some en-gNB measurement-ID is unknown.

This issue is broader and relevant in all EN-DC resource status reporting scenarios: MeNB-initiated reporting (where the en-gNB has no cause to indicate that the en-gNB UE ID is unknown in case the MeNB attempts updating or stopping the reporting) and en-gNB-initiatd reporting (where the MeNB can’t indicate the same). In LTE, the same Cause was used to indicate both measurement IDs: the one of the initiating node and the one of the reporting node.

Proposal 2: RAN3 should consider if the EN-DC resource reporting shall follow the LTE principle that there is a single Cause value to be used when either of the measurement IDs is unknown, or a separate Cause is to be introduced.

However, even if the LTE principle is kept, the existing cause name is incorrect, because it refers to the “eNB”. Therefore, there are two possible options to solve this issue:

- intrduce a new cause value as “Unknown E-UTRAN Node Measurement ID”

- update the “Unknown eNB Measurement ID”  to indicate Unknown Measurement ID

In order to make things clear, we slightly prefer to introduce a new cause value named as “Unknown E-UTRAN Node Measurement ID”.

Proposal 3: introduce a new cause “Unknown E-UTRAN Node Measurement ID” to indicate that the action failed because some E-UTRAN Node Measurement-ID is unknown.

4. Proposals

- In this contribution, we analyse the issues to support both eNB initiated and en-gNB initiated EN-DC Resource Status Reporting Initiation and the EN-DC Resource Status Reporting procedures, and get the following proposals:

Proposal 1: Update the eNB Measurement ID IE and en-gNB Measurement ID IE to E-UTRAN Node1 Measurement ID IE and E-UTRAN Node2 Measurement ID IE in the following X2AP messages:

  - EN-DC RESOURCE STATUS REQUEST

  - EN-DC RESOURCE STATUS RESPONSE

  - EN-DC RESOURCE STATUS FAILURE

  - EN-DC RESOURCE STATUS UPDATE

Proposal 2: RAN3 should consider if the EN-DC resource reporting shall follow the LTE principle that there is a single Cause value to be used when either of the measurement IDs is unknown, or a separate Cause is to be introduced.

Proposal 3: Introduce a new cause “Unknown E-UTRAN Node Measurement ID” to indicate that the action failed because some E-UTRAN Node Measurement-ID is unknown.

- The corresponding Rel-16 X2AP CR is provided in [3].

5. Reference

- R3-204479 Addition of SON feature, CATT, CMCC, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Qualcomm Incorporated, LG Electronics, NTTDOCOMO

- R3-207224 Load information for SN-initiated DC release, Nokia, Nokia Shanghai Bell, BT, Qualcomm Incorporated, ZTE, NEC, Ericsson

- R3-211532 Correction of en-gNB initiated EN-DC Resource Status Reporting, Rel-16 CR to TS 36.423