3GPP TSG-RAN WG3 Meeting #112-e	R3-211537

E-meeting, 17-28 May 2021

Title: 	Consideration on paging DRX in XnAP RAN Paging

Source: 	Huawei

Agenda item:	9.2.2

Document Type:	Discussion

1. Introduction

DRX information delivery for RRC_INACTIVE UE was discussed in RAN3#111, following conclusion can be found in chairman notes:

1) Whether and how to inform the paging eNB in Xn paging about eDRX information?

2) How to interpret the current paging DRX in Xn paging message?

Consensus to continue discussion on the basis of 1315 and 1317

To be continued on this basis...

In this document we further discuss the second topic.

3. Discussion

As we clarified in RAN3#111, how to configure the RAN Paging DRX to UE, is up to the anchor RAN node, the anchor RAN node can make the decision based on the eDRX information and UE specific DRX, the anchor RAN node can set the RAN Paging DRX to the UE with the value same as UE specific DRX, smaller than the UE specific DRX, and also the current standard does not preclude to set a bigger value than the UE specific DRX, e.g. in case the RAN paging resource is limited due to big number of inactive UEs or eDRX is configured. For the Paging DRX IE provided to neighbours in XnAP: RAN PAGING message, it should be the same as RAN Paging DRX configured to the UE, from the receiving neighbour RAN node point of view, it will use the received cycle to page the UE, i.e. RAN Paging, and do not need to care of how the value is set by the anchor NG-RAN node.

Currently there is no clear definition of the Paging DRX IE in XnAP: RAN PAGING message. Therefore we considered in RAN3#111 that if we do not care of the unnecessarily increased RAN paging load, it is also not precluded to provide the information as [3] mentioned, as the specification does not mandate to send the same value to the UE and neighbours, but after further analyses, we found some issues for the proposed change.

As we all know, eDRX is an optional feature for a cell, in SIB 1, there is a flag named as “eDRX-Allowed-5GC” to indicate if eDRX is allowed or not in the cell for the UE connected to 5GC

SystemInformationBlockType1-v1610-IEs ::=	SEQUENCE {

eDRX-Allowed-5GC-r16					ENUMERATED {true}		OPTIONAL,	-- Need OR

transmissionInControlChRegion-r16	ENUMERATED {true}		OPTIONAL,	-- Cond BW-reduced

campingAllowedInCE-r16				ENUMERATED {true}			OPTIONAL,	-- Need OR

plmn-IdentityList-v1610				PLMN-IdentityList-v1610		OPTIONAL,	-- Need OR

nonCriticalExtension					SEQUENCE {}			OPTIONAL

}

eDRX-Allowed-5GC The presence of this field indicates if idle mode extended DRX is allowed in the cell for the UE connected to 5GC. The UE shall stop using extended DRX in idle mode if eDRX-Allowed-5GC is not present when connected to 5GC.

The eDRX information carried in NGAP specification, is an IE with critically ignore, which means when a NG-RAN node does not support eDRX, upon receiving the eDRX information from the CN node, the NG-RAN node will ignore these parameters.

Observation 1: eDRX is an optional feature and may not supported by some NG-RAN nodes.

Observation 2: a cell supporting eDRX may broadcast that eDRX is not allowed.

Observation 3: NG-RAN node is not aware of whether eDRX is supported/allowed or not in other cells of neighbouring NG-RAN nodes.

In [2] and [3], the following sentence is proposed to be added as the semantics description of the Paging DRX IE in the RAN PAGING message:

Includes the RAN Paging cycle when eDRX has been configured, otherwise the shortest of the RAN paging cycle and the UE specific paging cycle, if allocated by upper layer.

In case the anchor RAN node does not support eDRX, it cannot distinguish whether eDRX has been configured or not, therefore with the above proposed change, the anchor NG-RAN node will provide the “shortest of the RAN paging cycle and the UE specific paging cycle” as the Paging DRX IE in the RAN PAGING message. As the eDRX is coordinated between UE and CN via NAS, if the UE is currently camping on a cell which supports and enables eDRX, the UE will monitor paging using the shortest of the RAN paging cycle and the UE specific paging cycle and the default paging cycle within the PTW, and monitor RAN paging cycle outside of PTW, paging missing will happen outside of the PTW.

For example, in case the RAN Paging cycle is 512, UE specific DRX is 256, default paging cycle is 256, and the anchor RAN node will provide 256 to the neighbour RAN node in XnAP: RAN PAGING message. And then the neighbour RAN node will use 256 to perform RAN paging, but the UE will monitor 512 outside of PTW,

Observation 4: in case the anchor NG-RAN node does not support eDRX, the RAN paging cycle has to be included as the Paging DRX IE in the RAN PAGING message, otherwise paging missing will occur.

With the above proposed change, it is also not clear what will be provided to the neigbour NG-RAN node, in case the anchor RAN node supports eDRX but it is configured to not use eDRX over the radio.

Observation 5: the “when eDRX has been configured” is confused from RAN point of view, does it mean eDRX has been configured to be allowed/not allowed by the last serving cell, or does it means that whether eDRX has been coordinated between UE and CN via NAS? The latter one is not aware by RAN.

Observation 6: The proposed semantics description in [2] and [3] is problematic in case the anchor RAN node does not support eDRX, and is not clear in case the anchor RAN node supports eDRX but it is configured to not use eDRX over the radio.

As discussed during RAN3#111 and the analyses above, the safe way to add definition/semantics description of the Paging DRX IE in the RAN PAGING message, is to clarify that the Paging DRX IE carries the RAN paging cycle.

Note that, how to set RAN Paging cycle, is up to anchor RAN node implementation, it can set the RAN Paging DRX to the UE with the value same as UE specific DRX, smaller than the UE specific DRX, or bigger than the UE specific DRX.

Proposal: Clarify that the Paging DRX IE in XnAP: RAN PAGING message carries the RAN paging cycle.

4. Conclusion and proposals

- In this contribution, we analyses the “to be continued” issue on paging DRX of inactive UEs, get the following observations and proposal:

Observation 1: eDRX is an optional feature and may not supported by some NG-RAN nodes.

Observation 2: a cell supporting eDRX may broadcast that eDRX is not allowed.

Observation 3: NG-RAN node is not aware of whether eDRX is supported/allowed or not in other cells of neighbouring NG-RAN nodes.

Observation 4: in case the anchor NG-RAN node does not support eDRX, the RAN paging cycle has to be included as the Paging DRX IE in the RAN PAGING message, otherwise paging missing will occur.

Observation 5: the “when eDRX has been configured” is confused from RAN point of view, does it mean eDRX has been configured to be allowed/not allowed by the last serving cell, or does it means that whether eDRX has been coordinated between UE and CN via NAS? The latter one is not aware by RAN.

Observation 6: The proposed semantics description in [2] and [3] is problematic in case the anchor RAN node does not support eDRX, and is not clear in case the anchor RAN node supports eDRX but it is configured to not use eDRX over the radio.

Proposal: Clarify that the Paging DRX IE in XnAP: RAN PAGING message carries the RAN paging cycle.

- The corresponding CR is provided in [4].

5. Reference

- R3-211055 Summary of Discussion for DRXinfo_delivery_inactive, ZTE

- R3-211315 Correction on the DRX information delivery for RRC_INACTIVE UE, Rel-16 CR to TS38.423, ZTE, Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell

- R3-211317 Correction on Paging DRX information for RRC_INACTIVE UE, Rel-16 CR to TS38.423, Nokia, Nokia Shanghai Bell, ZTE, Qualcomm, Ericsson

- R3-211538 Clarification on Paging DRX in RAN PAGING message, Rel-16 CR to TS 38.423, Huawei