3GPP TSG-RAN WG3 #112-e								R3-211586

17-28 May 2021

Online

Source:               ZTE Corporation

Title:                    Discussion on I-RNTI partitioning

Agenda item:      31.2.1

Document for:    Discussion and Decision

- Background

In the previous meeting, we have the following progress.

Previous in R3-206827, R3-206821 (noted) Previous summary of offline disc. in R3-206967 (noted) A standardized solution enabling an inter vendor interoperable way for an NG RAN node to deduce the identity of another NG RAN node from the received I-RNTI is needed Agree on the benefits of a solution that allows at least some flexibility in the selection of the Local Node ID length; further details FFS Previous summary of offline disc. in R3-211131 (noted) The description in the informative Annex C of TS38.300 is not sufficient, and a fully standardized solution to minimize OAM configuration needs to be produced by RAN3 The solution shall support flexible assignment of the maximum number of Inactive UE contexts per NG-RAN node The maximum number of Inactive UE Contexts may differ between NG-RAN nodes, and it may be changed after node deployment in a semi-static manner. Continue discussion on Xn-based solutions; other solutions (e.g. additional bits in I-RNTI) are not precluded Solutions based on OAM configuration may be considered if they fulfil requirements of: - flexibility in allocation of maximum number of Inactive UE contexts per NG-RAN node - Interoperability between vendors - support for RAN sharing - Minimized configuration effort To be continued...

In this paper, we continue to discuss this issue.

- Discussion

In order to support flexible assignment of the maximum number of Inactive UE contexts per NG-RAN node, in the paper [1], three alternative methods are provided to support flexible I-RNTI partitioning. Furthermore, the alternative 3 has much interesting.

- The length of (short) NG-RAN Node ID part in I-RNTI is self-contained in the I-RNTI, e.g., to separate 2bits in I-RNTI for using as I-RNTI structure indication.

- For this Alt3, a fixed number of bits in I-RNTI can be assigned to indicate the length of (short) NG-RAN Node ID, e.g., calling is I-RNTI structure indication. When a gNB receives the resume request, it will firstly resolve these bits to acquire the length of (short) NG-RAN Node ID. Based on this exact length of (short) NG-RAN Node ID, the gNB can determine the true structure of this I-RNTI and uniquely determine (short) NG-RAN Node ID.

- Such self-contained scheme has impact on the original structure/length of I-RNTI that will result in a reduction of the number of NG-RAN Node ID and UE context ID that can be indicated. In order to alleviate such impact, it’ll be better to assign as less bits as possible, e.g., 2 bits as full I-RNTI structure indication, and 1bits 2 bits as short I-RNTI structure indication. That means at most 4 of I-RNTI structures can be indicated for full I-RNTI structure indication, and at most 2 of I-RNTI structures can be indicated for short I-RNTI structure. The specification can assign 4 most likely lengths that have a reasonable distribution for full I-RNTI structure indication and 2 most likely lengths that have a reasonable distribution for short I-RNTI structure indication. Even in the future a vendor may have some different thinking about the length, they can still find the closest value to use. An example coding for such 3-bits I-RNTI structure bits might be as following:

Table 2

| Coding of 2bits for full I-RNTI structure indication | The indicated length of NG-RAN node ID | The indicated length of UE context ID |
| --- | --- | --- |
| 00 | 12 bits | 26 bits |
| 01 | 15 bits | 23 bits |
| 10 | 18 bits | 20 bits |
| 11 | 21 bits | 17 bits |
| Note | The indicated length = 12+3*value of the 2bits | The indicated length = 40-2 -(12+3*value of the 2bits) |

- The above example is for fullI-RNTI structure. For shortI-RNTI, it can have further less bits for I-RNTI structure indication, e.g., 1 bits. That means we need different coding for these indication bits.

If this Alt3 is preferred, the main change can be as following example (with assumption short NG-RAN node ID is agreed):

9.2.3.40 UE Context ID This IE is used to address a UE Context within an NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE UE Context ID; M. >RRC Resume. >>I-RNTI; M; 9.2.3.46. >>Allocated C-RNTI; M; BIT STRING (SIZE (16)); Temporary C-RNTI allocated to the UE by the cell where the RRC connection has been requested to be resumed, contained in the MAC RAR as defined in TS 38.321 [35] or in TS 36.321 [36].. >>Access PCI; M; NG-RAN Cell PCI 9.2.2.10; The cell PCI where the RRC connection has been requested to be resumed.. >RRC Reestablishment. >>C-RNTI; M; BIT STRING (SIZE (16)); C-RNTI contained in the RRCReestablishmentRequest message (TS 38.331 [10]) or RRCConnectionReestablishmentRequest message (TS 36.331 [14]).. >>Failure Cell PCI; M; NG-RAN Cell PCI 9.2.2.10. 9.2.3.46 I-RNTI The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node. The I-RNTI is partitioned into two parts, the first part identifies the NG-RAN node that allocated the I-RNTI and the second part identifies the UE context stored in this NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE I-RNTI. >I-RNTI full. >>I-RNTI full; M; BIT STRING (SIZE (40)); This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 40 bits (refer to I-RNTI-Value IE in TS 38.331 [10] and I-RNTI IE in TS 36.331 [14]). The most significant 2bits indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, e.g., xbits (x=12+3*value of the 2bits). In the remaining bits of I-RNTI, the most significant xbits identify the NG-RAN Node ID part of this NG-RAN Node and the least significant (40-2-x) bits identify the UE Context stored at the identified NG-RAN Node.. >I-RNTI short. >>I-RNTI short; M; BIT STRING (SIZE (24)); This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 24 bits (refer to ShortI-RNTI-Value IE in TS 38.331 [10] and ShortI-RNTI IE in TS 36.331 [14]). The most significant 1bits indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, e.g., xbits (x=6+2*value of the 1bits). In the remaining bits of I-RNTI, the most significant xbits identify the NG-RAN Node ID part of this NG-RAN Node and the least significant (24-1-x) bits identify the UE Context stored at the identified NG-RAN Node..

Proposal 1: In order to support flexible assignment of the maximum number of Inactive UE contexts per NG-RAN node, the length of (short) NG-RAN Node ID part in I-RNT shall be feasible and configured.

Proposal 2: The length of (short) NG-RAN Node ID part in I-RNTI is self-contained in the I-RNTI

If the above proposal2 are agreed, the corresponding CR is [2].

- Conclusions

In this contribution, we make the following proposals:

- Proposal 1: In order to support flexible assignment of the maximum number of Inactive UE contexts per NG-RAN node, the length of (short) NG-RAN Node ID part in I-RNT shall be feasible and configured.

- Proposal 2: The length of (short) NG-RAN Node ID part in I-RNTI is self-contained in the I-RNTI

If the above proposal2 are agreed, the corresponding CR is [2].

- References

- R3-210195  Discussion on I-RNTI partitioning, ZTE

- R3-211587 CR38423 for addition of I-RNTI structure indication