3GPP TSG-RAN WG3 #112-e								R3-211598

Online, 17 May – 27 May 2021

Source:               ZTE

Title:                    Analysis of New QoS Related parameters

Agenda item:      21.3

Document for:    Discussion and Decision

Introduction

The new WID of NR Industrial Internet of Things (IoT) and URLLC support was approved in RAN#86 and revised in RAN#88e [1]. In which, the following objective is included:

... - RAN enhancements based on new QoS related parameters if any, e.g. survival time, decided from SA2. [RAN2, RAN3]

Based on the discussion in RAN3#111e meeting, the following agreements on new QoS related parameters have been achieved[2]:

- Introduce Survival Time into RAN3 specifications in principle. FFS on the details e.g. minimum and maximum value of Survival Time. - Introduce Survival Time at NG/Xn/E1/F1 interface. - Survival Time is included for downlink. It’s still FFS whether to include Survival Time for uplink - Survival Time can be expressed with the time. FFS on e.g., the exact encoding, whether aperiodic type is allowed. - Wait for RAN2/SA2 on new TSCAI parameters.

In this contribution, we will mainly discuss the potential RAN3 impacts of survival time. Then we’ll give our proposals.

Discussion

FFS on the details of Survival Time

Based on the discussion in RAN3#111e meeting, some companies think that the value of survival time can be defined as a multiple of the data burst periodicity in TSCAI because they think that survival time is only used in periodic deterministic communications. However, considering the fact that survival time is used in aperiodic deterministic communication, we think that in order to facilitate the extension of future research, RAN3 can define the survival time by using a general unit (e.g. time value).

In the case of "time" as the survival time unit, we analyze the value range of survival time through the description of cases in TS 22.104. For the value range of survival time, it can be analyzed from two aspects: periodic deterministic communication and aperiodic deterministic communication.

In periodic deterministic communication, according to the performance requirements of periodic deterministic communication service in Table 5.2-1 of [3], the value of survival time is related to the transmission interval. In some scenarios, the survival time value is greater than or equal to three times the transmission interval (e.g., wired-2-wireless 100Mbit/s link replacement, plant asset management). Therefore, based on the fact that the periodicity of the TSC QoS flow is equal to the above transmission interval, the value of survival time is three times of the periodicity value, where the periodicity value is (0,..., 640000,...)us. And since the unit of periodicity is 1us, the unit of survival time also needs to be set 1us.

In aperiodic deterministic communication, based on the performance requirements of aperiodic deterministic communication in Table 5.3-1 of [3], it can be seen that only one scenario provides the value of survival time of 16ms.

Observation 1: survival time in terms of "time" can be used to describe both periodic and aperiodic deterministic communication, and is helpful for future research.

Observation 2: The range of survival time in periodic communication includes the value of survival time in aperiodic communication, and the unit of survival time is the same as the unit of periodicity.

Proposal 1 : The minimum and maximum values of survival time are 0us and 1920000us respectively, and the unit of survival time is 1us.

FFS on the uplink transmission of Survival Time

In the uplink transmission, as a new application layer parameter, how to use the survival time on the RAN side has not been concluded. Therefore, whether the survival time needs to be transmitted to the UE side needs to be further discussed. In other words, in the uplink transmission, there are two choices for the transmission of survival time: (i) transmission to gNB, (ii) transmission to UE. As we know in uplink transmission, UE can be seen as the sender and gNB can be seen as receiver. It may be more straightforward to let sender (UE) measure loss or failure on packet transmission and perform survival time counting. But it’s still possible for receiver (gNB) to perform this thing since receiver also can identify the failure on packet transmission based on the knowledge of granted resources. Therefore, we think it is necessary for RAN2 to further discuss the above two options.

Observation 3: In the uplink transmission, there are two options for the transmission of the survival time, which need to wait for the conclusion of RAN2.

FFS on whether aperiodic type is allowed

Based on the conclusion of the RAN2#113, R17 mainly studies the application method of survival time in periodic deterministic communication, and the following agreements have been achieved[4]:

- Only periodic traffic is considered for survival time work in Rel-17

Based on the above conclusion, we don't think it is necessary to study the scenario of survival time application in aperiodic deterministic communication.

Observation 4: R17 mainly studies the application method of survival time in periodic deterministic communication.

Based on the above analysis, we think that we need to add the downlink optional parameter survival time and the value range of this parameter to the TSC assistance information IE in TS38.413, TS38.423, TS38.463 and TS38.473. Due to the limitation of the number of documents, we provide the related example text proposal in appendix and the draft CR for TS 38.413 [5].

Conclusions

In this contribution, we make the following proposals:

Observation 1: survival time in terms of "time" can be used to describe both periodic and aperiodic deterministic communication, and is helpful for future research.

Observation 2: The range of survival time in periodic communication includes the value of survival time in aperiodic communication, and the unit of survival time is the same as the unit of periodicity.

Observation 3: In the uplink transmission, there are two options for the transmission of the survival time, which need to wait for the conclusion of RAN2.

Observation 4: R17 mainly studies the application method of survival time in periodic deterministic communication.

Proposal 1 : The minimum and maximum values of survival time are 0us and 1920000us respectively, and the unit of survival time is 1us.

- References

- 3GPP, RP-201310, Revised WID: Core part: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, RAN #88e, June, 2020.

- 3GPP, RAN3_111-e_agenda_with_Tdocs20210204_EOM, RAN3#111, February 2021.

- 3GPP, TS 22.104-g50 V16.5.0 "Service requirements for cyber-physical control applications in vertical domains; Stage 1 (Release 16)".

- 3GPP R2-2101954, Report for Rel-17 Small data and URLLC/IIoT and Rel-16 NR-U, Power Savings, and 2step RACH, Jan 25-Feb 5, 2021.

- 3GPP, R3-211599 CR for TS38.413 on new QoS related parameters, ZTE.

- Appendix

## <38.413 example text proposal>

<Start of the modified section>

9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  |
| Survival Time | O |  | 9.3.1.X |  |

FFS:	Whether Survival Time should be included in TSC assistance information Uplink.

9.3.1.X	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Survival Time expressed in units of 1 us. |

<Next of the modified section>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//SKIP the unrelated part//

id-SRVCCOperationPossible,

id-SupportedTAList,

id-SurvivalTime

id-Suspend-Request-Indication,

id-Suspend-Response-Indication,

id-TAI,

id-TAIListForPaging,

#### -- S

//SKIP the unrelated part//

SupportedTAItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication	PRESENCE optional	}|

{ID id-RAT-Information				CRITICALITY reject	EXTENSION RAT-Information			PRESENCE optional	},

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SuspendIndicator ::= ENUMERATED {

true,

...

}

Suspend-Request-Indication ::= ENUMERATED {

suspend-requested,

...

}

Suspend-Response-Indication ::= ENUMERATED {

suspend-indicated,

...

}

#### -- T

//SKIP the unrelated part//

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime		CRITICALITY ignore			EXTENSION SurvivalTime			PRESENCE OPTIONAL},

...

}

//SKIP the unrelated part//

### 9.4.7	Constant Definitions

//SKIP the unrelated part//

id-QosFlowFeedbackList									ProtocolIE-ID ::= 278

id-BurstArrivalTimeDownlink								ProtocolIE-ID ::= 279

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-SurvivalTime											ProtocolIE-ID ::= 2xy

<End of the modified section>

## <38.423 example text proposal>

<Start of the modified section>

9.3.1.115	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.116 |  |
| Burst Arrival Time | O |  | 9.3.1.117 |  |
| Survival Time | O |  | 9.3.1.X |  |

FFS:	Whether Survival Time should be included in TSC assistance information Uplink.

9.3.1.X	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Survival Time expressed in units of 1 us. |

<Next of the modified section>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//SKIP the unrelated part//

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-SurvivalTime

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

#### -- S

//SKIP the unrelated part//

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SymbolAllocation-in-Slot ::= CHOICE {

allDL				SymbolAllocation-in-Slot-AllDL,

allUL				SymbolAllocation-in-Slot-AllUL,

bothDLandUL			SymbolAllocation-in-Slot-BothDLandUL,

choice-extension	ProtocolIE-Single-Container { {SymbolAllocation-in-Slot-ExtIEs} }

}

#### -- T

//SKIP the unrelated part//

TSCAssistanceInformation ::= SEQUENCE {

periodicity				INTEGER (0.. 640000, ...),

burstArrivalTime		OCTET STRING				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime			CRITICALITY ignore			EXTENSION SurvivalTime			PRESENCE OPTIONAL},

...

}

//SKIP the unrelated part//

### 9.3.7	Constant definitions

//SKIP the unrelated part//

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-SurvivalTime																						ProtocolIE-ID ::= 2xy

//SKIP the unrelated part//

<End of the modified section>

## <38.463 example text proposal>

<Start of the modified section>

9.3.1.76	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.77 |  |
| Burst Arrival Time | O |  | 9.3.1.78 |  |
| Survival Time | O |  | 9.3.1.X |  |

FFS:	Whether Survival Time should be included in TSC assistance information Uplink.

9.3.1.X	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Survival Time expressed in units of 1 us. |

<Next of the modified section>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

//SKIP the unrelated part//

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-SurvivalTime

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

#### -- S

//SKIP the unrelated part//

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

SurvivalTime ::= INTEGER (0..1920000, ...)

#### -- T

//SKIP the unrelated part//

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime			CRITICALITY ignore			EXTENSION SurvivalTime			PRESENCE OPTIONAL},

...

}

Periodicity					::= INTEGER	(1..640000,	...)

BurstArrivalTime			::= OCTET STRING

//SKIP the unrelated part//

### 9.4.7	Constant Definitions

//SKIP the unrelated part//

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-SurvivalTime													ProtocolIE-ID ::= 1xy

<End of the modified section>

## <38.473 example text proposal>

<Start of the modified section>

9.3.1.142	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.143 |  |
| Burst Arrival Time | O |  | 9.3.1.144 |  |
| Survival Time | O |  | 9.3.1.X |  |

FFS:	Whether Survival Time should be included in TSC assistance information Uplink.

9.3.1.X	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Survival Time expressed in units of 1 us. |

<Next of the modified section>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//SKIP the unrelated part//

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-SurvivalTime

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

#### -- S

//SKIP the unrelated part//

SupportedSULFreqBandItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SymbolAllocInSlot ::= CHOICE {

all-DL				NULL,

all-UL				NULL,

both-DL-and-UL			NumDLULSymbols,

choice-extension			ProtocolIE-SingleContainer { { SymbolAllocInSlot-ExtIEs } }

}

#### -- T

//SKIP the unrelated part//

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime				CRITICALITY ignore			EXTENSION SurvivalTime			PRESENCE OPTIONAL},

...

}

//SKIP the unrelated part//

### 9.4.7	Constant Definitions

//SKIP the unrelated part//

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SurvivalTime										ProtocolIE-ID ::= 4xy

//SKIP the unrelated part//

<End of the modified section>