3GPP TSG-RAN WG3 Meeting #112-e	R3-211603

Online, 17 – 27 May 2021

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	E-CID LTE measurement in Rel-15

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has received an LS from RAN2 in [1], requesting RAN3 to “confirm whether gNB can report E-UTRA measurement to the LMF for UL E-CID positioning in Rel-15”.

In this paper, we analyze what is supported by the gNB in Rel-15 for UL E-CID and provide some proposals.

# 2	Discussion

The NRPPa procedures are used to transfer positioning related information between LMF and NG-RAN node, which can be either a gNB or ng-eNB.

In case of UL E-CID, the NG-RAN node provides the E-CID related measurements/information in the E-CID MEASUREMENT INITIATION RESPONSE (in case of on-demand) or E-CID MEASUREMENT REPORT (in case of periodic). A Rel-15 ng-eNB can provide the following:

1)	Cell Portion ID IE;

2)	E-CID Measurement Result IE containing CGI+TAC of the serving cell, NG-RAN access point position, and measured results for E-UTRA;

3)	Other-RAT Measurement Result IE containing GERAN and/or UTRAN measurements (applicable only in case of on-demand).

A Rel-15 gNB can provide all the above except the measured results for E-UTRA.  This is because:

-	the Measured Results IE contained in the E-CID Measurement Result IE is intended to be applicable only to the serving RAT; and

-	the Other-RAT Measured Results IE contained in the Other-RAT Measurement Result IE is applicable to non-serving RATs but does not support E-UTRA measurements in Rel-15.

Therefore, in response to the question from RAN2, the gNB cannot report E-UTRA measurements to the LMF for UL E-CID positioning in Rel-15.

One potential reason for the confusion is that NRPPa does not explicitly prohibit a Rel-15 gNB from including measured results for E-UTRA within the E-CID Measurement Result IE. However, the original intention was for this IE to contain only measurements for the serving RAT like in LPPa (from where this IE was copy/pasted). Since the NG-RAN node can have either of two serving RATs, NRPPa could be further clarified by adding a semantics description as follows:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> |  | Measurement results of the serving RAT. |
| >CHOICE Measured Results Value | M |  |  |  |
| […] |  |  |  |  |

# 3	Conclusions

In this paper, we analyzed whether a gNB can report E-UTRA measurements to the LMF for UL E-CID in Rel-15.  Based on the analysis, the following is proposed:

Proposal 1:	Respond to RAN2 that the gNB cannot report E-UTRA measurements to the LMF for UL E-CID positioning in Rel-15.

Proposal 2:	Add a semantics description to the Measured Results IE clarifying that it is the measurement results of the serving RAT.

For proposal 2, NRPPa CRs for R15 and R16 are provided in [2] and [3] respectively.

# References

R3-211421, LS on E-CID LTE measurement in Rel-15, RAN2

R3-211604, Clarification of E-CID Measurement Result (Rel-15 CR), Nokia, Nokia Shanghai Bell, Huawei

R3-211605, Clarification of E-CID Measurement Result (Rel-16 CR), Nokia, Nokia Shanghai Bell, Huawei