3GPP TSG-RAN WG3 Meeting #112-e                                                            	R3-211849

17 – 27 May 2021,E-Meeting

Title:	[Draft]LS on MRO for CHO mobility Enhancement

Response to:

Release:	Rel-17

Work Item:	          NR_ENDC_SON_MDT_enh-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name:	Liu Aijuan

E-mail Address:	Liuaijuan (at) catt (dot) cn

- Overall Description:

RAN3 discussed the scenario on CHO and reach the following agreements:

- RAN3 agreed to consider the following cases on CHO:

  - Case 1,3,5 for too late HO case

  - Case 1,2 for too early HO case

  - Case 1-10 for HO to wrong cell case

The cases are depicted in the annex.

  - For CHO to wrong cell, in case RLF occurs shortly after CHO recovery succeed, a timer between CHO recovery success and RLF occur is needed to identify whether UE can keep stable after CHO recovery success.

  - For CHO too late handover, the time duration from the last HO initialization to connection failure which happens after CHO configuration is useful for failure detection.

  - For CHO candidate cell list, it is not always feasible for the network to keep the CHO candidate cell list, so it is more reliable that the UE could record CHO candidate cell list in RLF-report.

- 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account and specify accordingly.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #113-e	    17th – 28th August 2021

4. Annex  Use case for CHO failure

1 CHO HO too late

2 CHO HO too early

3 CHO to wrong cell