3GPP TSG-RAN WG3 #112 electronic	R3-211875

Online, May 17 – 27, 2021

Agenda Item:	22.2.4

Source:	CATT

Title:	TP for 38.473 on MBS Context Management for Broadcast at F1 interface

Document for:	Discussion, Decision

1	Introduction

In last RAN3 meeting, there is no progress on MBS Context management over F1/E1, the following conclusion is still from RAN3#110-e meeting:

-	F1/E1 MBS Bearer management procedure can be discussed, but details on e.g. information to signal are pending RAN2/SA2 progress

In this contribution, we provide some proposals on MBS Context management procedure over F1 interface and the corresponding TPs is annexed below.

2	Discussion

According to the email discussion of the last RAN3 meeting, All companies agree that using non-UE associated F1/E1 procedures to setup the corresponding MBS context and shared F1-U/NG-U tunnel for a broadcast session.

Thus, at least for broadcast session, a new non-UE associated MBS Context management procedure needs to be introduced into F1 interface. For example, at least a MBS context setup procedure (class 1) needs to be specified in F1AP.

Observation 1: At least for broadcast session, a MBS context setup procedure (class 1) needs to be specified in F1AP.

Similar to the existing UE context management procedure over F1 interface, in addition to defining a MBS context setup procedure, the modification and deletion procedure of PTM bearer should also be considered, specifically, the gNB-CU initiated MBS context modification procedure is needed to allow the gNB-CU to modify a PTM bearer context in the gNB-DU. In addition, there is a need to introduce the gNB-DU initiated MBS context modification required procedure to allow the gNB-DU to modify a PTM bearer context (e.g., due to radio condition issue) and inform the gNB-CU.

Proposal 1: At least for broadcast session, a gNB-CU initiated MBS context modification procedure (class 1) should be specified in F1AP.

Proposal 2: At least for broadcast session, a gNB-DU initiated MBS context modification required procedure (class 1) should be specified in F1AP.

Finally, both gNB-CU and gNB-DU initiated MBS context release procedures need to be introduced into the F1 interface. Similar to the existing UE context release procedures, a gNB-CU initiated MBS context release procedure (class 1) should be specified in F1AP, and a gNB-DU initiated MBS context release request procedure (class 2) should be specified in F1AP.

Proposal 3: At least for broadcast session, a gNB-CU initiated MBS context release procedure (class 1) should be specified in F1AP.

Proposal 4: At least for broadcast session, a gNB-DU initiated MBS context release request procedure (class 2) should be specified in F1AP.

3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: At least for broadcast session, a MBS context setup procedure (class 1) needs to be specified in F1AP.

Proposal 1: At least for broadcast session, a gNB-CU initiated MBS context modification procedure (class 1) should be specified in F1AP.

Proposal 2: At least for broadcast session, a gNB-DU initiated MBS context modification required procedure (class 1) should be specified in F1AP.

Proposal 3: At least for broadcast session, a gNB-CU initiated MBS context release procedure (class 1) should be specified in F1AP.

Proposal 4: At least for broadcast session, a gNB-DU initiated MBS context release request procedure (class 2) should be specified in F1AP.

4	Reference

- RP-201038    “New Work Item on NR Multicast and Broadcast Services”

- R3-211029	Summary of discussion on MBS Bearer Management over F1/E1, CATT

5	Text Proposals for stage 3

5.1	TP for TS 38.473

--------------------------------Start of the First Change-----------------------------

8	F1AP procedures

8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  |  |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  |  |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  |  |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| MBS Context Setup |  | MBS CONTEXT SETUP REQUEST |  | MBS CONTEXT SETUP RESPONSE |  | MBS CONTEXT SETUP FAILURE |  |
| MBS Context Release (gNB-CU initiated) |  | MBS CONTEXT RELEASE COMMAND |  | MBS CONTEXT RELEASE COMPLETE |  |  |  |
| MBS Context Modification (gNB-CU initiated) |  | MBS CONTEXT MODIFICATION REQUEST |  | MBS CONTEXT MODIFICATION RESPONSE |  | MBS CONTEXT MODIFICATION FAILURE |  |
| MBS Context Modification Required (gNB-DU initiated) |  | MBS CONTEXT MODIFICATION REQUIRED |  | MBS CONTEXT MODIFICATION CONFIRM |  | MBS CONTEXT MODIFICATION REFUSE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| MBS Context Release Request (gNB-DU initiated) |  | MBS CONTEXT RELEASE REQUEST |  |

------------------------------------------Next Change---------------------------------------

8.X	MBS Context Management procedures

8.X.1	MBS Context Setup

8.X.1.1	General

The purpose of the MBS Context Setup procedure is to establish the MBS Context including MRB configuration.

8.X.1.2	Successful Operation

TBD

8.X.1.3	Unsuccessful Operation

TBD

8.3.X.4	Abnormal Conditions

TBD

8.X.2	MBS Context Modification (gNB-CU initiated)

8.X.2.1	General

The purpose of the MBS Context Modification procedure is to modify the established MBS Context, e.g., establishing, modifying and releasing radio resources for PTM bearer.

8.X.2.2	Successful Operation

TBD

8.X.2.3	Unsuccessful Operation

TBD

8.X.2.4	Abnormal Conditions

TBD

8.X.3	MBS Context Modification Required (gNB-DU initiated)

8.X.3.1	General

The purpose of the MBS Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a MBS bearer context (e.g., due to local problems) and inform the gNB-CU-CP.

The purpose of the MBS Context Modification Required procedure is to modify the established MBS Context, e.g., modifying and releasing radio resources for PTM bearer.

8. X.3.2	Successful Operation

TBD

8. X.3.3	Abnormal Conditions

TBD

8.X.4	MBS Context Release (gNB-CU initiated)

8.X.4.1	General

The purpose of the MBS Context Release procedure is to enable the gNB-CU to order the release of the MBS context associated with a specific MBS session.

8.X.4.2	Successful Operation

TBD

8.X.4.3	Abnormal Conditions

TBD

8.X.5	MBS Bearer Context Release Request (gNB-DU initiated)

8.X.5.1	General

The purpose of the MBS Context Release Request procedure is to enable the gNB-DU to request the gNB-CU to release the MBS context associated with a specific MBS session.

8.X.5.2	Successful Operation

TBD

8.X.5.3	Abnormal Conditions

TBD

------------------------------------------Next Change---------------------------------------

9.2.X	MBS Context Management messages

9.2.X.1	MBS CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a MBS context.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |
| MBS Session Level QoS parameters | M |  |  |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.2	MBS CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a MBS context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.3	MBS CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the MBS context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD. |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.4	MBS CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide MBS Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Level QoS parameters | O |  |  |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.5	MBS CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a MBS context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.6	MBS CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate a MBS context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | Cause |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Criticality Diagnostics |

9.2.X.7	MBS CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the modification of a MBS context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.8	MBS CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU to inform the gNB-DU the successful modification.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.9	MBS CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the MBS context associated with a specific MBS session.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.X.10	MBS CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the MBS context associated with a specific MBS session.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.11	MBS CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to release the MBS context associated with a specific MBS session.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TBD |  |  |  |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |

- -------------------------------------End of the Changes----------------------------------