3GPP TSG-RAN WG3 #112-e		                                   R3-211876

17 – 28 May,2021

Online

Source:	CATT,CBN, Huawei

Title:	TP for 38.413 on session management for broadcast service

Agenda Item:	22.2.2

Document for:	Discussion and Decision

Introduction

In RAN WG3 #110-e meeting, there were some discussions and agreements on session management in NG interface. In this contribution, we make further analysis on this topic and provide TP for 38.413 on session management for broadcast service.

Discussion

In last RAN3 meeting, it is agreed to have MBS Session Start/Release procedure for Broadcast but naming is FFS.

In the meanwhile, the following conclusions were included in the latest version of TR23.757:

- For MBS session management the following conclusions are reached as baseline for normative work:

The MBS session is identified throughout the 5G system transport on external interface towards AF and between AF and UE, and towards the UE with an MBS Session ID.

-	MBS Session ID may have the following types: TMGI (MBS broadcast and multicast Session), source specific IP multicast address (MBS multicast Session).

-	For MBS multicast Session, source specific IP multicast address can be assigned by 5GC or external network.

-	TMGI definition is updated for 5G MBS to be able to identify the MBS session when used for the Multicast Session Context and to identify the MBS service when used for the Multicast service context. (see Figure 8.2.2.2-1)

So, based on above agreements, it is clear that MBS Session Start Request message should include MBS session ID which is used to identify the broadcast service. Also, it is natural that QoS information of each flow in the MBS session and the service area should be included in this message. Then as to the N3 tunnel setup, it is already agreed that GTP-U tunnelling using a transport layer IP multicast method and shared N3 (GTP-U) Point-to-Point tunnel shall be supported. Since two options are available, one question needs to be resolved is which node makes the decision on whether point-to-point (PTP) tunnelling or GTP-U tunnelling using a transport layer IP multicast should be used.

From our point of view, the final decision could be decided by gNB. If GTP-U tunnelling using a transport layer IP multicast is supported by 5GC, tunnel information of UPF would be provided to gNB. After receiving the address, if it is not supported by gNB, it would provide downlink tunnel address towards 5GC to inform 5GC that PTP tunnelling should be used for this gNB. Otherwise, transport layer IP multicast would be used.

Then as to how to transfer the MBS context from 5GC to gNB, i.e. gNB drives the information directly from AMF or from SMF via a container, considering the separation of SM and MM in 5GC, it is reasonable to transmit the MBS session related information between AMF and gNB via a SM container which is transparent to AMF. So, for MBS session start procedure, we have the following proposals:

Proposal 1: For Broadcast, it is proposed to include MBS session ID IE, service area IE and a SM container named as MBS Session Resource Setup Request Transfer IE which includes per flow level QoS information and IP multicast address in MBS Session Start Request message.

Proposal 2: For Broadcast, it is proposed to include a SM container named as MBS Session Resource Setup Response Transfer IE which includes DL GTP tunnel for Point-to-Point transmission mode in MBS Session Start Response message.

For MBS Session Release procedure, to identify the broadcast service that should be released in gNB side, the MBS session ID which is included in MBS session start procedure should be included. Simiarly to the MBS session start procedure, there should be a SM container transferred between SMF and gNB. For the SM container from SMF to gNB i.e. MBS Session Resource Release Request Transfer IE,it could include the cuase to release the broadcast session. It is FFS which information should be included in MBS Session Resource Release Response Transfer IE in the response message. Then, for MBS Session Release procedure, the proposal is as below:

Proposal 3: For Broadcast, it is proposed to include MBS session ID and SM container i.e. MBS Session Resource Release Request Transfer IE, MBS Session Resource Release Response Transfer IE in MBS Session Stop Request and MBS Session Stop Response message separately.

Besides MBS Session Start and MBS Session Release procedure, we think another procedure i.e. MBS Session Modify which is used to update the information for broadcast service should also be introduced. The reason is that it is possible that the service area and QoS information of the flows in broadcast session is updated after the broadcast service is started. With MBS Session Modify procedure introduced, the gNB could update the corresponding MBS context based on the request from 5GC. As to the user plane information, for the time being, we could not see strong requirement to update. It could be discussed further.

Proposal 4: It is proposed to introduce MBS session Modify procedure for broadcast service. MBS session ID, service area and SM container MBS Session Resource Modify Request Transfer IE should be included in MBS session Modify Request message. For MBS Session Resource Modify Request Transfer IE, at least QoS information for the flow in MBS session should be included..

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: For Broadcast, it is proposed to include MBS session ID IE, service area IE and a container named as MBS Session Resource Setup Request Transfer IE which includes per flow level QoS information and IP multicast address in MBS Session Start Request message.

Proposal 2: For Broadcast, it is proposed to include a container named as MBS Session Resource Setup Response Transfer IE which includes DL GTP tunnel for Point-to-Point transmission mode in MBS Session Start Response message.

Proposal 3: For Broadcast, it is proposed to include MBS session ID and MBS Session Resource Release Command Transfer IE, MBS Session Resource Release Response Transfer IE in MBS Session Stop Request and MBS Session Stop Response message separately.

Proposal 4: It is proposed to introduce MBS session Modify procedure for broadcast service. MBS session ID, service area and MBS Session Resource Modify Request Transfer IE should be included in MBS session Modify Request message. For MBS Session Resource Modify Request Transfer IE, at least QoS information for the flow in MBS session should be included..

Text Proposal to 38.413

- 8	NGAP Procedures

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| MBS Session Start | MBS SESSION START REQUEST | MBS SESSION START RESPONSE | MBS SESSION START FAILURE |
| MBS Session Modify | MBS SESSION MODIFY REQUEST | MBS SESSION MODIFY RESPONSE | MBS SESSION MODIFY FAILURE |
| MBS Session Stop | MBS SESSION STOP REQUEST | MBS SESSION STOP RESPONSE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |

8.x	MBS Session Management Procedure

8.x.1	MBS Session Start

- 8.x.1.1	General

The purpose of the MBS Session Start procedure is to request the gNB to notify UEs about an upcoming MBS Session of an given broadcast serivce.

The procedure uses non-UE associated signalling.

- 8.x.1.2	Successful Operation

Figure 8.x.1.2-1. MBS Session Start procedure. Successful operation.

The AMF initiates the procedure by sending an MBS SESSION START REQUEST message to the gNB. If the gNB accepts the MBMS session start request, the gNB responds with the MBS SESSION START RESPONSE message.

- 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1. MBS Session Start procedure. Unsuccessful operation.

If the gNB is not capable of correctly processing the request(e.g. the MBS resources could not be established at all in any cell), the gNB shall send MBS SESSION START FAILURE message.

- 8.x.1.4	Abnormal Conditions

Void.

8.x.2	MBS Session Modification

- 8.x.2.1	General

The purpose of the MBS Session Modification procedure is to request gNB to update the information relatd to the previously established MBS session.

The procedure uses non-UE associated signalling.

- 8.x.2.2	Successful Operation

Figure 8.x.2.2-1. MBS Session Start procedure. Successful operation.

The AMF initiates the procedure by sending a MBS SESSION MODIFICATION REQUEST message to the gNB.

If the MBS Service Area IE is included in the MBS SESSION MODIFICATION REQUEST message, the gNB shall update the MBS service area and send the MBS SESSION UPDATE RESPONSE message.

If the MBS Session Resource Modify Request Transfer IE is included in the MBS SESSION MODIFICATION REQUEST message, the gNB shall update the MBS context and resources and send the MBS SESSION MODIFICATION RESPONSE message.

- 8.x.2.3	Unsuccessful Operation

Figure 8.x.2.3-1. MBS Session Modification procedure. Unsuccessful operation.

If the gNB is not capable of correctly processing the request, the gNB shall send MBS SESSION MODIFICATION FAILURE message.

- 8.x.2.4	Abnormal Conditions

Void.

- 8.x.3		MBS Session Stop

- 8.x.3.1	General

The purpose of the MBS Session Stop procedure is to release the MBS context corresponding to the previous established MBS session.

The procedure uses non-UE assocated signalling.

- 8.x.3.2	Successful Operation

Figure 8.x.3.2-1. MBS Session Stop procedure. Successful operation.

The AMF initiates the procedure by sending a MBS SESSION STOP REQUEST message to the gNB.

Upon receiption of the MBS SESSION STOP REQUEST message, the gNB shall respond with the MBS SESSION STOP RESPONSE message. The gNB node shall disable the reception of MBS data and release the affected resources and remove the MBS bearer context.

- 8.x.3.3	Abnormal Conditions

Void.

============ Next change ==============

- 9.2.x		MBS Session Management Messages

- 9.2.x.1	MBS SESSION START REQUEST

This message is sent by the AMF to establish a MBS context .

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Service Area | M |  | FFS |  |  |  |
| MBS Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Setup Request Transfer IE specified in subclause 9.3.4.x |  |  |

- 9.2.x.2	MBS SESSION START RESPONSE

This message is sent by the gNB to report the successful outcome of the request from the MBS SESSION START REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Setup Response Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Setup Response Transfer IE specified in subclause 9.3.4.y |  |  |

- 9.2.x.3	MBS SESSION START FAILURE

This message is sent by the gNB to report the unsuccessful outcome of the request from the MBS SESSION START REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Setup Failure Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Setup Failure Transfer IE specified in subclause 9.3.4.z |  |  |

- 9.2.x.4	MBS SESSION MODIFICATION REQUEST

This message is sent by the AMF to modify a MBS context .

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| Service Area | O |  | FFS |  |  |  |
| MBS Session Resource Modify Request Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Modify Request Transfer IE specified in subclause 9.3.4.xx |  |  |

- 9.2.x.5	MBS SESSION MODIFICATION RESPONSE

This message is sent by the gNB to report the successful outcome of the request from the MBS SESSION MODIFICATION REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Modify ResponseTransfer | O |  | OCTET STRING | Containing the MBS Session Resource Modify Response Transfer IE specified in subclause 9.3.4.yy |  |  |

- 9.2.x.6	MBS SESSION MODIFICATION FAILURE

This message is sent by the gNB to report the unsuccessful outcome of the request from the MBS SESSION MODIFICATION REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Modify Failure Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Modify Failure Transfer IE specified in subclause 9.3.4.zz |  |  |

- 9.2.x.7	MBS SESSION STOP REQUEST

This message is sent by the AMF to release the corresponding MBS context and the MBS-service-associated logical NG connection.

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Release Command Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Release Request Transfer IE specified in subclause 9.3.4.xxx |  |  |

- 9.2.x.8	MBS SESSION STOP RESPONSE

This message is sent by the gNB to acknowledge the MBS SESSION STOP message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Resource Release Response Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Release Response Transfer IE specified in subclause 9.3.4.zzz |  |  |

- 9.3.4.X	MBS Session Resource Setup Request Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TNL Information | O |  |  |  |  |  |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >GTP DL TEID | M |  | 9.3.2.5 |  |  |  |
| QoS Flows To Be Setup List |  | 1 |  |  |  |  |
| >QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |  |  |

- 9.3.4.Y	MBS Session Resource Setup Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Transport Layer Information | O |  | 9.3.2.2 |  |  |  |

- 9.3.4.Z	MBS Session Resource Setup Failure Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |  |  |

- 9.3.4.XX	MBS Session Resource Modify Request Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TNL Information | O |  |  |  |  |  |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >GTP DL TEID | M |  | 9.3.2.5 |  |  |  |
| QoS Flows To Be Setup List |  | 0…1 |  |  |  |  |
| >QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |  |  |

- 9.3.4.YY	MBS Session Resource Modify Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Transport Layer Information | O |  | 9.3.2.2 |  |  |  |

- 9.3.4.ZZ	MBS Session Resource Modify Failure Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |  |  |

- 9.3.4.XXX	MBS Session Resource Release Command Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | O |  | 9.3.1.2 |  |  |  |

- 9.3.4.YYY	MBS Session Resource Release Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| FFS | O |  |  |  |  |  |