3GPP TSG-RAN WG3 Meeting #112-e	R3-211912

E-meeting, 17-28 May 2021

Title: 	(TP to SCG TS 38.401 BL CR) SCG activation and deactivation in disaggregated NG-RAN architecture

Source: 	Huawei, InterDigital

Agenda item:	14.2

Document Type:	other

- Introduction

In this contribution, we provides the TP for 38.401 based on [1] [2].

- Reference

- R3-211910	(TP to SCG TS 38.423/38.473 BL CRs) Remaining open issues on SCG activation and deactivation,	Huawei

- R3-211911	(TP to SCG TS 38.463 BL CR) SCG activation and deactivation in E1,	Huawei

- TP to TS 38.401

### 8.4.x	SCG deactivation/re-activation

This clause gives the NR SCG deactivation/re-activation in MR-DC given that the SgNB consists of a gNB-CU and gNB-DU(s).

SN addition procedure

Figure 8.4.x-1: SCG deactivation/re-activation in SN addition procedure

1~2. refer to TS 37.340 [12].

3~4.The CU-CP of SN sends the UE CONTEXT SETUP REQUEST message to the DU of SN to deactivate the SCG of the UE.

5~6.The CU-CP of SN sends the BEARER CONTEXT SETUP REQUEST message to the CU-UP of SN to deactivate the SCG of the UE.

7~8. refer to TS 37.340 [12].

9. The CU-UP of SN sends the UE INACTIVITY NOTIFICATION to the CU-CP of SN to notifiy the use plane activity.

10~13. refer to TS 37.340 [12].

14~15.The CU-CP of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to re-activate the SCG of the UE.

16~17.The CU-CP of SN sends the BEARER CONTEXT MODIFICATION REQUEST message to the CU-UP of SN to re-activate the SCG of the UE.

18~19. refer to TS 37.340 [12].

SN modification procedure

Figure 8.4.x-2: SCG deactivation/re-activation in SN modification procedure

0-1. The CU-UP of SN sends the UE INACTIVITY NOTIFICATION to the CU-CP of SN to notifiy the use plane inactivity.

0-2~2. refer to TS 37.340 [12].

3~4.The CU-CP of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to deactivate the SCG of the UE.

5~6.The CU-CP of SN sends the BEARER CONTEXT MODIFICATION REQUEST message to the CU-UP of SN to deactivate the SCG of the UE.

7~8. refer to TS 37.340 [12]

9. The CU-UP of SN sends the UE INACTIVITY NOTIFICATION to the CU-CP of SN to notifiy the use plane activity.

10~13. refer to TS 37.340 [12]

14~15.The CU-CP of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to re-activate the SCG of the UE.

16~17.The CU-CP of SN sends the BEARER CONTEXT MODIFICATION REQUEST message to the CU-UP of SN to re-activate the SCG of the UE.

18~19. refer to TS 37.340 [12].