3GPP TSG-RAN WG3 #112-e	R3-212137

17 May– 28 May 2021

Online

Title: 	(TP for SON BL CR for TS 38.413) Introduce UE history information of MR-DC

Source: 	ZTE, Lenovo, Motorola Mobility, China Unicom

Agenda item:	10.2.1.4

Document for:	Approval

1	Introduction and discussion

As discussed in UE history information in MR-DC.

Proposal 3:To confirm that the relationship of each PCell with chronological sequence of PSCell is benefit for Master node.

Proposal 4: Instead of independent structure ,RAN3 to consider two-dimensional structure for SN UHI.

This contribution provided the TP for TS38.413.

2	TP to TS 38.413

*** Start changed ***

9.3.1.95	UE History Information

This IE contains information about cells that a UE has been served by in active state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Last Visited Cell Item |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. |
| >Last Visited Cell Information | M |  | 9.3.1.96 |  |
| >Visited PScells |  | 0..1 |  |  |
| >>Visited PScells |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. |
| >>>Last Visited PSCell Information | M |  | 9.3.1.96 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum no. of cells in the UE history information. Value is 16. |

*** End of changed ***