3GPP TSG-RAN WG3 Meeting #112-e	R3-212196

17 – 27 May 2021

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.460 | CR | 0046 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General principles for eNB CP-UP separation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo_enh-Core |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | E1 interface will be used to support eNBs CP-UP separation. The changes are needed to extend the principles to eNBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support of eNBs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE CP-UP separation is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.1, 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: Resubmission to RAN3#112-e |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

# 1	Scope

The present document is an introduction to the 3GPP TS 38.46x series of technical specifications that define the E1 interface. The E1 interface provides means for interconnecting a gNB-CU-CP and a gNB-CU-UP of a gNB-CU within an NG-RAN, or for interconnecting a gNB-CU-CP and a gNB-CU-UP of an en-gNB within an E-UTRAN, or for interconnecting an eNB* and an eNB-UP of an eNB within an E-UTRAN, or for interconnecting a ng-eNB-CU-CP and a ng-eNB-CU-UP of a ng-eNB-CU within an NG-RAN.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.461: "NG-RAN; E1 layer 1".

[4]	3GPP TS 38.462: "NG-RAN; E1 signalling transport".

[5]	3GPP TS 38.463: "NG-RAN; E1 Application Protocol (E1AP)".

[6]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[7]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[x]	3GPP TS 36.401: "E-UTRAN; Architecture Description”.

[y]	3GPP TS 37.470: "W1 interface; General aspects and principles”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

eNB*: as defined in TS 36.401 [x]

eNB-UP: as defined in TS 36.401 [x]

en-gNB: as defined in TS 37.340 [7]

gNB-CU: as defined in TS 38.401 [2]

gNB-CU-CP: as defined in TS 38.401 [2]

gNB-CU-UP: as defined in TS 38.401 [2]

gNB-DU: as defined in TS 38.401 [2]

gNB: as defined in TS 38.300 [6]

IAB: as defined in TS 38.300 [6]

ng-eNB-CU: as defined in TS 37.470 [y]

ng-eNB-DU: as defined in TS 37.470 [y]

ng-eNB-CU-CP: a logical node hosting the RRC and the control plane part of the PDCP protocol of the ng-eNB-CU for a ng-eNB. The ng-eNB-CU-CP terminates the E1 interface connected with the ng-eNB-CU-UP and the W1-C interface connected with the ng-eNB-DU.

ng-eNB-CU-UP: a logical node hosting the user plane part of the PDCP protocol of the user plane part of the PDCP protocol and the SDAP protocol of the ng-eNB-CU for a ng-eNB. The ng-eNB-CU-UP terminates the E1 interface connected with the ng-eNB-CU-CP and the W1-U interface connected with the ng-eNB-DU.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DL	Downlink

DRB	Data Radio Bearer

E1AP	E1 Application Protocol

IP	Internet Protocol

SCTP	Stream Control Transmission Protocol

TNL	Transport Network Layer

# 4	General aspects

This clause captures the E1 interface principles and characteristics.

## 4.1	E1 interface general principles

The general principles for the specification of the E1 interface are as follows:

-	the E1 interface is open;

-	the E1 interface supports the exchange of signalling information between the endpoints;

-	from a logical standpoint, the E1 is a point-to-point interface between a gNB-CU-CP and a gNB-CU-UP, or between an eNB* and an eNB-UP, or between a ng-eNB-CU-CP and a ng-eNB-CU-UP.

NOTE 1:	A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the E1 interface separates Radio Network Layer and Transport Network Layer;

-	the E1 interface enables exchange of UE associated information and non-UE associated information;

-	the E1 interface is future proof to fulfil different new requirements, support of new services and new functions.

NOTE 2:	The E1 interface is a control interface and is not used for user data forwarding.

## 4.2	E1 interface specification objectives

The E1 interface specifications facilitate the following:

-	inter-connection of a gNB-CU-CP and a gNB-CU-UP supplied by different manufacturers.

-	inter-connection of an eNB* and an eNB-UP supplied by different manufacturers.

-	inter-connection of a ng-eNB-CU-CP and a ng-eNB-CU-UP supplied by different manufacturers.

# 5	Functions of the E1 interface

## 5.1	General

The following clauses describe the functions supported over E1.

### 5.1.1	E1 interface management function

NOTE 1: for the functions below, the following equivalences can be applied to the following logical nodes:

- gNB-CU-UP = ng-eNB-CU-UP = eNB-UP

- gNB-CU-CP = ng-eNB-CU-CP = eNB*

The error indication function is used by the gNB-CU-UP or gNB-CU-CP to indicate to the gNB-CU-CP or gNB-CU-UP that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-CU-UP and the gNB-CU-CP.

The E1 setup function allows to exchange application level data needed for the gNB-CU-UP and gNB-CU-CP to interoperate correctly on the E1 interface. The E1 setup is initiated by both the gNB-CU-UP and gNB-CU-CP.

The gNB-CU-UP Configuration Update and gNB-CU-CP Configuration Update functions allow to update application level configuration data needed between the gNB-CU-CP and the gNB-CU-UP to interoperate correctly over the E1 interface.

The E1 setup and gNB-CU-UP Configuration Update functions allow to inform NR CGI(s), S-NSSAI(s), PLMN-ID(s), QoS information and NID(s) supported by the gNB-CU-UP.

The E1 setup and gNB-CU-UP Configuration Update functions allow the gNB-CU-UP to signal its capacity information to the gNB-CU-CP.

The E1 gNB-CU-UP Status Indication function allows to inform the overloaded or non-overloaded status over the E1 interface.

### 5.1.2	E1 bearer context management function

NOTE 1: for the functions below, the following equivalences can be applied to the following logical nodes:

- gNB-CU-UP = ng-eNB-CU-UP = eNB-UP

- gNB-CU-CP = ng-eNB-CU-CP = eNB*

The establishment of the E1 bearer context is initiated by the gNB-CU-CP and accepted or rejected by the gNB-CU-UP based on admission control criteria (e.g., resource not available).

The modification of the E1 bearer context can be initiated by either gNB-CU-CP or gNB-CU-UP. The receiving node can accept or reject the modification. The E1 bearer context management function also supports the release of the bearer context previously established in the gNB-CU-UP. The release of the bearer context is triggered by the gNB-CU-CP either directly or following a request received from the gNB-CU-UP.

This function is used to setup and modify the QoS-flow to DRB mapping configuration. The gNB-CU-CP decides flow-to-DRB mapping and provides the generated SDAP and PDCP configuration to the gNB-CU-UP. The gNB-CU-CP also decides the Reflective QoS flow to DRB mapping. The function is also used to send to the gNB-CU-UP the alternative QoS Parameters Sets when available for a QoS flow. For each PDU Session Resource to be setup or modified, the S-NSSAI, shall be provided in the E1 bearer context setup procedure and may be provided in the E1 bearer context modification procedure by gNB-CU-CP to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send the security information to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send to the gNB-CU-UP transport layer information to be used for data forwarding e.g. during handovers.

This function is used for the gNB-CU-CP to send the parameters for header compression for certain traffic types e.g. IP, Ethernet to the gNB-CU-UP.

This function is used for the gNB-CU-UP to notify the event of DL data arrival detection to the gNB-CU-CP. With this function, the gNB-CU-UP requests gNB-CU-CP to trigger paging procedure over F1 or Xn to support RRC Inactive state.

This function is used for the gNB-CU-UP to notify the gNB-CU-CP that an UL packet including a QFI value in the SDAP header not configured by the Flow Mapping Information IE is received for the first time at the default DRB. The gNB-CU-CP can take further action if needed.

This function is used for the gNB-CU-UP to notify the event of user inactivity to the gNB-CU-CP. With this function, the gNB-CU-UP indicates that the inactivity timer associated with a bearer, a PDU session or a UE expires, or that user data is received for the bearer, the PDU session or the UE whose inactivity timer has expired. The gNB-CU-CP consolidates all the serving gNB-CU-UPs for the UE and takes further action.

This function is used for the gNB-CU-UP to report data volume to the gNB-CU-CP.

This function is used for the gNB-CU-CP to notify the suspension and resumption of bearer contexts to the gNB-CU-UP.

This function also allows to support CA based packet duplication as described in TS 38.300 [6], i.e. one data radio bearer should be configured with at least two GTP-U tunnels between gNB-CU-UP and a gNB-DU.

This function is used to support the enhanced mobility operations as described in TS 38.300 [6] in the gNB-CU-UP.

### 5.1.3	Trace function

Editor’s note: it is FFS if this function can be applied to LTE CP-UP separation

The Trace function provides means to control trace sessions for a UE over E1 interface.

### 5.1.4	Load management function

Editor’s note: it is FFS if this function can be applied to LTE CP-UP separation

The load management function allows an gNB-CU-CP to request the reporting of load measurements to gNB-DU and is used by gNB-CU-UP to report the result of measurements admitted by gNB-CU-UP.

### 5.1.5	Measurement results transfer function

Editor’s note: it is FFS if this function can be applied to LTE CP-UP separation

The measurement results transfer is used by the gNB-CU-CP to transfer UE associated measurement results to the gNB-CU-UP.

### 5.1.6	Support for IAB

This function is used to update the DL/UL F1-U GTP-U tunnels for an IAB network.

## 5.2	TEIDs allocation

NOTE 1: for the functions below, the following equivalences can be applied to the following logical nodes:

- gNB-CU-UP = ng-eNB-CU-UP = eNB-UP

- gNB-CU-CP = ng-eNB-CU-CP = eNB*

The gNB-CU-UP is responsible for the allocation of the F1-U UL GTP TEID for each data radio bearer.

The gNB-CU-UP is responsible for the allocation of the S1-U DL GTP TEID for each E-RAB and the NG-U DL GTP TEID for each PDU Session.

The gNB-CU-UP is responsible for the allocation of the X2-U DL/UL GTP TEID or the Xn-U DL/UL GTP TEID for each data radio bearer.

# 6	Procedures of the E1 interface

## 6.1	Interface Management procedures

The E1 interface management procedures are listed below:

-	Reset procedure

-	Error Indication procedure

-	gNB-CU-UP E1 Setup procedure

-	gNB-CU-CP E1 Setup procedure

-	gNB-CU-UP Configuration Update procedure

-	gNB-CU-CP Configuration Update procedure

-	E1 Release procedure

-	gNB-CU-UP Status Indication procedure

## 6.2	Bearer Context Management procedures

The E1 bearer management procedures are listed below:

-	Bearer Context Setup procedure

-	Bearer Context Release Request (gNB-CU-UP initiated) procedure

-	Bearer Context Release (gNB-CU-CP initiated) procedure

-	Bearer Context Modification (gNB-CU-CP initiated) procedure

-	Bearer Context Modification Required (gNB-CU-UP initiated) procedure

Editor’s note: it is FFS if the procedure below can be applied to LTE CP-UP separation

-	DL Data Notification procedure

Editor’s note: it is FFS if the procedure below can be applied to LTE CP-UP separation

-	Bearer Context Inactivity Notification procedure

-	Data Usage Report procedure

-	MR-DC Data Usage Report procedure

## 6.3	UE Tracing procedures

Editor’s note: it is FFS if all these procedures can be applied to LTE CP-UP separation

The following procedures are used to trace the UE:

-	Trace Start procedure

-	Deactivate Trace procedure

-	Cell Traffic Trace procedure

## 6.4	Load management procedures

Editor’s note: it is FFS if all these procedures can be applied to LTE CP-UP separation

The load management procedures are listed as below:

-	Resource Status Reporting Initiation procedure

-	Resource Status Reporting procedure

## 6.5	Measurement results transfer procedures

Editor’s note: it is FFS if all these procedures can be applied to LTE CP-UP separation

The measurement results transfer procedures are listed as below:

-	GNB-CU-CP Measurement Results Information

## 6.6	IAB procedures

The IAB procedures are listed as below:

-	IAB UP TNL Address Update procedure

# 7	E1 interface protocol structure

Figure 7.1-1 shows the protocol structure for E1. The TNL is based on IP transport, comprising the SCTP on top of IP. The application layer signalling protocol is referred to as E1AP (E1 Application Protocol).

Figure 7.1-1: Interface protocol structure for E1

# 8	Other E1 interface specifications

This clause contains the description of the other related 3GPP specifications.

## 8.1	NG-RAN E1 interface: layer 1 (3GPP TS 38.461)

3GPP TS 38.461 [3] specifies the physical layer technologies that may be used to support the E1 interface.

## 8.2	NG-RAN E1 interface: signalling transport (3GPP TS 38.462)

3GPP TS 38.462 [4] specifies the signalling bearers for the E1AP for the E1 interface.

## 8.3	NG-RAN E1 interface: E1AP specification (3GPP TS 38.463)

3GPP TS 38.463 [5] specifies the E1AP protocol for radio network control plane signalling over the E1 interface.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>