3GPP TSG-RAN WG3 #112-e								R3-212763

Online, 17th – 27th May 2021

Title:	Summary of offline discussion on SN initiated SN Modification correction

Source:	Ericsson

Agenda Item:	9.3.8.1

Document for:	Approval

# Introduction

Partial modification is not allowed

CB: # 124_SNinitSNmod

- Check history

- spec text should clearly convey that partial modification is not allowed

(E/// - moderator)

Summary of offline disc R3-212763

# For the Chairman’s Notes

Propose the following two sets of CRs to be agreed:

Stage-3: Correction on SN initiated SN Modification procedure for EN-DC (Ericsson, NTT DOCOMO, INC., ZTE)

R3-212276 rev in R3-212851 – agreed

R3-212277 rev in R3-212852 – agreed

Stage-2: No partial success in the SN initiated SN Modification procedure for EN-DC (Ericsson, NTT DOCOMO, INC., ZTE, Nokia, Nokia Shanghai Bell, Huawei)

R3-212853 – endorsed

R3-212854 – endorsed

# Discussion

## Issue 1

Companies acknowledged that partial success/failure is not allowed during SgNB initiated SgNB Modification procedure, i.e., for EN-DC the SN initiated SN modification procedure shall be failed by sending SGNB MODIFICATION REFUSE message once any of the E-RABs is not successfully modified.

In the legacy discussion, firstly [1] was agreed to support PDCP duplication with CA, then [2] was accepted to clarify the usage of EN-DC Resource Configuration initiated by addition procedure. Afterwards the issue of partial success was observed and corrected by clarifying the condition of included E-RABs within the E-RABs Admitted To Be Modified List IE for the SN initiated SN modification procedure. However, the following paragraph was not removed due to its introduction in [2]:

For each E-RAB successfully modified or released as requested by the en-gNB, the MeNB shall inform the en-gNB, in the SGNB MODIFICATION CONFIRM message, the same value in the EN-DC Resource Configuration IE as received in the SGNB MODIFICATION REQUIRED message.

The above implies that some of the E-RABs can be successfully modified or released in the procedure, which is contradicting with RAN3’s conclusion. Does company agree to remove this paragraph and keep the conclusion of partial success not being allowed in the SgNB initiated SgNB Modification procedure?

| Company | Comment |
| --- | --- |
| E/// | Yes |
| NTTDOCOMO | Yes |
| Huawei | No, if the paragraph is removed, the requirement of using same value in the confirmed message is missing. |
| ZTE | Yes, I think it is the intention of this CR. |
| Nokia | Yes. The purpose of the paragraph was to make sure the EN-DC Resource Configuration IE is not changes for the sake of bearer type change. That purpose could be defined elsewhere, e.g. in stage-2 in more general way (for any procedure where it is used). |

## Issue 2

If company does not agree to remove the above paragraph in [4][5], is there any way to solve the contradiction? Please give your proposal below.

| Company | Comment |
| --- | --- |
| Huawei | Capture in chairman notes of the common understanding that partial success is not allowed. |
| ZTE | “Partial modification is not allowed” is already included in the chairman’s note. So, HW’s suggestion is feasible, but I do not think it is a good way. I try to provide a compromised suggestion, we have this CR and capture anther sentence “The receiving node is not allowed to change the values of EN-DC Resource Configuration IE.” in the Chairman’s note. |
| Nokia | Alternatively, it is added to the existing text that partial success if not allowed. |

# Conclusion, Recommendations

During email discussion, further comments were received on the corrections. Thus, two sets of CRs for stage-2 and stage-3 are provided.

- Stage-2 CRs to clarify that partial success is not allowed for the SN initiated SN modification procedure.

- Stage-3 CRs to remove the case of released E-RABs in the description since MN cannot reject the release request from SN.

# References

- R3-182939, Correction for PDCP Duplication, Ericsson

- R3-184267, Corrections on EN-DC Resource Configuration, Huawei

- R3-185222, CR on clarification of partial success on SgNB initiated SgNB Modification over X2, NTT DOCOMO, INC.

- R3-212276, Correction on SN initiated SN modification procedure for EN-DC, R15, Ericsson, NTT DOCOMO, INC.

- R3-212277, Correction on SN initiated SN modification procedure for EN-DC, R16, Ericsson, NTT DOCOMO, INC.