3GPP TSG-RAN WG3 Meeting #112-e	R3-212988

# E-meeting, 17-27 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0176 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of SCG activation and deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-06-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support SCG activation and deactivation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new sections about SCG activation and deactivation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support SCG activation and deactivation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x (new), 8.4.x.1 (new), 8.4.x.2 (new), 8.4.x.3 (new), 8.4.x.4 (new), 8.4.x.5 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.340 draftCR TS 38.423 CR 0633 TS 36.423 CR 1609 TS 38.473 CR 0777 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: capture TP R3-212832. |  |  |  |  |  |  |  |  |

8.4.3	SCG suspend/resume in RRC_INACTIVE

In the following, the procedure for SCG resume in RRC_INACTIVE is described.

Figure 8.4.3-1: SCG Suspend/Resume in RRC_INACTIVE

1. The CU of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to suspend the SCG of the UE before the UE enters into RRC_INACTIVE state from RRC_CONNECTED state.

2. The DU of SN sends the UE CONTEXT MODIFICATION RESPONSE to the CU of SN, and keeps all lower layer configuration for UEs without transmitting or receiving data from UE.

3~4: refer to TS 37.340 [12].

5. The CU of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to resume the SCG of the UE before the UE enters into RRC_CONNECTED state from RRC_INACTIVE state.

6. The DU of SN sends the UE CONTEXT MODIFICATION RESPONSE message to the CU of SN, and uses the previously stored lower layer configuration for the UE.

7~10: refer to TS 37.340 [12].

8.4.x	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

8.4.x.1	SN Addition with SCG Deactivation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and indicates that the SCG is deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU-CP sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU-CP, indicates the SCG deactivation.

6. The SN-CU-CP sends the SN addition Request Acknowledge towards the MN, indicates the SCG deactivation.

8.4.x.2	MN initiated SN Modification with SCG Deactivation

Figure 8.4.x.2-1: SCG Deactivation in MN initiated SN Modification procedure

1. SCG is activated.

2. The MN sends the SN Modification Request towards the SN, indicates the request of SCG deactivation.

3. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to indicate that the SCG is deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

5. The SN-CU-CP sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

7. The SN-CU-CP sends the SN Modification Request Acknowledge towards the MN.

8.4.x.3	MN initiated SN Modification with SCG Activation

Figure 8.4.x.3-1: SCG Activation in MN initiated SN Modification procedure

1. SCG is deactivated.

2. MN sends the SN Modification Request towards the SN, indicates the request of SCG activation.

3. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to indicate SCG is activated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

5. The SN-CU-CP sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

7. The SN-CU-CP sends the SN Modification Request Acknowledge towards the MN.

8.4.x.4	SN initiated SN Modification with SCG Deactivation

Figure 8.4.x.4-1: SCG Deactivation in SN initiated SN Modification procedure

1. SCG is activated.

2. The SN-CU-CP sends the SN Modification Require towards the MN, indicates the request of SCG deactivation.

3. The MN sends the SN Modification Confirm towards the SN-CU-CP.

4. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to indicate that the SCG is deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

5. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

6. The SN-CU-CP sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

7. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

Editor's note:	The order of the step 2, 3 and step 4,5,6,7 is FFS.

8.4.x.5	SN initiated SN Modification with SCG Activation

Figure 8.4.x.5-1: SCG Activation in SN initiated SN Modification procedure

1. SCG is deactivated.

2. The SN-CU-CP sends the SN Modification Require towards the MN, indicates the request of SCG activation.

3. The MN sends the SN Modification Confirm towards the SN-CU-CP.

4. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to indicate that the SCG is activated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

5. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

6. The SN-CU-CP sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

7. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

Editor's note:	The order of the step 2, 3 and step 4,5,6,7 is FFS