3GPP TSG-RAN WG3 Meeting #112-e                                                                            R3-212946

17-27 May 2021, E-meeting

Title:	[Draft] LS to RAN2 on reduction of service interruption during intra-donor IAB-node migration

Response to:

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:

- Name:			Milap Majmundar

- E-mail Address:	milap.majmundar@att.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN3 is currently evaluating the following two solutions for transfer of RRCReconfiguration for descendent IAB node over source path for reduction of service interruption during intra-donor IAB-node migration.

Solution 1:

The RRCReconfiguration message for the child IAB-MT is buffered in the parent IAB-DU, and it is sent only when a pre-requisite step is satisfied/performed. The IAB-donor-CU may indicate to the parent IAB-DU to buffer the RRCReconfiguration destined for its child IAB-MT. The buffered RRCReconfiguration may be released by the parent IAB-DU upon satisfaction of a condition. For example, in the case of the migrating IAB-node the RRCReconfiguration may be released upon successful IAB-node migration; and in the case of a descendent IAB-node the RRCReconfiguration buffered at the IAB-DU may be release upon reception of RRCReconfiguration by corresponding IAB-MT from its parent IAB-DU.

While exact details of Solution 1 are still FFS, an example procedure for Solution 1 is provided in Figure 1 below to help RAN2 better understand the general solution. Note that this is subject to change.

Figure 1. Example procedure for Solution 1 (R3-211740)

Solution 2:

The RRCReconfiguration message for the child IAB-MT is buffered in the child IAB-MT, and it is executed only when a pre-requisite step is satisfied/performed. The IAB-donor-CU may indicate to the child IAB-MT to buffer its own RRCReconfiguration destined and delay its execution. The buffered RRCReconfiguration may be executed by the child IAB-MT upon satisfaction of a condition. For example, in the case of the migrating IAB-node the migrating IAB-node may send an indication to its child IAB-MT about successful IAB-node migration; and a descendent IAB-node may send an indication to its child IAB-MT upon reception of such indication from its parent IAB-node.

While exact details of Solution 2 are still FFS, an example procedure for Solution 2 is provided in Figure 2 below to help RAN2 better understand the general solution. Note that this is subject to change.

Figure 2. Example procedure for Solution 2 (R3-211740)

RAN2 is requested to provide feedback regarding the following question.

Q1: Please provide feedback regarding issues and feasibility of Solutions 1 and 2.

.

# 2	Actions

To: RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide feedback on Solutions 1 and 2.

# 3	Dates of next TSG RAN3 meetings

RAN3#113e 	Aug. 16~27, 2021	online

RAN3#114e	Nov. 1~12, 2021	online