3GPP TSG-RAN3 Meeting #112-e	R3-211446

Electronic meeting, 17 - 27 May 2021

SA WG2 Meeting #143e	S2-2101076

# Feb 24th – March 9th, 2021 ; Elbonia             				(revision of S2-210xxxx)

# Title:	Reply LS on clarification request for eNPN features

# Response to:	S2-2100143/ R2-2102489

# Release:	Rel-17

# Work Item:	eNPN, NG-RAN_PRN_enh-Core

Source:	SA2

To:	RAN2

Cc:	RAN3, CT1, SA1

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	Devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none.

1. Overall Description:

SA2 thanks RAN2 for their LS on Enhancement for Private Network Support for NG-RAN.

SA2 would like to offer the following answers for the questions (Q1, Q2, Q3) raised by RAN2:

Questions related to SNPNs with subscription or credentials by a separate entity:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

[SA2 answer] Yes, These parameters should be set uniformly per SNPN.

Question 2: Shall Group IDs be broadcasted per SNPN or per cell?

[SA2 answer] Yes, It is assumed that that the Group IDs will be broadcast per SNPN.

Question related to support UE onboarding and provisioning for NPN:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?)

[SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection.

Even if there is no uniform support and a UE moves to a cell in an O-SNPN not supporting onboarding, SA2 foresees no impact to mobility procedures as remote provisioning can continue in the target cell. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

2. Actions:

To RAN2.

ACTION: 	SA2 kindly asks RAN2 to take the above answers into account for their work.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #144E   		12th April to 16th April 2021

TSG-SA2 Meeting #145E   		17th May to 28th May 2021