3GPP TSG-RAN WG3 #113-e																		   R3-213225

16-26 Aug 2021

Online

Title: 	Discussion on inter-MN handover without SN change

Source: 	ZTE

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

After RAN2 #114 e-meeting, an LS (R2-2106682 “LS on inter-MN handover without SN change”) is sent to RAN3.

1. Overall Description: For the inter-MN handover without SN change scenario, RAN2 discussed how MN can indicate whether SN should apply delta signalling or full configuration. For one of the options discussed, the signalling of the fields is as shown by the example below: - Inter-MN HO without SN change (implies SN may provide delta configuration) - SN UE X2/XnAP ID present - sourceConfigSCG not present - scg-RB-Config not present - Inter-MN HO without SN change (implies SN must apply full configuration) - SN UE X2/XnAP ID not present - sourceConfigSCG not present - scg-RB-Config not present RAN2 would like to ask RAN3 the following questions: Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN? Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration? 2. Actions: To RAN3 group. ACTION: RAN2 respectfully asks RAN3 to answer the above questions.

In this paper, we discuss the issue raised in the LSin, then further provide the reply LS.

- Discussion

Before we discuss the issue and answer the question, please see the following content in the TS37.340.

## 10.7 Inter-Master Node handover with/without Secondary Node change ### 10.7.1 EN-DC <Skip unrelated part> 2. If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SgNB Addition Request to the target SN including the UE context in the source SN that was established by the source MN. ### 10.7.2 MR-DC with 5GC <Skip unrelated part> 2. If the target MN decides to keep the source SN, the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN.

According to the TS37.340, it is true that in case of Inter-MN handover without SN change, the target MN shall send SN UE X2/XnAP ID to the SN within the SN addition request message.

What meaning of “If the target MN decides to keep the SN”?

- The SN is not changed, but the SN is only allowed to do full config.

- The SN is not changed, and the SN is allowed to choose either delta config or full config.

For the case 1, e.g., when the MN decides to do MN full configuration by itself, the SN has to do SN full configuration accordingly. In this case, the SN does not need to know whether the SN is changed or not.

For the case 2, only in this case, the SN needs to know whether the SN is changed or not, because the SN needs to acquire old configuration.

Observation 1: In case of Inter-MN handover with/without SN change, if the MN decides to do MN full configuration, it is useless for the SN to know whether the SN is changed or not.

Observation 2: In case of Inter-MN handover with/without SN change, if the MN decides to do MN full configuration, but it sends SN UE X2/XnAP ID to SN, the SN’s behaviour is not correct.

Proposal 1: In case of Inter-MN handover with/without SN change, if the MN allowed the SN to choose full/delta configuration, the MN sends SN UE X2/XnAP ID to SN and then SN decides to do either full or delta configuration.

If the above observations and proposal are correct, it seems better to clarify the corresponding description in current TS37.340, as below.

## 10.7 Inter-Master Node handover with/without Secondary Node change ### 10.7.1 EN-DC <Skip unrelated part> 2. If the target MN decides to keep the SN (i.e., the (target)SN is allowed to choose full/delta configuration.), the target MN sends SN Addition Request to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SgNB Addition Request to the target SN including the UE context in the source SN that was established by the source MN. ### 10.7.2 MR-DC with 5GC <Skip unrelated part> 2. If the target MN decides to keep the source SN (i.e., the (target)SN is allowed to choose full/delta configuration.), the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN.

So that, the following questions within the LS can be answered.

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?

Answer: No. In the inter-MN handover without SN change scenario, the SN UE X2/XnAP ID is not always required to be present when target MN sends SN Addition Request to SN, e.g., the MN decides to do MN full configuration by itself.

Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

Answer: Yes/but. Upon reception of SN UE X2/XnAP ID included in the SN addition request message, the target SN knows that the SN is not changed and it is allowed to choose full/delta configuration, it can further decide to provide either delta configuration or full configuration by sending “RRC config indication” IE included in the SN addition request acknowledge message to the Target MN.

3. Conclusion

After the above analysis, we provide the following proposal.

Observation 1: In case of Inter-MN handover with/without SN change, if the MN decides to do MN full configuration, it is useless for the SN to know whether the SN is changed or not.

Observation 2: In case of Inter-MN handover with/without SN change, if the MN decides to do MN full configuration, but it sends SN UE X2/XnAP ID to SN, the SN’s behaviour is not correct.

Proposal 1: In case of Inter-MN handover with/without SN change, if the MN allowed the SN to choose full/delta configuration, the MN sends SN UE X2/XnAP ID to SN and then SN decides to do either full or delta configuration.

We provide a draft reply LS and corresponding CRs.

4. Reference

[1] R2-2106682 LS on inter-MN handover without SN change, RAN2

[2] R3-213226 [DRAFT] Reply LS on inter-MN handover without SN change, ZTE

[3] R3-213227 Clarification on inter-MN handover without SN change in Rel-15, ZTE

[4] R3-213228 Clarification on inter-MN handover without SN change in Rel-16, ZTE