3GPP TSG-RAN WG3 #113-e								R3-213247

16-26 August 2021

Online

Source:               ZTE Corporation

Title:                    Discussion on I-RNTI partitioning

Agenda item:      31.2.1

Document for:    Discussion and Decision

- Background

In the previous meeting, we have the following progress.

Previous in R3-206827, R3-206821 (noted) Previous summary of offline disc. in R3-206967 (noted) A standardized solution enabling an inter vendor interoperable way for an NG RAN node to deduce the identity of another NG RAN node from the received I-RNTI is needed Agree on the benefits of a solution that allows at least some flexibility in the selection of the Local Node ID length; further details FFS Previous summary of offline disc. in R3-211131 (noted) The description in the informative Annex C of TS38.300 is not sufficient, and a fully standardized solution to minimize OAM configuration needs to be produced by RAN3 The solution shall support flexible assignment of the maximum number of Inactive UE contexts per NG-RAN node The maximum number of Inactive UE Contexts may differ between NG-RAN nodes, and it may be changed after node deployment in a semi-static manner. Previous summary of offline disc. in R3-212633 (noted) WA: a solution based on exchanges of Local gNB-ID over Xn should be pursued; Xn signaling impact should be limited Continue discussion on Xn-based solutions; other solutions (e.g. additional bits in I-RNTI) are not precluded Solutions based on OAM configuration may be considered if they fulfil requirements of: - flexibility in allocation of maximum number of Inactive UE contexts per NG-RAN node - Interoperability between vendors - support for RAN sharing - Minimized configuration effort To be continued...

In this paper, we continue to discuss this issue and give our proposals.

- Discussion

In order to support flexible assignment of the maximum number of Inactive UE contexts per NG-RAN node, the following four solutions are provided to support flexible I-RNTI partitioning:

Solution 1: Multiple Local gNB Identifiers per NG-RAN node, and Local gNB Identifiers are exchanged between gNBs[1]

In this solution, the length of local gNB identifier is fixed to a large value enough to avoid local gNB identity conflict, and the flexible number of UE identifiers per gNB is realized by flexible number of local gNB identifiers per gNB. E.g. for the I-RNTI of 40 bits, if the length of local gNB identifier is fixed to 30 bits, then 10 bits can be used to identify the UE context(at most 1024 UEs).

In this solution, the local gNB identifier should be exchanged between gNBs. If more than 1024 UEs in RRC_INACTIVE state exists in the gNB, multiple local gNB identifiers should be configured for the gNB. That means: the exchanged local gNB identifier between gNBs is a local gNB identifiers list, and each element of the list has 30 bits, the signalling consumption between gNBs will be large.

Furthermore, with the number of UEs in RRC_INACTIVE state changes in the gNB, the local gNB identifiers for the gNB may also change, which may trigger the local gNB identifier update frequently and cause large amount of Xn signalling.

Observation 1: Solution 1(e.g. Multiple Local gNB Identifiers per NG-RAN node) may lead huge Xn signalling consumption.

In RAN3#112-e meeting, there is a WA that Xn signaling impact should be limited.

Solution 2: One Local gNB Identifier per NG-RAN node, Local gNB Identifiers are exchanged between gNBs and the length of NG-RAN Node ID part in I-RNTI is self-contained in the I-RNTI[1][2][3].

In this solution, the length of local gNB identifier and the length of UE context identifier are flexible, a fixed number of bits in I-RNTI is assigned to indicate the length of (short) NG-RAN Node ID, e.g., calling it I-RNTI structure indication. When a gNB receives the resume request, it will firstly resolve these bits to acquire the length of (short) NG-RAN Node ID. Based on this exact length of (short) NG-RAN Node ID, the gNB can determine the true structure of this I-RNTI and uniquely determine (short) NG-RAN Node ID.

Such self-contained scheme has impact on the original structure/length of I-RNTI that will result in a reduction of the number of NG-RAN Node ID and UE context ID that can be indicated. In order to alleviate such impact, it’ll be better to assign as less bits as possible, e.g., 2 bits as full I-RNTI structure indication, and 1bit as short I-RNTI structure indication. That means at most 4 of I-RNTI structures can be indicated for full I-RNTI structure indication, and at most 2 of I-RNTI structures can be indicated for short I-RNTI structure. The specification can assign 4 most likely lengths that have a reasonable distribution for full I-RNTI structure indication and 2 most likely lengths that have a reasonable distribution for short I-RNTI structure indication. Even in the future a vendor may have some different thinking about the length, they can still find the closest value to use. An example coding for such 2-bits I-RNTI structure bits might be as following:

Table 1

| Coding of 2bits for full I-RNTI structure indication | The indicated length of NG-RAN node ID | The indicated length of UE context ID |
| --- | --- | --- |
| 00 | 12 bits | 26 bits |
| 01 | 15 bits | 23 bits |
| 10 | 18 bits | 20 bits |
| 11 | 21 bits | 17 bits |
| Note | The indicated length = 12+3*value of the 2bits | The indicated length = 40-2 -(12+3*value of the 2bits) |

The above example is for fullI-RNTI structure. For shortI-RNTI, it can have further less bits for I-RNTI structure indication, e.g., 1 bits.

With this solution, the local gNB identifier should also be exchanged between gNBs. But since once gNB has only one local gNB identifier, the local gNB identifier need not be updated frequently.

Observation 2: Solution 2 has limited X2 signalling impacts.

Solution 3: The NG-RAN Node ID uses a pointer of gNB identity(e.g. local NG-RAN node ID is derived based on the global NG-RAN node ID mod N) and the length of NG-RAN Node ID part in I-RNTI is self-contained in the I-RNTI[3][7].

This solution is similar with solution 2, the length of local gNB identifier and the length of UE context identifier are flexible, a fixed number of bits in I-RNTI is assigned to indicate the length of (short) NG-RAN Node ID, e.g., calling it I-RNTI structure indication. When a gNB receives the resume request, it will firstly resolve these bits to acquire the length of (short) NG-RAN Node ID. Based on this exact length of (short) NG-RAN Node ID, the gNB can determine the true structure of this I-RNTI and uniquely determine (short) NG-RAN Node ID.

But with this solution, the local NG-RAN node ID is derived based on the global NG-RAN node ID mod N, and need not be exchanged between gNBs. That means the value of some bits in NG-RAN node ID will be used to identify the gNB in I-RNTI. Thus, which bits in NG-RAN node ID will be used to identify the gNB in I-RNTI should be based on OAM, and the NG-RAN node ID coding rule may be limited, because the NG-RAN node ID should be unique in the whole network, and different operator may usually have different NG-RAN node ID coding rule.

Furthermore, with this solution, the length value of Local NG-RAN node ID is not specified and depends on OAM, which may lead Local NG-RAN node ID conflict between different operators and should be avoided.

Observation 3: Solution 3 still involves multiple OAM, and may lead Local NG-RAN node ID conflict between different operators.

Solution 4: NG-RAN node can maintain at least two local NG-RAN node identifiers for some time to support the Local NG-RAN node ID change [4].

This solution is an optimization of solution 2. e.g. when the local NG-RAN node identifier changes, but there is still some RRC_INACTIVE UE uses the old local NG-RAN node identifier, the gNB can allocate a new local NG-RAN node identifier for new RRC_INACTIVE UE, and delete the old local NG-RAN node identifier until there is not UEs using it.

With this solution, the local NG-RAN node identifier can be smoothly changed without affect the RRC_INACTIVE UE behaviour in the gNB. But this solution depends on whether or how frequently the local NG-RAN node identifier will be changed. Since this solution is an optimization of solution 2, it can be discussed until there is some conclusion on solution 2.

Observation 4: Solution 4 is an optimization of solution 2.

Proposal 1: Solution 2 is used for the flexible I-RNTI partitioning.

Proposal 1a: Exchanging on the local node identifier via Xn.

Proposal 1b: The length of local node identifier is self-contained in the I-RNTI.

Proposal 1c: The length value of Local NG-RAN node ID is specified.

Proposal 2: Whether solution 2 is necessary is FFS.

Based on the proposal and [5] [6], we provide the draft 38.423 CR[7].

- Conclusions

In this contribution, we make the following proposals:

Observation 1: Solution 1(e.g. Multiple Local gNB Identifiers per NG-RAN node) may lead huge Xn signalling consumption.

Observation 2: Solution 2 has limited X2 signalling impacts.

Observation 3: Solution 3 still involves multiple OAM, and may lead Local NG-RAN node ID conflict between different operators.

Observation 4: Solution 4 is an optimization of solution 2.

Proposal 1: Solution 2 is used for the flexible I-RNTI partitioning.

Proposal 1a: Exchanging on the local node identifier via Xn.

Proposal 1b: The length of local node identifier is self-contained in the I-RNTI.

Proposal 1c: The length value of Local NG-RAN node ID is specified.

Proposal 2: Whether solution 2 is necessary is FFS.

Based on the proposal and [5] [6], we provide the draft 38.423 CR[7].

- References

- R3-212310 Node Identifier for RRC Inactive, RAN3#112-e, Ericsson, Bell Mobility

- R3-211586 Discussion on I-RNTI partitioning, RAN3#112-e, ZTE Corporation

- R3-211665 Support of Local NG-RAN Node Identifier, RAN3#112-e, Nokia, Nokia Shanghai Bell

- R3-212014 NG-RAN node ID resolution from I-RNTI, RAN3#112-e, Huawei

- R3-211587	CR38423 for addition of I-RNTI structure indication, RAN3#112-e, ZTE Corporation, CR0528r1, TS 38.423 v16.5.0, Rel-17, Cat. B

- R3-210736 Addition of Local NG-RAN Node Identifier to resolve NG-RAN ID from I-RNTI, RAN3#111-e, Ericsson, AT&T, Verizon Wireless, Vodafone, CR0333r7, TS 38.423 v16.4.0, Rel-17, Cat. B

- R3-213248 CR for TS 38.423 on I-RNTI partitioning