3GPP TSG-RAN WG3 Meeting #113-e	R3-213261

# 16 – 26 August 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1811 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Detection of RACS support at target during N2/S1 handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-RAN |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 has requested RAN3 to support detection (at N2/S1 handover) by the source RAN node of whether the target RAN node does not support RACS. For X2 handover, this is accomplished via criticality and criticality handling, but this approach is not possible with N2/S1 handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new IE is added to the Source eNB to Target eNB Transparent Container IE and also the Target eNB to Source eNB Transparent Container IE. The new IEs are used to perform a handshake at handover, which is used to establish whether or not the source needs to include capability container(s) in future handover signalling. Impact analysis: This CR has isolated impact towards the previous version of the specification (same release). The impact can be considered isolated because it only provides a detection mechanism for the UE Radio Capability ID feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The source eNB cannot determine whether it is safe to omit the capabilities from handover signalling (resulting in either excess message size or handover failure). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2, 8.4.2.2, 9.2.1.7, 9.2.1.8, 9.2.1.XX, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR#0572 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the EPC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source eNB initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving MME. When the source eNB sends the HANDOVER REQUIRED message, it shall start the timer TS1RELOCprep. The source eNB shall indicate the appropriate cause value for the handover in the Cause IE.

The source eNB shall include the Source to Target Transparent Container IE in the HANDOVER REQUIRED message.

*** skip unchanged text in same section  ***

If the DAPS Request Information IE is included for an E-RAB in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUIRED message, it indicates that the request concerns a DAPS Handover for that E-RAB, as described in TS 36.300 [14].

If the Source-to-Target Feature Use Proposed IE is included in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUIRED message, it indicates that the source eNB proposes use of the indicated features in this procedure and future instances of the same procedure.

Interactions with E-RAB Management procedures:

>>>> NEXT CHANGE <<<<

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target eNB for the handover of a UE.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The MME initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. The HANDOVER REQUEST message may contain the Handover Restriction List IE, which contains roaming or access restrictions.

*** skip unchanged text in same section  ***

If the DAPS Request Information IE is included for an E-RAB in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUEST message, the target eNB shall consider that the request concerns a DAPS Handover for that E-RAB, as described in TS 36.300 [14]. The target eNB shall include the DAPS Response information List IE in the Target eNB to Source eNB Transparent Container IE within the HANDOVER REQUEST ACKNOWLEDGE message, containing the DAPS Response Information IE for each E-RAB requested to be configured with DAPS Handover.

If the IMS voice EPS fallback from 5G IE is included in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUEST message, the target eNB shall, if supported, store the information in the UE context and consider that the UE is handed over from NG-RAN to E-UTRAN due to an IMS voice fallback.

If the Source-to-Target Feature Use Proposed IE is included in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUEST message, the target eNB shall consider that the source eNB proposes that the target eNB shall use the indicated features in this procedure and future instances of the same procedure. The target eNB may set the corresponding feature indicators in the Target-to-Source Feature Use Accepted IE in the Target eNB to Source eNB Transparent Container IE within the HANDOVER REQUEST ACKNOWLEDGE message, to indicate that it is able to use the indicated features in this procedure and future instances of the same procedure.

>>>> NEXT CHANGE <<<<

#### 9.2.1.7	Source eNB to Target eNB Transparent Container

The Source eNB to target eNB Transparent Container IE is an information element that is produced by the source eNB and is transmitted to the target eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the external handover source to the target eNB.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC Handover Preparation Information message as defined in subclause 10.2.2 of TS 36.331 [16]. | - |  |
| E-RABs Information List |  | 0..1 |  |  | - |  |
| >E-RABs Information Item |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Forwarding | O |  | 9.2.3.14 |  | - |  |
| >>DAPS Request Information | O |  | 9.2.1.155 |  | YES | ignore |
| Target Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | - |  |
| UE History Information | M |  | 9.2.1.42 |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [16]) | YES | ignore |
| IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| UE Context Reference at Source | O |  | 9.2.1.144 |  | YES | ignore |
| Inter-system measurement Configuration | O |  | 9.2.1.151 |  | YES | ignore |
| Source Node ID | O |  | 9.2.1.152 |  | YES | ignore |
| Emergency Indicator | O |  | ENUMERATED (true, …) | Indicates an emergency EPS voice fallback | YES | ignore |
| Source-To-Target Feature Use Proposed | O |  | Feature List 9.2.1.XX | Indicates features whose use is proposed by the source. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.2.1.8	Target eNB to Source eNB Transparent Container

The Target eNB to Source eNB Transparent Container IE is an information element that is produced by the target eNB and is transmitted to the source eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the target eNB to the external relocation source.

This IE is transparent to EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 of TS 36.331 [16]. | - |  |
| DAPS Response Information List |  | 0..1 |  |  | YES | reject |
| >DAPS Response Information Item |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DAPS Response Information | M |  | 9.2.1.156 | Indicates the response to a requested DAPS Handover | - |  |
| Target-to-Source Feature Use Accepted | O |  | Feature List 9.2.1.XX | Indicates features whose use is accepted at target. | YES | ignore |

>>>> NEXT CHANGE <<<<

#### 9.2.1.XX	Feature List

This IE includes a list of features to be coordinated during handover preparation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Feature List | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a feature: first bit = RACS: other bits reserved for future use. Value '1' indicates 'use is proposed (by source node) or accepted’ (by target node). |

>>>> NEXT CHANGE <<<<

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RABInformationListItem,

id-E-RABItem,

id-GUMMEIType,

id-Bearers-SubjectToStatusTransfer-Item,

id-Time-Synchronisation-Info,

id-x2TNLConfigurationInfo,

id-eNBX2ExtendedTransportLayerAddresses,

id-MDTConfiguration,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-SignallingBasedMDTPLMNList,

id-MobilityInformation,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-eNBIndirectX2TransportLayerAddresses,

id-Muting-Availability-Indication,

id-Muting-Pattern-Information,

id-NRrestrictioninEPSasSecondaryRAT,

id-NRrestrictionin5GS,

id-Synchronisation-Information,

id-uE-HistoryInformationFromTheUE,

id-LoggedMBSFNMDT,

id-SON-Information-Report,

id-RecommendedCellItem,

id-RecommendedENBItem,

id-ProSeUEtoNetworkRelaying,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-M6Configuration,

id-M7Configuration,

id-RAT-Type,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDL,

id-extended-uEaggregateMaximumBitRateUL,

id-SecondaryRATDataUsageReportItem,

id-E-RABUsageReportItem,

id-UEAppLayerMeasConfig,

id-serviceType,

id-UnlicensedSpectrumRestriction,

id-CNTypeRestrictions,

id-DownlinkPacketLossRate,

id-UplinkPacketLossRate,

id-BluetoothMeasurementConfiguration,

id-WLANMeasurementConfiguration,

id-LastNG-RANPLMNIdentity,

id-PSCellInformation,

id-IMSvoiceEPSfallbackfrom5G,

id-RequestTypeAdditionalInfo,

id-AdditionalRRMPriorityIndex,

id-ContextatSource,

id-IntersystemMeasurementConfiguration,

id-SourceNodeID,

id-NB-IoT-RLF-Report-Container,

id-MDTConfigurationNR,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DAPSResponseInfoItem,

id-Bearers-SubjectToEarlyStatusTransfer-Item,

id-TraceCollectionEntityURI,

id-EmergencyIndicator,

id-Source-To-Target-FeatureUseProposed,

id-Target-to-Source-FeatureUseAccepted,

maxnoofCSGs,

maxnoofE-RABs,

maxnoofErrors,

*** skip unchanged text in same section  ***

#### -- F

FeatureList ::= BIT STRING (SIZE (16))

FiveGSTAC ::= OCTET STRING (SIZE (3))

FiveGSTAI ::= SEQUENCE {

pLMNidentity			PLMNidentity,

fiveGSTAC				FiveGSTAC,

iE-Extensions			ProtocolExtensionContainer { {FiveGSTAI-ExtIEs} } OPTIONAL,

...

}

*** skip unchanged text in same section  ***

SourceeNB-ToTargeteNB-TransparentContainer		::= SEQUENCE {

rRC-Container				RRC-Container,

e-RABInformationList		E-RABInformationList			OPTIONAL,

targetCell-ID				EUTRAN-CGI,

subscriberProfileIDforRFP	SubscriberProfileIDforRFP		OPTIONAL,

uE-HistoryInformation		UE-HistoryInformation,

iE-Extensions				ProtocolExtensionContainer { {SourceeNB-ToTargeteNB-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

SourceeNB-ToTargeteNB-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ID id-MobilityInformation					CRITICALITY ignore	EXTENSION MobilityInformation						PRESENCE optional}|

{ID id-uE-HistoryInformationFromTheUE		CRITICALITY ignore	EXTENSION UE-HistoryInformationFromTheUE		PRESENCE optional}|

{ID id-IMSvoiceEPSfallbackfrom5G			CRITICALITY ignore	EXTENSION IMSvoiceEPSfallbackfrom5G				PRESENCE optional}|

{ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex				PRESENCE optional}|

{ID id-ContextatSource						CRITICALITY ignore	EXTENSION ContextatSource							PRESENCE optional}|

{ID id-IntersystemMeasurementConfiguration	CRITICALITY ignore	EXTENSION IntersystemMeasurementConfiguration	PRESENCE optional}|

{ID id-SourceNodeID							CRITICALITY ignore	EXTENSION SourceNodeID										PRESENCE optional}|

{ID id-EmergencyIndicator					CRITICALITY ignore	EXTENSION EmergencyIndicator								PRESENCE optional}|

{ID id-Source-To-Target-FeatureUseProposed	CRITICALITY ignore	EXTENSION FeatureList								PRESENCE optional	},

...

}

SourceNgRanNode-ID ::= SEQUENCE {

global-RAN-NODE-ID		Global-RAN-NODE-ID,

selected-TAI		FiveGSTAI,

iE-Extensions		ProtocolExtensionContainer { { SourceNgRanNode-ID-ExtIEs} } OPTIONAL,

...

}

*** skip unchanged text in same section  ***

GNB-ID ::= BIT STRING (SIZE(22..32))

TargeteNB-ToSourceeNB-TransparentContainer		::= SEQUENCE {

rRC-Container		RRC-Container,

iE-Extensions		ProtocolExtensionContainer { {TargeteNB-ToSourceeNB-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargeteNB-ToSourceeNB-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSResponseInfoList						CRITICALITY reject	EXTENSION DAPSResponseInfoList	PRESENCE optional}|

{ ID id-Target-to-Source-FeatureUseAccepted			CRITICALITY ignore	EXTENSION FeatureList	PRESENCE optional	},

...

}

Target-ToSource-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the target RAN node to the source RAN node.

-- The octets of the OCTET STRING are coded according to the specifications of the target system.

>>>> NEXT CHANGE <<<<

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** skip unchanged text in same section  ***

id-NotifySourceeNB									ProtocolIE-ID ::= 320

id-eNB-EarlyStatusTransfer-TransparentContainer		ProtocolIE-ID ::= 321

id-Bearers-SubjectToEarlyStatusTransfer-Item		ProtocolIE-ID ::= 322

id-WUS-Assistance-Information						ProtocolIE-ID ::= 323

id-NB-IoT-PagingDRX									ProtocolIE-ID ::= 324

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 325

id-EmergencyIndicator								ProtocolIE-ID ::= 326

id-Source-To-Target-FeatureUseProposed				ProtocolIE-ID ::= XXX

id-Target-to-Source-FeatureUseAccepted				ProtocolIE-ID ::= YYY

END

>>>> END OF CHANGES <<<<