3GPP TSG-RAN WG3 Meeting #113-e	R3-213468

# Electronic meeting, 16th – 26th Aug 2021

Agenda item:		18.2

Source:	Intel Corporation

Title:	High level principle and Functional Framework of AI/ML enabled NG-RAN Network

Document for:		Discussion and Decision

# Introduction

During RAN3 #112e meeting, following open issues are captured for functional framework:

Open issues: - Mark the Model Performance Feedback in the Functional Framework (Figure 4.2-1) as FFS and continue discussions on what such transfer of information should entail and for which purpose. - The definition of the Model Deployment/Update function is FFS. Discussions need to be continued to identify what information will “Model Deployment/Update” transfer, whether this information will need to be standardised and, if not, what are the assumptions on this information

And following open issues are captured for high level principle:

- Discussions should be continued on the following principle, especially concerning what the level of accuracy is: If the inference function provides output predictions, an optional indication of the accuracy level for which the inference model is trained should be indicated to the nodes that request/subscribe to this information. - output from one model as input to another - high-level principles for inference function

In this contribution, we further discuss the open issues of functional framework and high-level principles. Besides, we also discuss the uncertainty of the outputs from AI/ML model and propose a solution for network to select the most appropriate UE(s) to use AI/ML model for performance improvement and decision making.

# Discussion

## High-Level Principle

### Information Exchanging between AI/ML models

In RAN3 #112e meeting, it is FFS whether output of one AI/ML model can be used for another AI/ML model. We consider two scenarios to discuss this issue.

Scenario 1

Based on implementation, the AI/ML model can directly take the historical information for training, e.g. historical resource utilization/traffic load information/event, historical UE location information to train a single AI/ML model for mobility optimization. The handover target cell may be the output of mobility optimization model. In this case, such output is not needed for other AI/ML models.

- AI/ML model may take historical information to train the model directly without requesting output from other AI/ML models.

Scenario 2

AI/ML model can be very complex considering the scenario and multiple agents/factors, especially in the wireless communication system. For those use cases which have multiple impacted agents and require large number of input vectors, it is normally hard to get a convergence model. For example, AI/ML model used for mobility optimization can be very complicated, as handover decision and optimization can be impacted by traffic load, resource status, radio measurements, UE trajectory, etc. In this case, it would be better to split a complex model into multiple AI/ML models, each one is responsible for different prediction functionality.

- A complicated AI/ML model can be split into multiple AI/ML models, where one AI/ML model output can be used by another AI/ML model.

It is also possible that some AI/ML model(s) can be used by multiple use cases. For example, traffic load prediction model can be used for load balancing, network energy saving, and mobility optimization. Hence, predicted traffic load can be directly used by AI/ML models trained for network energy saving, or for mobility optimization.

- A common AI/ML model can be used by multiple use cases, where the predicted results from the common AI/ML model can be used as inputs by multiple AI/ML models with different purposes.

Another example is that the predicted information is transmitted between different network nodes. One network node and its AI/ML model(s) can use the prediction results generated by AI/ML model(s) in another network node. For example, in our companion contribution [3], predicted resource status and predicted traffic load information and resource status are transmitted between NG-RAN nodes for load balancing use cases. The predicted results can further help the source node to make a more precise decision by understanding the future status of the target node. From system level, this can also help to avoid interrupting or downgrading existing traffic and service in the target node.

- The source gNB can use the outputs of AI/ML model from another gNB for prediction and decision making.

As defined in TR 37.817 [1], “Data Collection is a function that provides input data to Model training and Model inference functions”. The output of AI/ML model may be requested by multiple network entities. To avoid duplication and signaling exchanges across multiple network entities, we further propose that the output of AI/ML model should also following the principle, i.e. “only be transmitted per request”, same as other collected data. The output of AI/ML model should be collected back to “Data Collection”, so that other network nodes can request such data for model training and model inference following data registration and request procedure. To support this, “Model Inference” should transmit the output and the corresponding state when generating the output to “Data Collection”.

- “Model Inference” should transmit the inference output(s) and the corresponding state when generating the output to “Data Collection” function.

- AI/ML model should be able to use output of another AI/ML model by requesting the data from “Data Collection” function.

### UE Selection

It is well known that AI/ML training and inference can lead to heavy computation complexity and high time consumption. It is not always possible that AI/ML model can predict each UE behaviour simultaneously or generate the corresponding prediction results at the same time. This is because, the more users using AI/ML model in the network, the more complex AI/ML model can be, the longer latency system/UE will experience. This is not desirable to the end-users, as they may experience service degradation even after using AI/ML model. In this case, it is important to limit the number of UE using AI/ML model for inference, as normally system requires a real-time decision from Model Inference function. UE selection based on certain rules to maximize system performance is necessary.

Moreover, as discussed in section 2.2.1, prediction results may not be accurate, it is important not to use AI/ML model to those critical scenarios, for example, to the UE which has an extreme high reliability requirement.

- The network needs to select the most appropriate UEs to use AI/ML model for prediction, as it can help to reduce system computation complexity and reduce latency during performing model inference. Inaccurate decision can also be avoided.

The network may consider selecting the most appropriate UEs for ML actions based on QoS requirement. For example, UEs that only have non-GBR bearers/flows may be preferred for selection, as these are less likely to have important traffic such as voice. Additionally, even if a UE has GBR or even delay critical GBR flows/bearers, it may be selected if it is currently not sending/receiving traffic.

- Network should be able to select UE(s) for using AI/ML model based on UEs’ QoS requirement.

The network may also consider selecting UEs based on the RAN measurement results. If one UE has poor resource condition according to the channel measurements or performance measurement, network may decide to prioritize such UE using AI/ML model to improve its performance. For example, a UE with longer delay budget, e.g., delay budget ≥100ms, can be selected to perform reinforcement learning action if the serving cell radio utilization level is low, e.g., PRB usage for the cell is below 50%.

- Network should be able to select UE(s) for using AI/ML model based on RAN measurement.

Alternatively, the selection can be just based on any indication from CN or from UE itself. CN or UE can send an indication to NG-RAN based on its knowledge about its performance and communication requirement. This indication represents certain UE(s) should be selected to use AI/ML model to improve its performance based on information collected in CN/OAM. The indication may also be received from UE, indicating its preference of letting network optimize its performance via AI/ML model.

- Network should be able to select UE(s) for using AI/ML model based on indication from CN or UE.

## Functional Framework

### Information from Model Training to Model Inference

Model Deployment

In this section, we consider following two deployment scenarios of “Model Training” and “Model Inference”.

1. “Model Training” and “Model Inference” are collocated at the same network entity/UE

2. “Model Training” and “Model Inference” are separately located at the different network entities/UE

In RAN3 #112e meeting, following agreements are captured:

Where ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases

However, the standard impact should continue to be studied for each use cases if there’s any.

For scenario 1, AI/ML model is transferred from logical “Model Training” function to “Model Inference” function within the same network entity. In this case, there’s no standard impact introduced by model deployment/update. Model/Information exchanges between “Model Training” and “Model Inference” is purely based on implementation at the network or at the UE side.

- When “Model Training” and “Model Inference” collocated at the same network entity or UE, model deployment/update and model performance feedback have no standards impact.

For scenario 2, “Model Training” function needs to transfer the trained model to “Model Inference” via interfaces between different network entities (e.g. F1 interface, NG interface, interface between OAM and RAN) or between network and UE (e.g. Uu interface). On the other hand, not every network entity has AI/ML capability and one may not prefer to use AI/ML model to make decisions. To avoid unnecessary deployment, network entity with “Model Inference” should request AI/ML model(s) from the “Model Training” function. For example, gNB may request AI/ML model(s) from MDA (in OAM), or UE may request AI/ML model(s) from the network.

In RAN3 #112e meeting, following content is captured in draft TR 37.817 [1] as one of the high-level principles:

- The Model training and Model inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the algorithm.

Similar as information which is used to train/execute the AI/ML algorithm, “Model Inference” function should also be able to request AI/ML model deployment from “Model Training” function. Only the network entity/UE which requests AI/ML model(s) from “Model Training” function can receive the model and generate inference results.

- “Model Training” function should deploy AI/ML model to “Model Inference” function per request.

Model Update

AI/ML model can be continuously trained and integrated based on the feedback from “Action” and data collected from other network entities. To help continuously improve network performance with better AI/ML model, “Model Training” function may provide updated model(s) to “Model Inference” based on network or certain UE’s performance feedback. Alternatively, “Model Training” function can also provide the updated model periodically. The network entity which holds “Model Inference” can also request a Model Update, similar as requesting model deployment.

- “Model Training” function should be able to update AI/ML model to “Model Inference” function based on feedbacks (e.g. performance feedback), periodically or per “Model Inference” request.

Accuracy Level to Model Inference and Actor

It was agreed in RAN2 #112e meeting to continue the discussion “If the inference function provides output predictions, an optional indication of the accuracy level for which the inference model is trained should be indicated to the nodes that request/subscribe to this information”.

Machine learning model is trained based on a large set of data so that network can easily predict values/status of the network in near future. However, the prediction result may not be always correct or accurate. Incorrect or biased prediction values may lead to wrong decision, causing poor network performance or even network shutdown.

The accuracy can be measured by how often predictions equal labels [2]. It can be calculated for each AI/ML model by comparing the predictions and true values. With such information, one may know how well this AI/ML model is trained, which also indicates the trust level of using prediction results generated by this AI/ML model.

To avoid negative performance impact, during model training phase, for each trained AI/ML model, “Model Training” function should be able to generate the corresponding accuracy, representing how often the predictions equal labels. To help the requested network entities which uses the outputs from AI/ML model(s) understand whether the predicted results can be trusted or not, the accuracy level should be provided to “Model Inference”. Furthermore, as “Model Inference” can receive the updated AI/ML models from “Model Training” function, network nodes with “Model Inference” function can select whether to use the updated model or keep using the original one based on the accuracy level. If the accuracy level of the previous AI/ML model is higher than the updated AI/ML model, “Model Inference” function may decide not to use the updated model by implementation.

- Accuracy level can help “Model Inference” function understand the trust level of using AI/ML model for prediction. It can also help decide which model should be used when receiving the updated model.

- “Model Training” function should transfer accuracy level to “Model Inference” function when deploying/updating AI/ML model.

Additionally, “Model Inference” function should also propagate the received accuracy level of AI/ML model to “Actor”. “Actor” node can decide whether to trust the outcome of ML model inference or not based on the accuracy level corresponding to inference results generated from AI/ML model. If the accuracy level is high, “Actor” node can take action according to the received policy or results; otherwise, “Actor” node can ignore the received inference results and continue with the legacy mechanism.

- “Model Inference” function should propagate accuracy level to “Actor” function.

### Infromation from Model Inference to Model Training

Model Performance Feedback

From the previous discussions, it was not clear what should be included in the model performance feedback. From our understanding, this feedback is used to capture how environment is responding to a certain decision made by model inference. Such feedback may only be available after action is taken. Thus, we think that model performance feedback is not needed directly from “Model inference” to “Model training”.

- Performance feedback of a model could be only available from the environment after a certain action is taken. The direct performance feedback from “Model inference” to “Model training” is not needed.

Model Inference Assistance Information

However, considering the scenario where AI/ML model is deployed in a separate network entity for “Model Inference” with “Model Training”, each network node holding “Model Inference” function can be optimized and compressed to meet its own capability. To provide such customized machine learning models, some basic information, such as hardware capability, ML capability, etc, should be provided to “Model Training”. In this case, “Model Inference assistance information” is needed from “Model Inference” to “Model Training”.

- Consider “Model Inference Assistance Information” from “Model Inference” to “Model Training”, indicating capability of the network entity which holds “Model Inference”.

### Model Validation

Validating the ML model is important to ensure its accuracy. Basically, when the model is trained, validating the model with different set of data (e.g. different from training data) provides an opportunity to further improve the model quality, which further avoids making wrong decisions taken by the machine in the real-life prediction.

In this case, besides training data provided to “Model Training” function and inference data provided to “Model Inference” function, “Data Collection” should also provide validation data to “Model Training”, so that the accuracy of the trained model can be guaranteed.

- “Data Collection” function should also provide validation data to “Model Training” function for ML model validation.

- “Model Training” should also perform model validation based on the validation data set received from “Data Collection” to further improve model accuracy.

A text proposal for the above proposals is provided in Annex A.

- Agree the text proposal in Annex A.

# Conclusion

In this contribution, we discussed remaining FFSs of functional framework of RAN intelligence network, a text proposal to TR 37.817 [1] is also proposed to be agreed.

We propose the following observations and proposals:

Observation 1: AI/ML model may take historical information to train the model directly without requesting output from other AI/ML models.

Observation 2: A complicated AI/ML model can be split into multiple AI/ML models, where one AI/ML model output can be used by another AI/ML model.

Observation 3: A common AI/ML model can be used by multiple use cases, where the predicted results from the common AI/ML model can be directly used as inputs by multiple AI/ML models with different purposes.

Observation 4: The source gNB can use the outputs of AI/ML model from another gNB for prediction and decision making.

Proposal 1: “Model Inference” should transmit the inference output(s) and the corresponding state when generating the output to “Data Collection” function.

Proposal 2: AI/ML model can should be able to use output of another AI/ML model by requesting the data from “Data Collection” function.

Proposal 3: The network needs to select the most appropriate UEs to use AI/ML model for prediction, as it can help to reduce system computation complexity and reduce latency during performing model inference. Inaccurate decision can also be avoided.

Proposal 4: Network can should be able to select UE(s) for using AI/ML model based on UEs’ QoS requirement.

Proposal 5: Network can should be able to select UE(s) for using AI/ML model based on RAN measurement.

Proposal 6: Network can should be able to select UE(s) for using AI/ML model based on indication from CN or UE.

Observation 5: When “Model Training” and “Model Inference” collocated at the same network entity or UE, model deployment/update and model performance feedback have no standards impact.

Proposal 7: “Model Training” function should deploy AI/ML model to “Model Inference” function per request.

Proposal 8: “Model Training” function should be able to update AI/ML model to “Model Inference” function based on feedbacks (e.g. performance feedback), periodically or per “Model Inference” request.

Observation 6: Accuracy level can help “Model Inference” function understand the trust level of using AI/ML model for prediction. It can also help decide which model should be used when receiving the updated model.

Proposal 9: “Model Training” function should transfer accuracy level to “Model Inference” function when deploying/updating AI/ML model.

Proposal 10: “Model Inference” function should propagate accuracy level to “Actor” function.

Proposal 11: Performance feedback of a model could be only available from the environment after a certain action is taken. The direct performance feedback from “Model inference” to “Model training” is not needed.

Proposal 12: Consider “Model Inference Assistance Information” from “Model Inference” to “Model Training”, indicating capability of the network entity which holds “Model Inference”.

Proposal 13: “Data Collection” function should also provide validation data to “Model Training” function for ML model validation.

Proposal 14: “Model Training” should also perform model validation based on the validation data set received from “Data Collection” to further improve model accuracy.

Proposal 15: Agree the text proposal in Annex A.

# References

- TR 37.817, E-UTRA and NR; Study on enhancement for Data Collection for NR and EN-DC (Release 17), v0.2.0

- Accuracy metrics, https://keras.io/api/metrics/accuracy_metrics/

- R3-213470, AI/ML based load balancing, Intel Corporation

# Annex A – Text Proposal to TR 37.817 [1]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# 4	General Framework

Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)

4.1	High-level Principles

The following high level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of Model inference function should be studied case by case.

- RAN3 should focus on the analysis of data needed at the Model training function from external functions, while the aspects of how the Model training function uses inputs to train a model are out of RAN3 scope.

- Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- The Model training and Model inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the algorithm.

- The Model inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that are subject to actions based on the output from model inference.

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- AI/ML model can use output of another AI/ML model by requesting the data from the Data Collection Function.

- Network can select the most appropriate UEs to use AI/ML functionality based on UEs’ QoS requirement, RAN measurement, indication from CN or UE, etc.

- The Model inference function should be able to request AI/ML model(s) to be used for inference toward the Model training function and to avoid unnecessary deployment.

- The Model training function should be able to provide updated AI/ML model(s) toward the Model inference function based on feedbacks, request from Model Inference function or periodically.

- Accuracy level of each AI/ML model is generated at the Model training function. It is forward to the Model inference function and propagate to Actor, indicating the accuracy of AI/ML model and generated inference outputs.

## 4.2	Functional Framework

Editor’s Note: Data Preparation aspects may be further refined

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1.

- Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific pre-processing of data is not carried out in the Data Collection function.  
Examples of input data may include measurements from Ues or different network entities, performance feedback, AI/ML model output.

  - Training Data: information needed for the AI/ML model training function.

  - Validation Data: information needed to validate the performance of the trained AI/ML model

  - Inference Data: information needed as an input for the Model inference function to provide a corresponding output.

- Model Training is a function that performs the training and validation of the ML model. The Model training function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation of raw data), if required. The Model training function is responsible for generating accuracy level of ML model during ML model validation.

- Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). The Model inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation of raw data), if required.

- Actor is a function that receives the output from the Model inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

- Feedback: Information that may be needed to derive training or inference data or performance feedback.

- Model deployment/update: Deploy or update AI/ML model to Model Inference function per request or based on feedback. The Model deployment should also include accuracy level of each AI/ML model.

- Model Inference outputs: Outputs of Model Inference and the corresponding state when generating the outputs.

- Model Inference Assistance Information: Information that may be need for the Model training function when deploy/update AI/ML models.