3GPP TSG-RAN WG3 Meeting #113-e	R3-213493

# Online, Aug 16th – 26th 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for Introduction of QoE measurements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei, China Southern Power Grid |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2021-07-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Running CR for introduction of QoE measurements in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR captures the agreements made since RAN3#112-e meeting. (detailed agreements are listed in the end of the CR) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoE measurements will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CRxxxx TS 38.306 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

<Next modification>

X	 QoE Measurement Collection

X.1 Overview

This function enables collection of application layer measurements from the UE. The supported service types are:

-	QoE Measurement Collection for streaming services;

-	QoE Measurement Collection for MTSI services.

-	QoE Measurement Collection for VR services.

-	QoE Measurement Collection for MBS services.

-	QoE Measurement Collection for XR services.

  - X.2	 QoE Measurement Collection

X.2.1 General

X.2.2 RAN overload QoE handling

X.2.3 QoE Measurement handling in RRC_Idle and RRC_Inactive

  - X.3	 QoE measurement continuity for mobility

The continuity of signalling based QoE Measurement Collection for intra-system intra-RAT mobility is supported. For each QoE measurement configuration, the area scope parameter is included in the QoE configuration, and the network is responsible for keeping track of whether the UE is inside or outside the area. The QoE measurement continuity should be supported in RRC_CONNECTED and RRC INACTIVE state. In RRC_CONNECTED state, the source gNB needs to transmit signaling based QoE Measurement configurations of a UE to the target gNB. To support RRC_INACTIVE state mobility, all QoE measurement configurations of a UE can be fetched from the node hosting the UE context. Multiple sets of QoE measurement configuration should be supported during mobility.

  - X.4	 RAN visible of QoE measurement

RAN visible QoE measurement support DASH streaming and VR service, RAN visible QoE collection can be configured only if QoE measurements for application layer are configured for the same service type. RAN visible QoE parameters should be derived from QoE metrics data collected from UE, and it is a subset of legacy QoE metrics. Only the UEs with RAN visible capability can support RAN visible QoE reporting. The RAN generates the RAN visible QoE measurement configuration and send it to UE. Multiple simultaneous QoE measurements need to be supported, and RAN visible QoE activation/deactivation procedure need to be supported. The RAN visible QoE measurement configuration and reporting need to be transmitted outside the application layer QoE container.

  - X.5	 Per-slice QoE measurement

  - X.6	 Radio related QoE measurement

The radio-related QoE measurements are reported for all types of supported services, and immediate MDT can be used for Radio-related measurements for QoE analysis. If new radio-related measurements, with respect to what is currently specified in MDT, are required for NR QoE management, these additional radio-related QoE measurements will be specified as a part of MDT measurements.

In order to evaluate and improve the QoE, the NG-RAN node could trigger a MDT measurement for radio-related measurements towards a certain UE. Radio-related measurement and QoE measurement can be configured simultaneously by OAM for the alignment, and OAM is responsible for the QoE and MDT correlation.

Annex	RAN3 agreements on NR QoE

RAN3 agreements for information, which wlll be removed from final CR.

RAN3#112-e (May 2021)

- QoE measurement Configuration and Reporting

	- Liaise SA5 on the support of (de)activation of NR QoE, including concerns on whether current Trace Function could support QoE mechanism, decoupling of deactivation, failure handling and QoE Reference

	- Introduce a new IE "QoE Reference" explicitly over interfaces at least for s-based, whether it can be applied to m-based and whether it is per service type or per slice depends on feedback from SA5

	- Introduce the following additional new IEs:

- a list of UE Application layer measurement configuration IE for each service type.

- inside each UE Application layer measurement configuration IE:

- Container.

- a numerated IE indicating service type (e.g., Streaming services, MTSI services, VR, MBMS, XR).

- Area scope (a list of cells/TA/TAI/PLMN).

- Support for Mobility

	- Include signaling based QoE measurement configuration in handover preparation messages i.e. in XnAP: HANDOVER REQUEST, NGAP: HANDOVER REQUEST.

	- Signaling based QoE measurement configuration is stored in NG-RAN when UE enters RRC_INACTIVE and is propagated to new serving NG-RAN using Retrieve UE context procedure when UE resumes RRC connection in another NG-RAN i.e. include signaling based QoE configuration in RETRIEVE UE CONTEXT RESPONSE in XnAP.

	- Include multiple sets of signaling-based QoE measurements configuration in Xn/NG: HANDOVER REQUEST and Xn: RETRIEVE UE CONTEXT RESPONSE.

	- Management based QoE should not override an existing signaling based QoE configuration.

	- Option 1 is agreed by RAN3 on area handling for QoE i.e. the network is responsible for keeping track of whether the UE is inside or outside the area and the network configures/releases configuration accordingly. Send LS to RAN2 and SA4 informing RAN3 agreements.

	- Upon the reception of QoE configuration on a non-supporting node, the target node should not set up any QoE session with MCE and should not initiate any QoE measurement collection.

	- Liaise SA4 to check if QoE requirement for ongoing session continuity is also applicable for NR QMC and in case QoE configuration release is received during an ongoing session.

- Per slice QoE measurement

None

- RAN visible QoE

	- The service types supported in the Rel17 RAN-visible QoE framework are DASH streaming and VR.

	- WA: The following metrics, pertaining to DASH streaming and VR services, should be supported in the Rel17 RVQOE framework:

- Buffer Level

- Average Throughput

- Playout Delay

	- LS to other WGs, based on the resolution of the WA above, is expected at the next RAN3 meeting.

	- The following is supported within the RVQOE framework:

- RAN-visible QoE metrics: a subset of legacy QoE metrics data collected from UE, which are useful for RAN.

- RAN-visible QoE values: a set of values derived from QoE metrics data through a model/function defined in collaboration with SA4 (pending SA4).

	- WA: The RAN generates the RVQOE measurement configuration

	- The UE is assumed to indicate to the RAN its capability with respect to providing RVQOE metrics (LS to RAN2 seems needed).

	- WA: RVQOE collection can be configured only if QoE measurements are configured for the same service type.

	- Together with the QoE measurements, the RVQOE is supported in the following aspects:

- Activation, and deactivation procedures

- WA: Multiple simultaneous QoE measurements

	- WA: the ID used to identify QoE measurements is reused for identifying the RVQOE measurements.

	- WA: the RVQOE report is provided inside a dedicated IE, outside the QoE report container.

	- Whether transfer of RVQOE configuration to the target be supported will be discussed after the basic solution for mobility has been defined.

	- Whether the RVQOE report can be signaled from the target to the source at handover will be discussed after the basic solution for mobility has been defined.

	- WA: gNB-CU may signal RVQoE report to gNB-DU over F1

- Alignment of Radio-Related Measurement and QoE Measurements

	- Immediate MDT is taken as baseline for the collection of Radio-related Measurements to assist QoE analysis.

	- Existing measurements specified for immediate MDT can be used for Radio-related measurements for QoE analysis.

	- New radio-related measurements, if any, should be defined in the SON/MDT WI.

	- Radio-related measurement and QoE measurement can be configured simultaneously by OAM for the alignment.

	- OAM (e.g. TCE or MCE) is responsible for correlation.