3GPP TSG-RAN3 Meeting #113-e	R3-213503

16 – 26 August 2021 E-Meeting

Title: 	(TP on UE history information for 36.413) Addition of UE history information for SN

Source: 	CATT, CMCC

Agenda item:	10.2.1.4

Document Type:	other

# 1. Introduction

This contribution is the TP for TS36.413 v16.6.0 with addition of UE history information for SN.

# 2. Discussion

As discussed in R3-213502, UE history information for SN in MR-DC is introduced.

# 3. Annex – TP

Start of the first change

9.2.1.42	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1 .. <maxnoOfCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. | - | - |
| >Last Visited Cell Information | M |  | 9.2.1.43 |  | - | - |
| > Last Visited PSCell List |  | 0..1 |  |  |  |  |
| >> Last Visited PSCell Item |  | 1..< maxnoOfPSCellsinUEHistoryInfo > |  |  |  |  |
| >>> Last Visited PSCell Information | M |  | 9.2.1.x |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoOfCellsinUEHistoryInfo | Maximum length of the list. Value is 16. |
| maxnoOfPSCellsinUEHistoryInfo | Maximum length of the list. Value is 16. |

9.2.1.x	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  | - |  |
| >E-UTRAN Cell |  |  |  |  | - |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | 9.2.1.Y |  | - |  |
| >NG-RAN Cell |  |  |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | OCTET STRING | Defined in TS 38.413 [39]. |  |  |

9.2.1.Y	Last Visited E-UTRAN PSCell Information

The Last Visited E-UTRAN PSCell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell Global Cell ID | O |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| PCI | O |  | INTEGER (0..503, …) | Physical Cell Identifier of PSCell |  |  |
| EARFCN | O |  | 9.2.1.95 | DL EARFCN for FDD or EARFCN for TDD |  |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the PSCell in seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the PSCell in 1/10 seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 40950. | YES | ignore |

End of the last change