3GPP TSG-RAN WG3 Meeting #113-e	R3-213589

# Online, 16th - 26th August 2021

Agenda Item:	20.2.1

Source:	Ericsson

Title:	[TP for BL CR 38.300] On Cell Identifiers for NR NTN

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-213588. Changes to the BL CR are highlighted.

# 2	Text Proposal

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 16.x.5 Signalling [FFS]

The Cell Identity used in following cases corresponds to a fixed geographical area, irrespective of the orbit of the NTN payload or the types of service links supported.

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information (as defined in TS 38.413 [26]);

-	The Cell Identity used for Paging Optimization in NG interface (as defined in TS 38.413 [26]);

-	The Cell Identity used for Area of Interest (as defined in TS 38.413 [26]);

-	The Cell Identity used for PWS (as defined in TS 38.413 [26]).

Editor’s note:	it is FFS whether the Cell Identity indicated by the gNB to the Core Network should be specified per function/procedure/feature as above.

If NG-RAN decides to also use Uu Cell Identities on the NG and Xn interfaces, the Cell Identities shall be allocated in a way that Uu Cell Identities can be distinguished from mapped Cell Identities.

The Cell Identity included within the target identification of the handover messages (as defined in TS 38.413 [26] and TS 38.423 [x]) allows identifying the correct target cell.

NOTE:	The Cell Identity included in other cases, e.g. for RAN Paging Area, may correspond either to a Uu Cell Identity or to a fixed geographical area or both.

Editor’s note:	 from rapporteur if the text stay as it is, the reference for TS 38.423 should be introduced in TS 38.300

NOTE:	In the current version of this specification it is expected that served NTN cells and their neighbour relation ships are not exchanged via Xn.

The mapping between Cell Identities and geographical areas is configured in the RAN and Core Network.

Editor’s note:	It is assumed that the method how the gNB derives the Cell Identity provided to the CN is a matter of implementation and configuration, and it is FFS whether this fact requires normative or informative specification text.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>