3GPP TSG-RAN WG3 Meeting #113-e	R3-213894

E-meeting, 16– 26 August, 2021

Agenda item:	18.4

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TR 37.817): Standards Impacts for the AI/ML Energy Saving Use Case

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #112-e, the following agreements were made:

Progress the prioritized use cases on energy saving, load balancing, traffic steering/mobility optimization, i.e. at least by identifying their impact on the specifications, before considering any new use case.

Where ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases

In RAN3 #112-e it was also agreed that detailed standards impacts for the energy saving use case should continue at the next meeting. In this contribution, we focus on those standards impacts for AI/ML energy saving.

# 2	Discussion

Energy saving functionality has been introduced to reduce the network energy consumption and the energy-related operational expenses when possible. Energy saving deployments consider capacity booster cells that are deployed on top of coverage cells to enhance capacity for E-UTRA or NR in single or dual connectivity. Those cells are allowed to be optimized by being switched off when capacity is not needed and re-activated on demand. Switching off a cell can be done by the OAM and by the NG-RAN node owning the capacity cell which can autonomously switch it off using e.g., cell load information.

An NG-RAN node can initiate handover to offload traffic from the cell being switched off (and a reason for this handover can be indicated to help the node in future actions). Neighbours are informed over Xn by the owner of the cell about the switch off decision. Idle mode UEs can be prevented from camping on a cell that is switched off and incoming handovers can be prevented as well. Neighbours keep cell configuration data even when a cell is inactive. An NG-RAN node not owning capacity booster cells, can request re-activation of a capacity cell from a neighbour over Xn if there is a capacity need for such. In this case, cell activation procedure is used. Neighbours are also informed over Xn about the switch on (reactivation) decision over Xn interface. Switch on can be decided also by OAM.

An NG-RAN node may perform autonomous switch-off of its cells. It can also request re-activation of a list of cells owned by a neighbour. This can be configured to an NG-RAN node by the operator. OAM may also configure policies for a switch-off action at an NG-RAN node or for re-activation of a switched-off cell.

OAM and RAN have different levels of information to make their decisions. OAM receives (non real-time) counters, Key Performance Indicators (KPI)s, and alarms from the full network while RAN receives (real-time) UE measurements, own load measurements (e.g. radio load, TNL load), load measurements from neighbour gNBs, etc. In a sense, OAM has a broader view of the network performance received at a longer time scale as opposed to RAN that has a narrower view (but on a more real-time scale). Thus, OAM and RAN can have different views on which cells should be switched-off or switched-on. For instance, if RAN and OAM take independent actions it may happen that OAM instructs a cell to be switched-off through a policy while an algorithm running at a neighbour NG-RAN node may have needed it activated. These conflicts could be resolved with proper coordination. We illustrate this through the example of Figure 1, which depicts different possibilities on the training and inference at the OAM and/or gNB.

Figure 1 Location options of AI/ML Energy Saving functionality: a) Training at OAM/Inference at gNB, b) Training and Inference at OAM, c) Training and Inference both at OAM and gNB, and d) Training and Inference at gNB.

Options a) and b) of Figure 1 correspond to a solution where the ML functionality is only located in OAM. It can be related to existing solutions when OAM provides policies to the RAN about energy saving actions with the difference that the policies are given by an ML algorithm in OAM. Those policies can be given in 2 ways. In Option a) OAM can provide a gNB with a trained ML model. Execution of the ML model provides the necessary energy saving actions/policies to the RAN. Another alternative is that OAM executes the trained ML Model and provides its output, a decision, to the gNB as in Option b). Note that in Figure 1 b) the Actor could alternatively be located also in the OAM when OAM can take directly energy saving actions for the gNBs in the RAN.

As mentioned earlier, an gNB may perform autonomous switch-off of its cells. If the gNB is capable of performing ML locally (by training and executing an ML Model), then an autonomous switch-off decision could be made at the gNB, as illustrated in Option d). This option would correspond to a distributed ML energy saving solution.

However, as discussed earlier since RAN and OAM have different network views and access to different types of data, it is possible that the “best” ML Energy Saving decision from OAM’s point of view is in conflict with the best local ML decision in the RAN. Therefore, in this case coordination in the ML actions between OAM and gNB is needed.

Furthermore, even though in existing specification OAM may define energy saving policies for the RAN in case this policy is an outcome of an ML Model running at the OAM, it is possible that the energy saving ML Algorithm in OAM has not received sufficient data to be trained to the required accuracy. Therefore, the policy provided to the RAN from OAM may not be optimal, but may still be a “good” decision. In this case, OAM may prefer that RAN selects its best local energy saving action.

All placement options are applicable for unsupervised and supervised learning. However, if training and inference are located at different network entities then online Reinforcement Learning is not applicable. Still, in case training is at OAM and inference at gNB, Reinforcement learning with Offline Training is possible by letting the Q-table/network(s) to be trained periodically at the OAM and the inference to happen at the gNB.

Proposal 1: All the four options shown in Figure 1 for location of the AI/ML Energy Saving function should be supported for the ML energy saving use case.

Proposal 2: Agree the corresponding TP for TR 37.817 in the Annex.

# 3	Conclusion

We make the following proposals:

Proposal 1: All the four options shown in Figure 1 for location of the AI/ML Energy Saving function should be supported for the ML energy saving use case.

Proposal 2: Agree the corresponding TP for TR 37.817 in the Annex.

# Annex - TP for TR 37.817

# 5       Use Cases and Solutions for Artificial Intelligence in RAN

## 5.1	Network Energy Saving

### 5.1.1	Use case description

To meet the 5G network requirements of key performance and the demands of the unprecedented growth of the mobile subscribers, millions of base stations (BSs) are being deployed. Such rapid growth brings the issues of high energy consumption, CO2 emissions and operation expenditures (OPEX). Therefore, energy saving is an important use case which may involve different layers of the network, with mechanisms operating at different time scales.

Cell activation/deactivation is an energy saving scheme in the spatial domain that exploits traffic offloading in a layered structure to reduce the energy consumption of the whole radio access network (RAN). When the expected traffic volume is lower than a fixed threshold, the cells may be switched off, and the served Ues may be offloaded to a new target cell.

Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments. The optimal EE decision depends on many factors including the load situation at different nodes, RAN nodes capabilities, KPI/QoS requirements, number of active Ues and UE mobility, cell utilization, etc.

However, the identification of actions aimed at energy efficiency improvements is not a trivial task. Wrong switch-off of the cells may seriously deteriorate the network performance since the remaining active cells need to serve the additional traffic. Wrong traffic offload actions may lead to a deterioration of Energy Efficiency instead of an improvement. The current energy-saving schemes are vulnerable to potential issues listed as follows:

- Inaccurate cell load prediction. Currently, energy-saving decisions rely on current traffic load without considering future traffic load.

- Conflicting targets between system performance and energy efficiency. Maximizing the system’s key performance indicator (KPI) is usually done at the expense of energy efficiency. Similarly, the most energy efficient solution may impact system performance. Thus, there is a need to balance and manage the trade-off between the two.

- Conventional energy-saving related parameters adjustment. Energy-saving related parameters configuration is set by traditional operation, e.g., based on different thresholds of cell load for cell switch on/off which is somewhat a rigid mechanism since it is difficult to set a reasonable threshold.

- Actions that may produce a local (e.g. limited to a single RAN node) improvement of Energy Efficiency, while producing an overall (e.g. involving multiple RAN nodes) deterioration of Energy Efficiency.

To deal with issues listed above, ML techniques could be utilized to leverage on the data collected in the RAN network. ML algorithms may predict the energy efficiency and load state of the next period, which can be used to make better decisions on cell activation/deactivation for ES. Based on the predicted load, the system may dynamically configure the energy-saving strategy (e.g. the switch-off timing and granularity, offloading actions) to keep a balance between system performance and energy efficiency and to reduce the energy consumption.

### 5.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

There are several possible location options for AI/ML energy saving. We list them next in Figure 5.1.2-1:

Figure 5.1.2-1 AI/ML Energy Saving function location options

#### 5.1.2.1 AI/ML Training and Inference coordination between OAM and RAN

This corresponds to Option c) of Figure 5.1.2-1. There exists an interplay between the AI/ML energy saving actions of OAM and RAN, and specifically for the capacity layer of a gNB. OAM will need to receive sufficient training data to train an ML Algorithm and provide the best Energy Saving for a gNB. An easy way to provide this information could be reporting. Namely, a gNB (both capacity and coverage layers of a gNB) may provide OAM with reports that contain information useful for training an energy saving ML algorithm in the OAM. Those energy saving reports can contain information such as the current load or predictions of load regarding the cells of the capacity layer, an expected capacity layer power consumption or energy efficiency or any metric measuring the saving in energy at the expected load levels. Similarly, energy saving reports from the coverage layer can contain information on the additional traffic that needs to be handled in the event that a capacity cell is switched-off. This additional traffic may correspond further to predicted traffic calculated at the coverage layer and could be mapped into a cost for cell switch off. After receiving this information, OAM may decide and instruct the capacity layer of a gNB about which cells should be switched off or when a cell that is switched-off should be switched on again for one or more time intervals.

In existing specifications, OAM provides RAN with an energy saving policy for hard switch-off decisions on certain cells of a gNB. An OAM energy saving policy must be followed by a gNB. When intelligence is available both in the OAM and in the RAN, OAM may give some freedom in the RAN with respect to using the given OAM policy. This may be the case if OAM is not absolutely confident that the ML Algorithm running internally is trained sufficiently well if training hasn’t reached enough maturity. In such cases, a gNB may be in better position to decide the energy saving action using local ML intelligence. In other situations, and according to current baseline methods, OAM can switch-off cells in the RAN based on the outcome of ML intelligence in OAM. OAM can alternatively request a gNB to apply an Energy Saving policy (provided as an outcome of local ML in the OAM), if it has high confidence that this is the best energy saving solution. The output of energy saving can be which cell(s) at a gNB should be switched -off, which cells that are switched-off may be switched-on again, and timing information for the switch-on and switch-off patterns.

#### 5.1.2.2. AI/ML in the gNB

This option corresponds to Option d) of Figure 5.1.2-1. Energy saving decisions taken locally at a gNB could be improved by utilizing prediction information on incoming UE traffic, as well as prediction information about traffic that may be offloaded from a candidate energy saving cell to a neighbour cell ensuring coverage. In case, coverage and capacity layers belong to different gNBs, the capacity layer may request an estimate of the additional traffic that will be offloaded to the coverage layer from the latter for a certain cell switch-off action at the capacity layer. Exchange of predicted load information between gNBs can enable distributed energy saving decisions based on the expected traffic to be handled by a coverage cell.